KAON 571

NGAO Observing Scenarios

D. Le Mignant1,

E. McGrath2, C. E. Max2

1. W. M. Keck Observatory

2. Center for Adaptive Optics, University California Santa Cruz

# Introduction

## Purpose of the document

As part of the NGAO System Design phase (KAON 414), the NGAO Science Case Requirements Document have been presented and discussed in KAON 455 while KAON 548 provided the summary table in support of system architecture evaluations.

The purpose of this report is to complement KAON 455 and 548 and document the observing scenarios for the NGAO Science Cases presented in KAON 455 for both Key Science Drivers and Science Drivers.

## References

In the following sections, the references to the KAON 455 requirements are noted in italic: (reqt #1.2) refers to the Science Case requirement #1.2 found in the KAON 455 tabulated requirements. An other important reference for the science requirements is KAON 548 which presents a summary for the requirements.

In developing the observing scenarios, we have made some assumptions based on the lessons learned for LGS operations at Keck (KAON 463). Particularly:

- the fraction of LGS science time lost to weather is 25%.

- only 55% of the nights year-round are considered fully photometric

- the software architecture will allow for parallel operations of subsystems, reducing the overall overhead

# Science Operations and Observing  Efficiency

## Science Operations

The paradigm for the NGAO science operations is to optimize the observing efficiency with the astronomer(s) being in control of the observations with the technical support of Keck staff.

The science operations tools for NGAO include a suite of tools:

- the planning tools to: 1) search and select the AO Natural Guide Stars, 2) compute and predict AO performance, 3) prepare the observing sequences, 4) estimate the exposure time and 5) estimate the observing efficiency.

- the observing sequences control (OSC) tools to: 1) load, 2) check, 3) command and 3) view the observing sequences for the AO and the science instruments. These include Users’ Interfaces and Execution Modules for the astronomer and the observing support personnel. These tools are high-level command tools and coordinate the actions of the sequencer for each subsystem (Science Instrument, Telescope, AO, Laser, Data Server, etc).

- the post-observing tools to: 1) select the data to save, 2) provide an estimate for  the PSF calibrations (including PSF reconstruction) and 3) archive data when applicable.

The science operations tools will be detailed in the next phases of the NGAO design (PD and DD).

## NGAO Acquisition Sequence

Three types of targets needs to be acquired on the NGAO photo-sensors:

- the NGS for the LOWFS, the TWFS and/or the WFS

- the astronomical sources for the science instruments:

  - for the d-IFS: target galaxies plus simultaneous science field calibrators

  - for the narrow field science instruments: the science target

- the Laser Guide Stars for the HO WFS.

The LGS acquisition section will be detailed in the next iterations of the observing scenarios, depending of the options selected for the LGS asterisms and the point-and-shoot mode.

The table below presents a generic description for the acquisition sequence (see KAON 567).

| Step | Observing Step | Parallel Steps | Remarks |
| --- | --- | --- | --- |
| 0 | Select next target: - assess science priority - check target elevation range - check observing conditions - check LTCS conditions | Complete integration on current science target or calibrator. When target selected from Planning Tools, then information is loaded in OSC Tools. | It is not clear yet whether the astronomer will have to run these checks manually or whether it will be automated. |
| 1 | Upon completion of readout of science array, LGS is shuttered, AO loops open and key-system feedback parameters are saved then the Science Operations (ScOp) Tools triggers the telescope slew. | OSC Tools parses information, and get ready for execution: - NGS parameters for acquisition - AO configuration - Instrument configuration |  |
| 2 | Telescope slews | The OSC Tools sends command to the subsystem sequencer and setup sequences are executed as appropriate by the AO, Laser, Science Instrument subsequencers. |  |
| 3 | Telescope Pointing Adjustment on one of the NGS (brightness allowing). This steps is automatically performed by the NGS acquisition subsystem with the visual check of the Observing Assistant (OA). Upon success, pointing corrections are applied and next telescope slew is commanded from the OSC. |  | This step is required when the NGS are fainter than R=18 mag. (TBC) or when the telescope slews by more than x deg in elevation/azimuth range. |
| 4 | Telescope coarse registration on the science field. NGS acquisition subsystem runs an automated routine to record and process image, ID the NGS in the field wrt catalog data then compute required offset. Visual check of process by OA. Upon success, position offsets are applied to telescope. | LGS propagation and acquisition steps initiated. If small telescope centering offset (<5 “ – TBC), uplink TT correction closed with very low gain. Pickoff positioning and LOWFS setup complete including background. | Need to implement the acquisition for the vibration/wind shake reference. Need addit’al study for telescope guiding. Not clear if the HOWFS will require a background. |
| 5 | Telescope fine registration on the science field: If photons are not detected at the expected SNR on the LOWFS (or NGS not on Pointing Origin), then NGS acquisition subsystem runs a second iteration. Visual check of process by OA. Iterate if necessary (to be detailed). Upon completion adjust telescope pointing model. | Pick-off mirrors for science and TWFS in position | Need to check the conditions for the dichroic during this step. Need to be able to adjust pointing model for telescope even though PO != REF. |
| 6 | AO subsystem control: 1) low gain on woofer and MEMs, 2) increase gain on UT, 3) start telescope guiding, 4) adjust woofer and MEMs gain, 5) initiate TWFS + tomography optimization | Science instrument is setup: optics and read modes are set and confirmed. May record first exposure to check centering with point-source and expected SNR/coadd. Monitor image quality and assess optimization progress. | UTT control (all/individual?) Assuming redundant information from USNO-B, GSC-II and SDSS, uncertainty in the field centering should be ~0.2” (TBC). |
| 7 | Science integration starts |  |  |

Table 1: Generic acquisition steps for NGAO

## Observing sequences: dither and offset

The observing sequences will be coordinated at the Observing Sequence Tool level to minimize the  overhead during dither and offset scripts: as soon as the instrument reports that the readout is complete, the OS tool will command the dither or offset either in instrument pixel or sky coordinates. KAON 558 proposes possible scenarios for dither and offset. We anticipate that any repositioning of the observing reference frame of less than 5” (TBC) will be performed by repositioning internal optics and will not require to move the telescope (hence not require to open any loops). These we propose to define as “dither” and should take less than 5 seconds.

Conversely, any move that require to move the telescope and open the AO loops are defined as offset, and may require  longer overhead (~ 15 -30 sec - TBD)

## Observing Efficiency Budget

We have developed a simple MS excel-based tool to estimate the observing efficiency for the NGAO science cases. Each science case has its own spreadsheet where open shutter time and overhead have been estimated for the science target and the calibration standard. We have made the following assumptions:

- Open shutter time on the sky-calibration (telluric and flux standards, sky background) is considered science time.

- The main contributors to the observing overhead will be the telescope slews, telescope pointing adjustment and field registration, NGAO and science field acquisitions, fine centering and re-centering, dither/offset + instrument setup + readout.

- For each contribution (open shutter and every overhead), we estimate the minimum, median and maximum values and calculate a weighted average with the following weights (1/6, 4/6, 1/6).

- Estimates for the overhead contributions from telescope slews, pointing adjustments and NGS acquisitions are derived from KAON 567.

- Additional overhead has been included for LTCS interrupts in form of an integer number of individual science integration (hence a higher impact for long integration) and subsequent time for re-centering the science target(s). Note that LTCS interrupts will be greatly reduced by the use of the “first-on-target” rule and the use of planning tools to avoid science observing during planned interrupt request from the Laser Clearinghouse.

- The overhead for dither/offset will be reduced compared to the current system if we adopt a different strategy for dither/offset (KAON 558)

- No time loss has been included for NGAO system failure at this point as we know very little about the NGAO failure rate. This could be considered overly optimistic, yet it is counterbalanced by the allocation of maximum values for every overhead.

The Observing Efficiency Tool can be downloaded from:

http://www.oir.caltech.edu/twiki_oir/bin/view/Keck/NGAO/NGAOObservingScenarios

# Galaxy Assembly and Star Formation History

The NGAO Science Case on Galaxy Assembly and Formation History is one of the five Key Science Drivers, which place the most restrictive or technologically challenging constrains on the NGAO system.

The requirements for this science case are described in Section 2.1.1 in KAON 455.  The following sub-sections document the observing scenarios.

## General observing strategy and observing efficiency:

The observing goal for this science case is to survey more than 200 galaxies over a few years (reqt #1.2) . This is possible by using a deployable NIR spectrograph over an area of ~ 3 to 5 arcmin. The number of targets per  field of regard depends on the  number density for the class of objects being studied and varies between 0.1 to 40 arcmin-2  (see KAON 455, table 3). Depending on the object brightness, the NGAO+dIFS will dedicate ~ 1.5 to 4 hours of open shutter time on a set of targets (reqt #1.2).

In estimating the observing efficiency, we have made the following assumptions:

- In parallel to observing a set of extragalactic target, we anticipate the observer will dedicate one of the deployable units on a point-like source to calibrate for the PSF, and possibly for the telluric absorption (depending on the spectral class for the point-like source). This point-like source will also be used to monitor the centering of the field.

- Yet, for every set of targets, we anticipate the observer will calibrate for the telluric absorption after the science spectroscopy observations and within a certain interval in air mass and time. This should add approximately 10 min of observing time to the observations of the set of targets. In addition, we anticipate the observer will want to calibrate the flux by observing flux standards, twice trough the night.

- We assumed individual integration time between 15 and 30 min on the d-IFS for the science targets and between 1 and 3 min for the standards.

- For an observation on a set of targets, we find that it will take 12 min on average for the full setup from initiating the telescope slew till first start of science-quality exposure. The open shutter time will be 155 min on average with 6.5 min of overheads for dither/readout and 5.3 min for re-centering. The observation of the telluric of flux standard will take approximately 10 min including ~ 55% total overhead (telescope slew, acquisition, dither, readouts).

- We anticipate 27 min loss due to LTCS interrupts, due to the long integration time.

Using these assumptions, we derive that it will be possible to observe 3.0 science fields including the telluric standards and two flux standards for an average night of 10.25 hours numbers.

The total observing efficiency (open shutter on science, including standards) is 81%.

Assuming 3, 5 and 6 deployable units used (minimum, median, maximum, respectively) on the science targets, this leads to 9 to 18 observed targets (15 average) per full observing night.

Number of allocated nights: The PI astronomer and her/his team would need 11 to 22 full observing nights to collect data for ~ 200 objects. Assuming a weather-loss fraction of 0.25 (KAON 463), this requires 15 to 29  allocated nights (spread over a few semester) to collect the needed sample of data.

Observing model: astronomers are requesting half to full night(s) for this science case in classical observing mode. The astronomer(s) will be performing the observations remotely (either from Waimea or elsewhere) and assessing the data quality on-the-fly. The observations require less than 1 mag of extinction in the V band (for the use of lasers). The PI and the science team are responsible for  the backup observing program.

(Time domain astronomy: this does include any time-domain astronomy)

## Pre-observing planning:

This section presents the actions undertaken by the astronomers prior to the observations with emphasis on the use of the NGAO planning tools.

- Simulations: Astronomers will want to run a first iteration of simulation at the time of the proposal and estimate the SNR per spatial and spectral resolution element assuming:

  - A model for the galaxy (geometry, size and flux) provided by the astronomer.

  - A 2-D PSF or an estimate for the SR, EE, provided by the NGAO planning tools.

  - A exposure time calculator to get an estimate for the SNR that can be achieved.

Subsequent iterations for the simulations may take into account a more realistic geometry for the AO guide stars and include other constrains such as the observing conditions (elevation range and an approximate seeing value)

- NGS selection: The NGAO Natural Guide Star finder tool allows the astronomer to search for, extract and save information for a few constellations of at least three natural guide stars. The information is retrieved from on-line or local astronomical catalogs (USNO-B, GSC-II, SDSS). The planning tools helps the astronomer to register the AO guide stars with respect to the science targets. The data will be saved and used for the acquisition at the telescope.

Some of the “legacy” high-z galaxy fields are well documented and have been surveyed at different wavelengths by ground and space telescopes. For these cases, it is possible to extract the NGS location directly from these legacy. Yet, the NIR magnitude and color information may be missing and be extracted from the astronomical catalogs.

- Observability:

- Observing conditions: the conditions will need to be transparent (less than ~ 1 mag. of extinction at 589 nm) for the use of lasers.

What is the photometric requirement?

## AO observing mode:

The observations with the d-IFS will require the LGS AO mode with the medium to wide configuration for the LGS asterism. The configuration for the LGS asterism will be optimized as a function of the location of the science targets for the d-IFS and the location of the NGS. The planning tools will help the astronomer to select the LGS and NGS configuration that optimizes an observational quantity (SR, EE or target selection)

The AO rotator will be in position angle mode: observers will set the orientation of the North axis, as projected on the d-IFS for each observation of a set of targets.

In order to get an accurate estimate for the PSF, the astronomer will select to dedicate one of the d-IFS unit to observe a faint star in the field, while brighter natural stars from the field will be used for the LOWFS and the truth WFS.

The observer may choose to observe a set of bright photometric or telluric calibrators with the d-IFS. We anticipate the use of the d-IFS in NGS AO mode to maximize the observing efficiency (better SNR, minimal overhead).

## Science instrument configuration:

The science instrument configuration will be detailed during the Preliminary and Detailed Design phases for NGAO in parallel to the studies for the d-IFS.

KAON 548 provide details for the d-IFS: a spaxel size of 70 milli-arcsec (2 pixels of 35 mas/spaxel), a field-of-view of ~ 1.0 x 3.0 arcseconds per units. Six spectrograph units deployable over a field of regard of at least 120 arcseconds in diameter.

## Observing sequences:

### Acquisition

The acquisition sequence for this key science case requires careful planning. Three types of targets needs to be acquired: the NGS for LOWFS and the TWFS, the astronomical sources for the d-IFS (target galaxies plus simultaneous science field calibrators) and the LGS. The LGS acquisition section will be detailed in the next iterations of the observing scenarios, depending of the options selected for the LGS asterisms and the point-and-shoot mode. The main challenge is the “blind acquisition & centering” of the science targets on a science array with an accuracy of less than 10% of the field of view.

KAON 467 provides details for acquiring the field stars on the acquisition camera.

We propose the following acquisition steps:

- Telescope slew and AO system setup

- Adjust telescope pointing next to science field

- Register NGS for LOWFS and TWFS on the acquisition camera

Once the targets are acquired Several observing sequences are anticipated for this science case:

- Dither / offsetting

- Micro-dithering

- Filter sequences

## Science Calibrations:

- Flat-field calibrations

- PSF calibrations

- Spectral calibrations

- Photometry

- Astrometry

## Post-Observing

- Data Product

- Data Reduction and Analysis

- Data archiving

Backup science is treated as a different observing scenario.

# Nearby Active Galactic Nuclei

# Precision Astrometry: Measurements of General Relativity Effects in the Galactic Center

# Imaging and characterization of extrasolar planets around nearby stars

# Multiplicity of Minor Planets

# QSO Host Galaxies

# Gravitational Lensing

# Astrometry Science in Sparse Fields

# Resolved Stellar Populations in Crowded Fields

# Debris Disks and Young Stellar Objects

# Size, Shape and Composition of Minor Planets

# Characterization of Gas Giant Planets

# Characterization of Ice Giant Planets

# Backup Science