Pre-Observing Interfaces

Design Manual

Document Reference   :    WMKO.NGAO.SD.WBS3.4.1.1

Keck AO Note             :    KAON ###

October 8, 2007

Prepared by:

        - D. Le Mignant, W. M. Keck Observatory

        - E. McGrath, Center for Adaptive Optics

Reviewed by:

Approved by:

TABLE OF CONTENTS

1. Introduction	3

2. Scope and Applicability	3

3. Related Documents	3

4. Revision History	3

5. Overall Requirements for the Pre-Observing Interfaces	4

6. AO Guide Star Tools Requirements	4

6.1.	Performance Requirements	6

6.1.1.	User-friendly, flexible and reliable interfaces	6

6.1.2.	Loading and resolving science target information	6

6.1.3.	Searching for candidate guide stars	7

6.1.4.	Select and load science instrument setup	7

6.1.5.	Select and load candidate AO setup	7

6.1.6.	Select and load observing conditions setup	8

6.1.7.	Operational	8

6.1.8. Implementation Requirements	8

6.1.9.	Design Requirements	8

7. Science Observations Simulation Tools Requirements	8

7.1.	Performance Requirements	8

7.1.1.	Parametric	8

7.1.2.	Operational	8

7.2.	Implementation Requirements	8

7.3.	Design Requirements	8

7.3.1.	Technological	9

7.3.2.	Standards Related	9

7.3.3.	Integration Related	9

8. Interface Requirements	9

9. Reliability Requirements	9

9.1.	Risk and Mitigation	9

10. Service and Maintenance Requirements	9

11. Documentation Requirements	9

12. Glossary	9

# Introduction

The purpose of this document is to specify and design the software tools and interfaces that the observer will require to prepare for the science observations.

As defined in KAON 414, this effort refers to the WBS 3.4.1.1 and WBS 3.4.1.2, that are defined as:

- WBS 3.4.1.1. AO Guide Star Interface: Specify and design the Method and Interfaces to search for suitable Guide Star given a science field, the isoplanatic (isokinetic) patch, and position angle requirements. Explore the requirements on parameters and format for the Guide Star information (proper motion, color, extended, etc), to be used for space command submission and the science operations.

- WBS 3.4.1.2. Science Observations Simulation Tools:  Define Method and design Interfaces to Simulate Science Observations scenarios, including image performance (SR, EE) as a function of observing conditions (seeing, elevation, etc); throughout; background emission and calibrations requirements. Explore the ability to compare various possible scenarios and conditions and save the output parameters.

# Scope and Applicability

The products of this WBS element will include 1) outputs for the FRD 2.0, 2) two Design Manuals that will present a conceptual design to demonstrate the feasibility of the tools, with an emphasis on the end-user point of view, and a flexible design approach to facilitate inclusion of new modules as the science instruments are defined.

This manual will include a  summary of the interface requirements, and describe an architecture that will meet the requirements. We will provide a detailed list of the requirements, I/O parameters, algorithms, etc, for the functions and graphical tools. Particular attention will be given to the algorithm for the AO simulation tools as well as the level of expertise for the end user. We will also lay down  the requirements for tool portability, development for inclusion of science instrument modules, optimization and maintenance. It may be possible to re-use some of the expertise developed while building the AO Guide Star Tool and the OSIRIS GUIs.

Presently, this document is in the working draft status.

# Related Documents

The Reference Documents are;

- KAON 414: System Design Phase Systems Engineering Management Plan

- The Functional Requirements Documents - version 1.0

- The Keck LGS AO page: http://www2.keck.hawaii.edu/optics/lgsao/

  - And its AO Guide Star tool: http://www2.keck.hawaii.edu/software/findChartGW/acqTool.php

# Revision History

The revision history is indicated in the table below:

| Revision | Date | Author(s) | Reason for revision/remarks |
| --- | --- | --- | --- |
| 1.0 | Oct. 7, 2007 | D. Le Mignant | Original Issue |
| 1.1 | Oct. xx, 2007 | - Le Mignant E. McGrath | Working draft |

# Overall Requirements for the Pre-Observing Interfaces

The overall requirements for the Pre-Observing Interfaces are summarized in the Functional Requirement Documents.

The pre-observing interfaces will allow the observer to simulate, evaluate and plan for an observing scenario using the NGAO science instruments any time prior to the observations, particularly during the observing time proposal submission process. Prior to, and during the observing night, the observer will use the same sets of tools to assess and refine the observing strategy according to the observing conditions.

The pre-observing interfaces encompass:

    - the AO Guide Star Tools to search adequate astronomical catalogs for natural guide sources, identify them, optionally with help from the observer, and save all relevant information required to simulate and perform the science observations.

    - the Science Observations Simulation Tools to load the relevant observing parameters for AO, and the science instrument, and simulate the performance of the AO + science instruments as a function of possible observing conditions. The results will be expressed and saved in terms of image quality and observing efficiency quantities.

The detail requirements for the AO Guide Star Tools and for the Science Observations Simulation Tools are presented in Sec. 6 and 7, respectively.

# AO Guide Star Tools Requirements

This section presents the requirements for the AO Guide Star Tools. The requirements are of three kinds: 1) performance requirements, 2) implementation requirements and 3) design requirements.

- The AO Guide Star Tools will identify a guide star constellation given a science target or a science field, an observing date, and a science instrument configuration.

- The method to use the tools will be different depending on the science case and the time in the pre-observing phase (e.g, observing time proposal, observations planning, or on-the-fly during observations).

- Once evaluated and reviewed, the output parameters for the guide stars constellation and instrument configurations will be saved in a format shared by the NGAO observing tools.

Figure 1: Simplified top-level algo

Step 1.0:

Collect, resolve and load science target(s) data

Step 0:

Resolve science target information

Step 1.9:

Select list of astronomical catalogs

Step 2.0:

Search for candidate guide stars within 4 arcmin

Step 3.0:

Sort, select and display candidate constellations

Step 2.1:

Select and load science instrument setup

Step 2.2:

Select and load AO setup

Step 2.3:

Select and load observing default conditions  setup

Step 5.0:

Optimize choice of constellations and AO setup

Step 4.1:

Select and load observing “real” observing conditions setup

Step 4.0:

Simulate AO performance for the observing scenario(s)

Step 5.1:

Simulate science instrument performance for the observing scenario(s)

rithm for the AO Guide Star Tools.

A simplified top-level algorithm for the AO Guide Star Tools, including its relation with the AO and science instrument is shown in Fig. 1. The functions included only in the AO Guide Star Tools are shown against a light-green background. The other functions, likely to be shared with the Science Simulation tools are shown against a light-blue background.

The term Guide Star is to be used in a generic sense as in Guide Source. At this point in the design, we are not excluding that the Guide Star can be either a point-like source or an extended source.

## Performance Requirements

The performance requirements can be expressed both in terms of parametric performance or in terms of operational performance.

### User-friendly, flexible and reliable interfaces

This section lists the requirements for user-friendly, flexible and reliable AO Guide Star Tools:

- The AO Guide Star tools are to be designed with the end-user in mind. The tools appearance should be very intuitive. The help manual should be easily accessible.

- The tools should be designed to work for the various phases of observing planning: observing time proposal, final observing planning (afternoon before), and on-the-fly observing planning for any required changes during observations.

- The tools should allow the user to load data and parameters either manually or from a file (using formatting rules).

- The tools should allow the user to save data and parameters in one or more files.

- The tools should propose template files for the AO and Instrument configurations and allow the user to create and save his/her own modified template files.

- The tools o the user end should be configurable using either previously saved files or dedicated template files.

- The tools will be available as an application that can be operated from any computer, either remotely or locally.

  - The application can be launched from a web-interface and directly from an Operating System (OS).

  - The OS considered are Mac, PC and Solaris.The web-browsers most-commonly used by astronomers on these platforms are Firefox, Safari, Internet Explorer (as of 2007).

  - Examples of such flexible tools have been designed in Java.

- The tool interfaces will automatically check for updates. The various versions of the tools should be compatible, yet the tool should check for possible compatibility conflicts for data format, parameters and tool configurations.

- The tool will use version-controlled configuration files for TBD calibration parameters (such as telescope pointing restrictions,  field-of-regards parameters, etc). These files should be downloadable from the internet.

### Loading and resolving science target information

This section describes the requirements on the information for the science targets, and how this information will be retrieved if not provided.

- The science target information needs to include:

  - Target Name: Maximum 15 characters, spaces allowed

  - RA: hh mm ss.sss

  - Dec: +dd mm ss.ss

  - Equinox: yyyy.yy or APP meaning apparent. The prefered Equinox is TBD.

- The precision goal for the coordinates should be better than 0.05 ″ to minimize any registration errors.

- A list of optional parameters follows:

  - X mag =m.m science target brightness in photometric X band, X=V, R, I, z, J, H, K).

  - pmra =+s.ssss proper motion

  - pmdec=+a.aaa proper motion

  - ttdra=s.ssss (RA differential tracking rate, in arcsec/hr divided by 15).

  - ttddec=a.aaa (Dec differential tracking rate, in arcsec/hr).

  - size = x.xx″ : apparent angular size of the object at the time of the observations

  - comments=” “  after the required fields

- The tool will read the name for the science target or the science field either from a file (using documented formatting rules) or from an input.

  - if only the name of the science targets is provided, the tool will request the user to select between SIMBAD, NED or Horizon databases to resolve RA, DEC, Equinox, and TBD for the target.

  - if the Horizon database is selected, the tool will request a UT date (day and hour range) as well as a sampling rate (15 min by default) to retrieve the information.

- Once the data for the science targets is retrieved, the user should have the possibility to edit and save the data and parameters in one or more files.

### Searching for candidate guide stars

This section describes the requirements for searching candidate guide stars.

- The tool needs to search for candidate guide stars in a 4 arcmin radius from the center of the science field.

- The apparent brightness for the candidate sources may range from -5 to +21 mag. in the photometric band referenced in the catalog. The upper and lower limits for the brightness should be configurable by the user.

- The number of guide stars to retrieve should be configurable by the user.

- The tool will search for stars and sources in a list of astronomical survey catalogs.

  - The catalog are public or private domain catalog compiled from data between 0.5 and 2.2 µm.

  - Each catalog will provide coordinates that can be translated into the Guide Star Tools RA, DEC and preferred Equinox.

  - The goal for the registration error between various catalogs is 0.1″ .

  - A request configuration file will specify for each catalog the fields, formats, conversion and value ranges for the data to be retrieved.

    - Here include as an example the data we are retrieving for USNO 1.0 versus 2MASS PSC

- The tools will search for data in astronomical catalogs available either on-site or from the internet.

- The tools will have the option to search among a subset from a list of catalogs.

- The tools should be easily upgrade-able to search from more astronomical catalogs as they become available.

- The information retrieved will be sorted by catalog. The user should be able to view the list of retrieved stars and sort them by a column (field) of choice.

### Select and load science instrument setup

This section presents the requirements for loading the science instrument setup for the AO Guide Star Tools.

- The setup for the science instrument includes the following TBD parameters:

  - Instrument array(s) field of view and geometry, including pixel scale and number of pixels, instrument array PA, array center location

  - Position Angle on the sky

  - Centering reference for the science target

  - Position of slits, focal masks

  - Position of PSF camera?

- The spatial sampling used for the tools will be 0.05 ″.

- The relative accuracy for registering and superimposing the sources using the catalog information with the field-of-views of the AO sensors and science instruments should be 0.1 ″.

- The selection of the science instrument setup should use template files, that can be modified/saved/loaded by the user. The templates will be defined as the instruments and observing modes are designed in detail.

- Options should be provided for taking into account additional TBD requirements from dithering, nodding around the science object(s).

### Select and load candidate AO setup

This section describes the requirements for selecting and loading the AO setup:

- This setup is mandatory for the tools to work

- The setup for the AO system includes the following TBD parameters:

  - AO mode: NGS, LGS

  - LGS asterisms (wide or narrow)

  - LGS TT mode? point and shoot?

  - WFS sensing wavelength for low-order sensor

  - Position of PSF camera?

- The spatial sampling used for the tools will be 0.05 ″.

- The relative accuracy for registering and superimposing the sources using the catalog information with the field-of-views of the AO sensors and science instruments should be 0.1 ″.

- The tool should be able to propose a few candidate AO setup based on the guide star candidates.

- The selection of the AO setup will use template files, that can be modified by the support team. The templates will be defined as the AO systems are designed in detail.

- Options should be provided for taking into account additional TBD requirements from dithering, nodding around the science object(s).

### Select and load observing conditions setup

This section describes the requirements for selecting and loading user-defined options:

- This setup is optional and will be likely used in the later phases of observation planning. By default the AO guide star tools will adopt default values for these parameters.

- The setup for the observing conditions setup includes the following TBD parameters:

  - Observing dates, used to derive:

    - Observability hours and range

    - Airmass

    - Telescope pointing direction

    - Sky brightness:

      - % of full moon and distance

      - time to 12 deg twilight

  - Cn2 and Seeing from TBD typical cases of seeing at zenith

    - the user should be able to select from a few typical situations: 20 percentile seeing, 50 percentile seeing, variable seeing and jet-streams-seeing.

  - Other TBD science requirements: priority to EE, SR, smooth PSF over the field?

### Operational

The operational performance requirements are indicated as follow:

  - The tool will read a file or request input parameters for Observing Date (UT or Local Time).

  - The tool will calculate and display the observability and airmass of the target for the given UT or LST date. It will include any telescope-related limitations (e.g, laser or telescope elevation limits).It will estimate and display the % of full moon brightness, as well as angular distance to the moon and the sun.

  - The tools will search for suitable guide stars from astronomical data bases and

## Implementation Requirements

### Design Requirements

# Science Observations Simulation Tools Requirements

## Performance Requirements

### Parametric

### Operational

## Implementation Requirements

## Design Requirements

### Technological

### Standards Related

### Integration Related

# Interface Requirements

# Reliability Requirements

## Risk and Mitigation

# Service and Maintenance Requirements

# Documentation Requirements

# Glossary