Palomar Adaptive Optics Test Plan

| Title | Switching Reflecting Spot for NGS |
| --- | --- |
| Date |  |
| Lead |  |
| Time requested | 1h |
| Required conditions | Optics down, in lab or on the telescope (daytime) |

Purpose Shift the dichroic spot from the all dichroic to the NGS reflecting spot

Previous analysis

Test procedure - Register and lock on white light - Move acq_z to 15800 (focused on dichroic beamsplitter) - Move the wfs_z to 80000 - Illuminate the beamsplitter with a lamp - Shift the spot location by noting the current location of the rotary mount and rotating 10.6mm from there (1 full turn = 1mm) - Remove lamp and return wfs_z 10000 - Register and lock on the white light - Check visually if the spot appears centered (this is easier to see if you switch to the laser and turn it up all the way) - If it is not centered in X, you will have to rotate the optic - Move wfs_z out of the way if necessary - Loosen the threaded ring on the optic (no need to remove it). - Slightly push the optic out in order to loosen the rubber ring that holds it in place. - Rotate and shift the optic in its mount in order to visually center the spot in the acquisition camera. - Switch to the white light, register and lock - With all the loops open, move ttm_a in .25” steps until the white light spot shows up on the edge of the reflecting spot in the acquisition camera. Note the number of steps. - Close the TT loop to recenter, then open it. - Move ttm_a in -.25” steps until the white light shows up on the opposite edge of the reflecting spot in the acquistion camera. Note the number of steps - If the number of steps on each side differ by more than 2, repeat step 9 and retest with the TTM - Check ttm_b in the same way - If the number of steps on each side differ by more than 2, adjust the rotary stage slightly and retest with the TTM - Note the final difference in steps in X and Y.