Palomar Adaptive Optics Test Plan

| Title | Internal Focusing for LGS operations |
| --- | --- |
| Date | 1/5/2007 V4 |
| Lead | M. Troy |
| Time requested | 1.5 hr |
| Required conditions | - |

Purpose We outline here the procedure to co-focus the LOWFS, HOWFS and PHARO for night time operation. All work is done with the white light.

Test procedure - Setup - Go to DNGS mode - lgs_x should be in the LGS position (92000) - LOWFS - load zero offsets - Set rate to 500Hz - HOWFS - load zero offsets - Set rate to 200 Hz - Load pixel_gains_lse_sep03 - Take HOWFS background - Acquire white light in HOWFS with power at 35, Lock TT and DM loops - Make sure LOWFS is off - Take and load a white light flat map - IDL> ao_make_dm_flat_map,’filename’ - co-focus LOWFS and HOWFS to white light - Lock in dual mode - Acquire LOWFS using acqview - Click ACQ - Click on spot when it appears - Click “Send LOWFS” - Take LOWFS background - From lgstools, click “zero cent” - Lock TT on LOWFS - If HO residuals are close, lock DM on HOWFS - zero LOWFS focus using wfs_z - start, lgsfoc, set the LOWFS zero point to zero, set gain to 0.25 - Turn lgsfoc “on” - Note: insure HOWFS residuals show no focus, may need to decrease DM proportional servo loop term. - turn off lgsfoc - note the old and new wfs_z position - re-focus white light to zero focus on DM - Go to NGS mode - close HOWFS loops on white light - Set DM proportional gain to zero - Setup TAO to offload focus to telescope, but prompt user (1mm of secondary motion = 22.9 mm of white_z (or wfs_ motion) - Use the TAO prompts to move the ‘white_z’ motor to zero the focus on the DM (optional). - Open and close DM loop after moving white_z - Note old and new white_z position - Reset DM proportional gain to 0.02 - Make new white light flat map - Re-focus acquisition camera - In acqview, click ACQ - Set the video camera to integrate at 1Hz, saving 1 frame a second - Set white light power to 20 - Check to make sure the peak counts are less then ~100. If necessary decrease the white light power - Shut off white light and take and save a background using ao_plot_vid_image in IDL, call it acq_wl_bkgd - Turn the white light back on and focus using - ao_focus_loop, ‘current acq_z position’ -600, 300, 5, ‘acq’ ,’acq_wl_’, sky=’acq_wl_bkgd’, time_to_avg=2 - move acq_z to optimal position and note new position - Save new AO config file called LGS - PHARO alignment - re-focus PHARO - lock HOWFS on zero offsets - focus PHARO to maximize Strehl/minimize FWHM. - re-tune centroid offsets in HOWFS

Results and conclusions 4/2/07 (v4) wfs_z: starting position: 17200, ending position: 18172 white_z: starting position: 25750, ending position: 25750 acq_z: starting position: 12700, ending position: 12500 1/5/07 (V3.1) wfs_z: starting position: 10800, ending position: 10540 white_z: starting position: 29790, ending position: 33256 acq_z: starting position: 14600, ending position: 14600 12/6/06 wfs_z: starting position: 10400, ending position: 10800 white_z: starting position: 29792, ending position 29792 acq_z: starting position: 14600, ending position: 14600 Did tune up on PHARO, no refocus req’d 10/11/2006 wfs_z: starting position:10650 um, ending position: 7260 um white_z: starting position: 27500 um, ending position: 29792 um acq_z: starting position: 14600 um, ending position: 14740 um did re-focus and tune up PHARO 9/7/2006 wfs_z: starting position:10659 um, ending position: 7150 um white_z: starting position: 27500 um, ending position: 30917 um saved as 060908a did not re-focus acq camera, procedure above was not written did re-focus and tune up PHARO