Palomar Adaptive Optics Test Plan

| Title | BTO shutdown |
| --- | --- |
| Version | 1.1 |
| Date released | 5/15/2007 |
| Lead | Laser Operator |
| Time requested | 30 min. at end of last night of an observing run. |
| Required conditions | N/A |

Purpose Shut down and protect optics and electronics of the BTO.

Test procedure Prime Focus Requires: Kimwipes, tape, compressed air, 0.05" allen key, radio. - Dust off LLT primary and install primary mirror cover. - Turn off LLT FSM controller (on/off switch on front of controller). - Dust off LLT FSM and install blue metal cover (stored in box in PF elevator, tiny screws require 0.05" allen key). - Wrap up Q3 beamsplitter with kimwipe and tape. - Wrap up LLT diagnostics beamplitter wedge with kimwipe and tape. Trolley Requires: Kimwipes, tape, compressed air. - Sent trolley to bottom of track (move trolley 5100000). - Dust and cover trolley mirror with kimwipe and tape. - Dust and cover Q1 beamsplitter with kimwipe and tape. - Send trolley to storage position (move trolley 2650000). - Turn off all BTO electronics on network power switch: - telnet viswa_power; /off 2, 3, 4, 6; /x Coude Lab Requires: compressed air, 0.05" allen key. - Turn off 660nm laser power supply (on/off button on front). - Turn off Newport FSM controller, ESP300 controller, and delay generator. - Dust off Coude FSM and Install blue metal cover (stored on top edge of bench). - Cover BTO optical bench. - Turn off tiny switches on backs of both microphones, and turn off microphone power supply. - Turn off video camera power supply (can be done by reaching between old optics above and to the right of the Coude bench).

Results and conclusions