Palomar Adaptive Optics Test Plan

| Title | Laser Acquisition on science nights |
| --- | --- |
| Version | 5.1 |
| Date released | 4/4/07 |
| Lead | A. Bouchez,M. Troy |
| Time requested | 30 min. |
| Required conditions | N/A |

Purpose Acquire LGS, focus LLT, determine LGS magnitude and spot size,

Test procedure - Prepare to project laser - Telescope at zenith. - Dome open. - Mirror cover open. - Raise windscreen. - Spotters on station. - Radar on and adjusted. - ASCAM and IRCAM on and alarms reset. - BTO configured for 589nm laser at zenith. - Laser alarm system ready. - AO Setup - Check stimulus position on HOWFS. - Move SSMs to center stimulus on HOWFS if necessary. - Configure AO to look at sky - Verify LLT FSM mirror position (last good position, previous night) - Confirm that best available flatmap is loaded. - Set TAO to LGS mode - Move LGS_X to NGS position. - Move ACQ_Z to LGS position (9950 um) - Focus LLT to last good position (or NGS focus - 300). bto move llt_focus 11780 - check/set laser focus to correct value bto move laser_focus 7000 (4/4/07 UT) - Set Acq ON, integration = 2s. - In an appropriate experiment directory: IDL> ao_plot_vid_image - Project laser. - Move LGS to a clear location on Acq. offset llt_a +X=up; offset llt_b +X=left If LGS is not in the Acq. FOV, use ellipticity of dichroic spots (apex points to LGS). - Focus LLT on Na layer - Manually offset LLT focus in 30um steps (bto offset llt_focus...), checking FWHM with ACQVIEW Gaussian fit at each position: -30, 0, 30 - Determine laser focus (after major laser changes) - Block laser, save a 2s background image using ao_plot_vid_image. - Rough focus laser spot, looking at image in acq. camera bto_control “offset laser_focus …” in steps of 2000 - ao_focus_loop, 0, 2000, 7, ‘laser_focus’,’save_name, sky=’sky_name’,time=6.0 - bto move laser_focus XXX - Save a final image of the LGS. Note file name below.

Results and conclusions