Palomar Adaptive Optics Test Plan

| Title | PHARO pupil motion |
| --- | --- |
| Version | 1.0 |
| Date released | 4/9/2008 |
| Lead | A. Bouchez |
| Time requested | 1 hr. |
| Required conditions | Dome closed |

Purpose Measure the amount of pupil motion observed by PHARO over the accessible sky.

Previous analysis None which I am aware of.

Test procedure A. Verify pupil alignment at zenith - Install the pupil mask on the deformable mirror. Gloves required. - Record a 2s dark with the PHARO block in to subtract from subsequent images. - Set up PHARO for pupil imaging: Pupil imaging, 25 mas field, J filter, standard cross pupil mask. - Check the pupil image on PHARO using white-light stimulus. If necessary, adjust PHARO’s pitch and yaw to align the PHARO pupil mask to the DM pupil mask to better than 5 pixels. - Remove the DM pupil mask. - Turn on the high lamps, open the mirror cover, set stimulus selection motor to “sky”. - Check the telescope pupil on PHARO. If necessary, adjust the AO system’s FM1 mirror to align the PHARO pupil mask to the telescope secondary to better than 5 pixels. B. Measure pupil motion as a function of zenith angle. - System configuration: - DM pupil mask not installed. - Mirror cover open, high lamps on. - PHARO configuration: Pupil imaging, 25 mas field, J filter, standard cross. - Record a 10s dark (PHARO block in). - Record 10s pupil images at the following HA/Dec positions. Fill in image numbers in the table below. -6:00; -4:00; -2:00; 0:00; +2:00; +4:00; +6:00. -20.0; N/A; N/A; N/A; N/A. 0.0. +20.0. +40.0. +60.0.

Results and conclusions