Keck Adaptive Optics Note 456

Next Generation Adaptive Optics:

System Requirements Document

Version 0.2

January 19, 2007

1	Introduction	1

2	Scope and Applicability	1

3	References	2

3.1	Related Documents	2

3.2	Referenced Drawings	2

4	Revision History	2

5	Background	3

5.1	Purpose	3

5.2	Motivation for the Development of NGAO	3

5.3	Overview	3

5.4	System Overview	3

6	Science Overall Requirements	4

6.1	Purpose and Objectives	4

6.2	Performance Requirements	4

6.3	Science Instrument Requirements	5

7	Observatory Overall Requirements	7

7.1	Purpose and Objectives	7

7.2	Facility Requirements	7

7.3	Science Instrument Requirements	8

7.4	Operational Requirements	9

8	Optical Requirements	11

8.1	Purpose and Objectives	11

8.2	Performance Requirements	11

8.2.1	Parametric Performance Requirements	13

8.2.2	Operational Performance Requirements	13

8.3	Implementation Requirements	13

8.4	Design Requirements	13

9	Mechanical Requirements	14

9.1	Purpose and Objectives	14

9.2	Performance Requirements	14

9.2.1	Parametric Performance Requirements	14

9.2.2	Operational Performance Requirements	14

9.3	Implementation Requirements	14

10	Electronic/Electrical Requirements	15

10.1	Purpose and Objectives	15

10.2	Performance Requirements	15

10.2.1	Parametric Performance Requirements	15

10.2.1.1	Electrical Power and Thermal Dissipation	15

10.3	Implementation Requirements	15

10.3.1	Feature Implementation Requirements	15

10.4	Design Requirements	16

10.4.1	Technological Design Requirements	16

10.4.2	Integration Related Design Requirements	16

11	Safety Requirements	17

11.1	Purpose and Objectives	17

11.2	Scope	17

11.3	Laser Safety Requirements	17

11.4	Laser Projection Safety Requirements	17

11.4.1	Aircraft Safety	17

11.4.2	Space Command	17

12	Software Requirements	18

12.1	Purpose and Objectives	18

12.2	Scope	18

12.3	Performance Requirements	18

12.3.1	Parametric Performance Requirements	18

12.3.2	Operational Performance Requirements	18

12.3.2.1	Overhead	18

12.3.2.2	Error Recovery	18

12.4	Implementation Requirements	18

12.4.1	Feature Implementation Requirements	18

12.4.1.1	User interfaces	18

12.4.1.2	Data Reduction Pipeline	19

12.4.1.3	Target Computers	19

12.5	Design Requirements	19

12.5.1	Technological Design Requirements	19

12.5.2	Integration Related Design Requirements	19

13	Interface Requirements	20

13.1	Purpose and Objectives	20

13.2	Performance Requirements	20

13.2.1	Parametric Performance Requirements	20

13.2.2	Operational Performance Requirements	20

13.3	Implementation Requirements	21

13.3.1.1	Optical Requirements	21

13.3.1.2	Mechanical	21

13.4	Design Requirements	21

13.4.1	Technological Design Requirements	21

13.4.2	Integration Related Design Requirements	21

13.4.2.1	Optical Interface	21

13.4.2.2	Mechanical Interface	21

13.4.2.3	Electrical/Electronic Interface	21

14	Reliability Requirements	22

14.1	Purpose	22

14.2	Scope	22

14.3	Performance	22

15	Spares Requirements	22

16	Service and Maintenance Requirements	22

17	Documentation Requirements	23

17.1	Documentation Package	23

17.2	Drawings	23

17.2.1	Required Drawings	23

17.3	Electrical/Electronic Documentation	23

17.4	Software	23

18	Glossary	24

Error! No table of figures entries found.

Table 1: NGAO Graphical User Interfaces	18

Table 2: Glossary of Terms	24

# Introduction

This document describes the requirements for the Next Generation Adaptive Optics (NGAO) system to be built for the W. M. Keck Observatory (WMKO).

The requirements in this document are at a draft level appropriate for the system design phase.  Further development of the requirements will take place in the next phase of the project (preliminary design).  In particular, parametric performance requirements given at this stage are intended to indicate the scope and format of the requirements, but do not in all cases establish final values for the specified parameters.  In some cases values for these parameters have yet to be established and are given as TBD.

A more generic set of requirements for new WMKO instrumentation is described in the Observatory’s “Instrumentation Baseline Requirements Document.”  These requirements are also applicable to NGAO.  The NGAO System Requirements Document will take precedence over the “Instrumentation Baseline Requirements Document” in the event of a conflict.

It is important to understand that at this stage of development the requirements provide a basis for identifying the parameters that will be part of the system’s specifications, but the values given are subject to change as the development continues.  During the next phases of the project work will be done to refine the requirements for review at the preliminary design review.  The final requirements to be reviewed at the detailed design review will form the basis for the acceptance test criteria for the instrument.

The purpose of this requirements document is to define and communicate the Observatory’s expectations for the NGAO-specific design and implementation.  A requirements document describes the system in terms of the needed scientific and technical performance.  The document also expresses specific requirements for implementation or design where those requirements are essential to satisfactory integration and interoperation of NGAO with the observatory systems.

The document avoids prescribing specific design or implementation solutions except for solutions that embody the Observatory’s unique knowledge or experience.  The document establishes requirements for the NGAO that will guide the design through the detailed design phase.

# Scope and Applicability

This document establishes requirements for all aspects of NGAO beyond those already specified in the “Instrumentation Baseline Requirements Document”.  This document also establishes requirements for changes to related Keck telescope subsystems and software where required.

This document does not address the requirements for the science instruments that will work with NGAO, although it does cover the NGAO interfaces to these instruments.  Separate system requirements documents will need to be prepared for each of these instruments as part of their design process.

# References

## Related Documents

- Instrumentation Baseline Requirements Document.

- KAON 455.  Science Case Requirements Document.

## Referenced Drawings

Table X lists the drawing numbers, revisions and date, source and title for all drawings referenced in this document.

# Revision History

| Version | Date | Author | Reason for revision / remarks |
| --- | --- | --- | --- |
| 0.1 | Jan. 16, 2007 | Wizinowich | Initial version |

# Background

## Purpose

The purpose of the background section of this document is to provide context and related information for the requirements defined in later sections of this document.

## Motivation for the Development of NGAO

The motivation for the development of NGAO is provided in the NGAO proposal (KAON 400) and NGAO proposal executive summary (KAON 399).  Further scientific motivation is provided in the NGAO science case requirements document (KAON 455).

## Overview

## System Overview

# Overall Requirements

## Science Requirements

### Purpose and Objectives

The purpose of the science overall requirements section is to summarize and convey requirements that apply generally to the overall NGAO system and its accessories.  These are based on the Science Case Requirements Document (KAON 455) and, in this initial release, also on the NGAO proposal (KAON 399).

### Performance Requirements

A first cut at the performance requirements, based on the science requirements, is taken directly from the NGAO proposal.  These will be updated as the science case requirements and performance budgets become better defined.

| Budget | Recommended Value | Based on Requirements |
| --- | --- | --- |
| Throughput to Science Instrument (telescope + AO) | >= 70% at 0.6-5.5 um | Science object wavelength |
|  | >= 60% at 5.5-14 um | Magnitude limit |
|  |  | Desired SNR |
|  |  | Typical exposure time |
| Emissivity to Science Instrument (telescope + AO) | <= sky emissivity at K, L & M | Science object wavelength |
|  |  | Magnitude limit |
|  |  | Desired SNR |
|  |  | Typical exposure time |
|  |  | Companion sensitivity |
| Wavefront Error Budget | 140 nm for 1% sky coverage (celestial sphere average, z=30) | Image quality |
|  | 160 nm for 20% sky coverage | Sky coverage |
|  | 190 nm for 80% sky coverage |  |
| Photometric accuracy | 0.01 mag at 0.7-2.5 um for < 5" from H < 16 NGS |  |
|  | 0.02 mag at 0.7-3.5 um for < 10" from H<16 NGS |  |
|  | 0.05 mag at 0.9-2.5 um for < 20" off-axis & 20% sky coverage |  |
|  | 0.1 mag at 0.7-2.5 um for < 20" off-axis & 20% sky coverage |  |
| Astrometric accuracy | 0.1 mas for Galactic Center |  |
|  | < 10 mas for 0.7-3.5 um & 30% sky coverage |  |
|  | < 50 mas for 0.7-3.5 um & 50% sky coverage |  |
| Polarimetry accuracy | < 0.5% |  |
| Companion Sensitivity | > 4 magnitudes at 0.055" at 1-2.5 um for Galactic Center |  |
|  | > 10 mags at 0.5" at 0.7-3.5 um for 30% sky coverage & < 20" object diameter |  |
| Observing Efficiency | <= 20 min overhead per night | Facility-class |
|  | <= 120 sec between end of slew & ready for science exposure |  |
|  | <= 10 sec between start of nod command & ready for science exposure |  |
|  | <= 120 sec to switch between LGS & NGS mode |  |
|  | <= 600 sec to switch between NGS & LGS mode (assuming daytime setup) |  |
|  | <= 120 sec to switch between science instruments (assuming daytime setup) |  |
| Up-time | <= 5% of time lost to problems | Facility-class |
| Electrical Power | <= 10 kW for AO system |  |
|  | <= 20 kW for laser system |  |
| Thermal Dissipation | <= 100W from AO system |  |
|  | <= 100W from laser system |  |
|  | <= 50W at top-end |  |
| Weight | <= 10,000 kg for AO system |  |
|  | <= 10,000 kg for laser system |  |

The atmosphere and telescope parameters assumed for achieving these numbers need to be summarized in this KAON or a document that can be referenced.

### Science Instrument Requirements

The NGAO system must be capable of supporting the following science instruments (in rough order of priority), based on the NGAO proposal:

- Visible imager.  Wavelength range = 0.6 to 1.1 µm.  Field of view = 20”x20”.  Image sampling = 6 mas pixels.

- Near-IR imager.  Wavelength range = 1.0 to 2.45 µm.  Field of view = 20”x20”.  Image sampling = 10 mas.

- Deployable near-IR Integral Field Unit (IFU).  Wavelength range = 1.0 to 2.45 µm.  Field of Regard = 1.5’x1.5’ with 3x3.4” fields of view for each IFU.  Image sampling = 100 mas.

- Near-IR IFU.  Wavelength range = 1.0 to 2.45 µm.  Field of view from 2”x1.25” to 16”x5”.  Image sampling = 20 to 100 mas.

- Visible IFU.  Wavelength range = 0.6 to 1.1 µm.  Field of view from 1.2”x1.36” to 12”x6.8”.  Image sampling = 20 to 100 mas.

- Thermal near-IR imager.  Wavelength range = 3.0 to 5.3 µm.  Field of view = 25”x25”.  Image sampling = 25 mas.

It is TBD whether the NGAO system will need to support any of the existing science instruments (NIRC2, NIRSPEC or OSIRIS).  The Interferometer and OHANA are discussed in section 7.3.

## Observatory Overall Requirements

### Purpose and Objectives

The purpose of the overall requirements section is to convey requirements that apply generally to the overall instrument and its accessories based on the Observatory’s requirements.

Note that the Observatory’s standard requirements for all new instrumentation are summarized in the Instrumentation Baseline Requirements Document.  Additional Observatory requirements specific to NGAO are listed in the following sections.

### Facility Requirements

The following are requirements imposed by the nature of the existing facility.

| # | Observatory Facility Requirements | Discussion |
| --- | --- | --- |
| 1 | The NGAO system & science instruments should be located on the Nasmyth platform of one of the Keck telescopes | The Keck telescope foci and Nasmyth deck storage locations are already heavily utilized. The current AO systems occupy the left Nasmyth platform locations of both telescopes. HIRES occupies the right Nasmyth on Keck I while DEIMOS and NIRSPEC share the right Nasmyth on Keck II. The Cassegrain foci are occupied by LRIS (and MOSFIRE in the future) on Keck I and by ESI on Keck II. The bent Cassegrain ports are believed to have inadequate space and weight capacities. The prime focus could potentially be available but there would be many constraints on an instrument at this location. The most viable option is in the location of an existing AO system. Alternatives would be to decommission HIRES or for the existing AO system and the NGAO system to be able to share the same platform. |
| 2 | The NGAO system should accommodate the entire Keck pupil | The Keck primary has a maximum edge-to-edge diameter of 10.949 m. |
| 3 | If the existing f/15 or f/40 secondary mirrors are used then the NGAO system will be constrained by the resultant f/#, focal plane and pupil location | Both telescopes have f/15 secondary mirrors, as well as chopping secondary units that can accommodate f/25 and f/40 secondary mirrors. The choice of f/15 secondary mirrors for the current AO systems was largely driven by the resultant reduced size of the AO systems and the availability of PCS (Phasing Camera System) via a rotation of the tertiary mirror. The inability of the current systems to chop at the telescope pupil has been a limitation for thermal IR observations. The focal length of the telescope with the f/15 secondary mirror is 150 m. The 10.949 m primary corresponds to an f/13.66 beam with an exit pupil diameter of 1.460 m located 19.948 m in front of the focal plane. |
| 4 | The NGAO facility must not compromise the performance of a non-NGAO instrument when that instrument is being used for science or engineering | This requirement is intended to ensure that the NGAO facility, when not in use, does not introduce vibrations or stray light that might impact the performance of another science instrument or an engineering instrument such as PCS. |
| 5 | The NGAO facility must not compromise the performance of the telescope when the telescope is used for non-NGAO observations. | The NGAO system should not impact the dynamic performance of the telescope through vibrations or different telescope dynamics. If NGAO hardware is to be mounted in the top-end then it must be designed/implemented not to compromise the secondary mirror performance. |
| 6 | The NGAO system should provide compensation for field rotation | The Alt-Az telescope design requires compensation for field rotation in order to keep the science field fixed on the science instrument. |
| 7 | The NGAO system should provide compensation for pupil rotation | The Alt-Az telescope design and the irregular shape of the Keck primary mirror require that NGAO system provide appropriate compensation for pupil rotation. Examples: The existing Keck AO system updates the reconstructor to compensate for pupil orientation. The NIRC2 coronagraph mask rotates to match the rotating pupil. |
| 8 | The NGAO system should provide compensation for LGS projector rotation | The Alt-Az telescope design will cause the laser projector (and the resultant LGS asterism) to rotate with respect to the Nasmyth platform. Compensation will be required to maintain the off-axis LGS on the corresponding wavefront sensor. |

### Science Instrument Requirements

In addition to the science instrument requirements specified in the previous section the NGAO facility must allow the Observatory to continue supporting Interferometer science with the two Keck telescopes.

| # | Observatory Instrument Requirements | Discussion |
| --- | --- | --- |
| 1 | The NGAO facility should support the Keck Interferometer (KI) | The KI dual star modules (DSM) currently move into both AO enclosures on rails to feed the KI. The requirement to feed the KI requires that collimated light can be fed to the DSM and that the field rotation, pupil rotation, longitudinal dispersion and polarization from the NGAO system and the AO system on the other telescope be identical. See KAON 428 Implications and Requirements for Interferometry with NGAO. |
| 2 | The NGAO system should support the OHANA interferometer | Injection modules are currently placed on each AO bench to feed an optical fiber that goes to the KI. In future the output from these fibers will be interfered with those from multiple telescopes. |

### Operational Requirements

The purpose of this section is to document the Observatory’s overall requirements for the support of science operations.

| # | Observatory Operational Requirements | Discussion |
| --- | --- | --- |
| 1 | The NGAO system must be facility-class | Facility-class has many implications on safety, operability, reliability, maintainability, lifetime, documentation, configuration management, etc. |
| 2 | The NGAO system must complete an operations transition review where operational responsibility is transferred from development to operations | This has implications on defining transition requirements and on training. |
| 3 | The time between decommissioning an AO capability on the telescope where the NGAO system is to be installed and making NGAO available for limited shared-risk science must not be longer than 6 months | Down time Impact on Interferometer and AO science at the Observatory. |
| 4 | The telescope downtime required to implement NGAO must not be longer than 5 days |  |
| 5 | The NGAO system must be capable of supporting 275 nights/year of science operation |  |
| 6 | The Mauna Kea laser projection requirements must be satisfied | This includes requirements on laser power, wavelength, laser traffic control participation, aircraft safety and space command. |
| 7 | The NGAO system must operate within specifications under the normal summit temperature and humidity conditions |  |
| 8 | The NGAO facility should not require more than two engineering nights per year for system maintenance |  |
| 9 | The NGAO facility should not routinely require more than 30 minutes of daytime telescope restriction on a science night |  |
| 10 | The NGAO facility should be designed to keep the total annual operational personnel and non-personnel costs below the following budgets: - Four FTEs and $400k (FY07 dollars) in non-personnel costs for the first 100 nights/year of science operations - An additional cost of 0.5 FTEs and $50k (FY07 dollars) in non-personnel costs for each additional 50 nights of science operations. |  |

# Optical Requirements

## Purpose and Objectives

The purpose of this section is to describe requirements for the performance, implementation and design of the NGAO optical system.

Preliminary optical parametric performance requirements are grouped into two categories, typical parameters and goal parameters.  Requirements for typical parameters are given where enough is known to establish a range of values for the listed parameters.  Requirements for goal parameters are given where scientific or technical uncertainty prevents a full definition of the achievable range of values for the listed parameters.

During the preliminary and detailed design phases these requirements (which at least in part drive the specifications for the system) will be refined into a set of specifications.  In this process goal parameters will be refined through research and development activity into typical values.  The minimum and maximum values for parameters listed in the requirements for goal parameters indicate the acceptable bounds on worst-case performance.

## Performance Requirements

| # | Optical Performance Requirements | Discussion |
| --- | --- | --- |
| 1 | A wavelength range of 0.6 to 1.0 µm must be provided to the visible science instruments (imager and IFU) |  |
| 2 | A wavelength range of 1.0 to 2.45 µm must be provided to the NIR science instruments (imager, IFU and deployable IFU) |  |
| 3 | A wavelength range of 3.0 to 5.3 µm must be provided to the thermal NIR imager |  |
| 4 | A wavelength range of 1.1 to 14 µm must be provided to the Interferometer |  |
| 5 | A wavelength range of 1.1 to 2.45 µm must be provided to the OHANA injection module |  |
| 6 | The telescope plus NGAO throughput to the science instruments shall be ≥ 70% at 0.6-5.5 µm |  |
| 7 | The telescope plus NGAO throughput to the science instruments shall be ≥ 60% at 5.5-14.0 µm |  |
| 8 | The telescope plus NGAO emissivity to the science instrument shall be ≤ the sky emissivity at K, L and M |  |
| 9 | A field of view of 20”x20” must be provided to the science imagers (visible and NIR) |  |
| 10 | A field of view of 12” diameter must be provided to the visible IFU |  |
| 11 | A field of view of 16” diameter must be provided to the NIR IFU |  |
| 12 | A field of regard of 1.5’ diameter must be provided to the NIR deployable IFU |  |
| 13 | A field of view of 25”x25” must be provided to the thermal NIR imager |  |
| 14 | A field of view of 1’ diameter must be provided to the Interferometer |  |
| 15 | A field of view of 5” diameter must be provided to the OHANA injection module |  |
| 16 | The field of view provided to the science instruments must be centered on the telescope elevation axis to within 2” |  |
| 17 | The focal ratio provided to the science instrument should be TBD |  |
| 18 | The exit pupil location provided to the science instrument should be TBD |  |
| 19 | The rms wavefront error for the combined telescope, NGAO system and visible and NIR imagers shall be TBD for the following TBD conditions |  |
|  | The rms wavefront error for the combined telescope, NGAO system and visible and NIR IFUs shall be TBD for the following TBD conditions |  |
|  | The rms wavefront error for the combined telescope, NGAO system and deployable NIR IFU shall be TBD for the following TBD conditions |  |
|  | The rms wavefront error for the combined telescope, NGAO system and thermal NIR imager shall be TBD for the following TBD conditions |  |
|  | The rms wavefront error for the combined telescope and NGAO system delivered to the Interferometer shall be TBD for the following TBD conditions |  |
|  | The residual static wavefront errors from the NGAO system shall be ≤ 50 nm rms as delivered to the visible and NIR imagers |  |
|  | The NGAO system shall introduce ≤ TBD % polarization in the path to the TBD science instruments |  |
|  | The stability of the field produced by the NGAO system shall be TBD as delivered to the TBD science instruments |  |
|  | Instability must not decrease the Strehl on the visible and NIR imagers by more than 5% during any 1-hour period or over a 3 ºC temperature change | The image improvement provided by the facility must be stable over periods of hours, under stable atmospheric conditions. The wavefront sensor calibrations must be stable with time, temperature and telescope position. This requirement does not include performance degradations due to changing atmospheric conditions, or different magnitude and location of NGS or LGS |
|  | Instability must not decrease the Strehl on the visible and NIR imagers by more than 10% during any 24-hour period or over a 5 ºC temperature change |  |

| # | Optical Performance Requirements | Discussion |
| --- | --- | --- |

### Parametric Performance Requirements

### Operational Performance Requirements

## Implementation Requirements

## Design Requirements

# Mechanical Requirements

## Purpose and Objectives

The purpose of this section is to describe requirements for the performance, implementation and design of the NGAO mechanical systems.

The mechanical requirements address issues of design, reliability and maintainability.  Based on experience with previous instruments the observatory is sensitive to certain aspects of performance, implementation and design that have proven to be important factors in the up time of its instruments.  The mechanical requirements section has as a main objective the description of the expected performance, features and configuration of the instrument’s mechanical systems.  A secondary objective is to identify specific areas where experience indicates particular attention is required with respect to performance, implementation or design.

In this revision of the document some of the mechanical requirements are very detailed, and others are broader.  These broader requirements may need to be broken down into more detailed requirements in a further revision of this document.  In the case of parametric requirements, many of the values given are starting points for a more detailed analysis that will take place in the next phase of the instrument’s design.

## Performance Requirements

### Parametric Performance Requirements

### Operational Performance Requirements

## Implementation Requirements

| Budget | Recommended Value | Based on Requirements |
| --- | --- | --- |
| Weight | <= 10,000 kg for AO system on telescope |  |
|  | <= 150 kg for hardware in the top-end module |  |
|  | <= 10,000 kg for laser system on telescope |  |

# Electronic/Electrical Requirements

## Purpose and Objectives

The purpose of this section is to describe requirements for the performance, implementation and design of the NGAO electronic and electrical systems.

The electronic/electrical requirements address issues of safety, design, reliability and maintainability.  Based on experience with previous instruments the observatory is sensitive to certain aspects of performance, implementation and design that have proven to be important factors in the up time of its instruments.  The electronic/electrical requirements section has as a main objective the description of specific requirements for implementation and design.

A key consideration is the safety of personnel and equipment, and proper electrical design and implementation practices in compliance with recognized standards are an essential aspect of electrical safety.  A second consideration is the electromagnetic compatibility of the instrument with the observatory systems, and specific implementation and design requirements are given to aid in achieving the required electromagnetic emissions and susceptibility performance.

## Performance Requirements

### Parametric Performance Requirements

#### Electrical Power and Thermal Dissipation

The preliminary electrical power and thermal dissipation requirements for NGAO are given in the following table:

| Budget | Recommended Value | Based on Requirements |
| --- | --- | --- |
| Electrical Power | <= 10 kW for AO system |  |
|  | <= 20 kW for laser system |  |
| Thermal Dissipation | <= 100W from AO system |  |
|  | <= 100W from laser system |  |
|  | <= 50W at top-end |  |

## Implementation Requirements

### Feature Implementation Requirements

## Design Requirements

### Technological Design Requirements

### Integration Related Design Requirements

# Safety Requirements

## Purpose and Objectives

Safety is the paramount concern for all activities at the observatory.  The purpose of this section is to provide requirements related to specific safety concerns during the operation and handling of NGAO.

## Scope

Unless otherwise indicated all of the requirements of this section apply to all components of NGAO.  The general safety requirements are already contained in the Instrumentation Baseline Requirements Document.  This section in addition covers the additional laser safety and laser projection safety requirements.

## Laser Safety Requirements

## Laser Projection Safety Requirements

This section covers the requirements on safety for projecting the laser beams outside the dome.

### Aircraft Safety

### Space Command

# Software Requirements

## Purpose and Objectives

The software requirements section describes requirements for performance, implementation and design.  Based on experience with previous instruments the observatory is sensitive to certain aspects of performance, implementation and design that have proven to be important factors in the up time of its instruments.  The software requirements section has as a main objective ensuring compatibility of the NGAO software with existing observatory software systems.  A secondary objective is guiding the selection of software architecture and implementation decisions towards those that fit within the software skill sets at the observatory in order to maximize the ability of the observatory to support and maintain the NGAO software.

## Scope

Unless otherwise indicated all of the requirements of this section apply to all software components of NGAO.

## Performance Requirements

### Parametric Performance Requirements

### Operational Performance Requirements

#### Overhead

#### Error Recovery

## Implementation Requirements

### Feature Implementation Requirements

#### User interfaces

Table 1: NGAO Graphical User Interfaces

| Name | Description |
| --- | --- |

#### Data Reduction Pipeline

#### Target Computers

## Design Requirements

### Technological Design Requirements

### Integration Related Design Requirements

None.

# Interface Requirements

## Purpose and Objectives

This section is reserved for interface requirements that are not addressed by other portions of the document.

## Performance Requirements

### Parametric Performance Requirements

None.

### Operational Performance Requirements

| # | Interface Requirements | Discussion |
| --- | --- | --- |
|  | An interface should be provided to offload tip/tilt errors to telescope pointing through the telescope drive and control system (DCS) |  |
|  | An interface should be provided to offload focus errors to the secondary mirror piston through DCS |  |
|  | An interface should be provided to offload coma errors to the secondary mirror tilt through DCS |  |
|  | An interface should be provided to offload segment stacking errors to the active control system (ACS) |  |
|  | An interface should be provided to write all appropriate NGAO keywords to the science instrument FITS header |  |
|  | An interface should be provided to allow NGAO to read/write science instrument keywords |  |

## Implementation Requirements

#### Optical Requirements

See §Error: Reference source not found.

#### Mechanical

See §Error: Reference source not found.

## Design Requirements

### Technological Design Requirements

None.

### Integration Related Design Requirements

#### Optical Interface

#### Mechanical Interface

#### Electrical/Electronic Interface

# Reliability Requirements

## Purpose

A process should take place to confirm that the NGAO system will provide a high level of reliability for a 10 year lifetime.

## Scope

Unless otherwise indicated all of the requirements of this section apply to all components of NGAO.

## Performance

System downtime should be minimized by a combination of component reliability, ease of repair, maintenance and appropriate sparing.

| # | Reliability Performance Requirements | Discussion |
| --- | --- | --- |
|  | ≤ 5% of observing time lost to problems |  |

# Spares Requirements

TBD

# Service and Maintenance Requirements

# Documentation Requirements

## Documentation Package

## Drawings

### Required Drawings

## Electrical/Electronic Documentation

## Software

# Glossary

Table 2 defines the acronyms and specialized terms used in this document.

Table 2: Glossary of Terms

| Term | Definition |
| --- | --- |
| AO | Adaptive Optics |
| DCS | Drive and Control System |
| FOV | Field Of View |
| FWHM | Full Width at Half Maximum. |
| IFU | Integral Field Unit |
| KAON | Keck Adaptive Optics Note |
| LGS | Laser Guide Star |
| MTBF | Mean Time Between Failures |
| NGAO | Next Generation Adaptive Optics |
| NGS | Natural Guide Star |
| NIR | Near InfraRed |
| SSC | Science Steering Committee |
| TBC | To Be Completed |
| TBD | To Be Determined |
| WMKO | W. M. Keck Observatory |
| WRT | With Respect To |