AO system outline

Architectural Assumptions

Overall top level requirements:

LGS WFS (what is sensed)

(HO)

NGS WFS (what is sensed)

(HO, LO, Tip/Tilt, Truth)

AO enclosures

(AO relay, Rotators, AO bench, Enclosure)

Diagnostics

(Calibration/Built-in-Test/Vibration mitigation/Telemetry/PSF knowledge/Atmospheric profile)

Acquisition

(LGS, NGS and Science)

Real Time Control Systems

(Tomography, Temporal Control)

Non Real Time Control Systems

(Motion, Sequencer)

Correctors

(Tip/Tilt, High Order)

Each above get its own sections for

Architectural Assumptions

Optical Requirements

Mechanical Requirements

Electronic/Electrical Requirements

Safety Requirements

Software Requirements

Interface Requirements

Reliability Requirements

Spares Requirements

Service and Maintenance Requirements

Laser facility functional requirements

Architectural Assumptions:

Overall top level requirements:

Laser system

Diagnostics (M^2, beam profile, power, position, size)

Beam transport optics (e.g. fiber relay/ mirrors, control to keep laser stable at the output).

LTA (includes the asterism forming optics and uplink TT mirror and diagnostics)

Safety, interlock requirement (personnel, air-traffic/satellites, telescope collisions)

Uplink AO?

Laser facility functional requirements

Architectural Assumptions:

* median Na layer altitude = 90 Km

* maximum Na layer speed = 30 m/s

* baseline power = 150 W  split equally between the lasers

* the LGS asterism size and design are not arch. dependent

* 9 beacon asterism

* Each beacon can be moved around between 20" - 120" except the central beacon, which always remains at the center.

* each beacon has its own tip tilt

* No high order correction on the uplink.

* All performance specifications are stated at median conditions as stated in the top level of the Functional Requirements Document.

Overall top level requirements:

* Shall produce a guide star that brighter than 10 Mv [TBLU]

* LGS must point 60 deg. off zenith [To be looked up]

* LGS spot size must be xx [cm] at the sodium layer

* The throughput of the BTO

Laser system

optical requirements

1. The NGAO laser must have a M^2<1.2 at the exit of the laser.

2. beam size

3. LAser system must provide quasi-realtime diagnostic of power, beam profile, M^2.

mechanical requirements

1. must fit within a xx, yy, zz m volume

Electronic/Electrical Requirements

The laser system must be dissipate less than xx watts into the ambient air. Rest of the heat will be dissipated via. glycol cooling lines provided.

Safety Requirements

Shall follow ANSI regulations on laser safety.

Software Requirements

Interface Requirements

Reliability Requirements

Spares Requirements

Service and Maintenance Requirements

Documentation Requirements

Diagnostics

Optical requirements

Electronic/Electrical Requirements

Safety Requirements

Software Requirements

Interface Requirements

Reliability Requirements

Spares Requirements

Service and Maintenance Requirements

Documentation Requirements

Beam transport optics

Optical requirements

Electronic/Electrical Requirements

Safety Requirements

Software Requirements

Interface Requirements

Reliability Requirements

Spares Requirements

Service and Maintenance Requirements

Documentation Requirements

LTA (includes the asterism forming optics and uplink TT mirror and diagnostics)

Optical requirements

Electronic/Electrical Requirements

Safety Requirements

Software Requirements

Interface Requirements

Reliability Requirements

Spares Requirements

Service and Maintenance Requirements

Documentation Requirements

Safety, interlock requirement