NGAO Split 20” Relay / 120” dNIRI Architecture

Major Assumptions

- LGS pick-off first

- Pick-off to d-NIRI second & requires a dichroic changer (to switch which wavelengths go to dNIRI sensors and which goes to narrow field science instruments)

- D-NIRI provides platform for 2 TT, 1 TTFA, 1 TWFS over 180” dia field for both dNIRI and narrow field science

- PSF provided by 1 of dNIRI science channels

- All MEMS on TT platforms

- Remove laser dichroic for NGS WFS

- NGS WFS fed by narrow field relay (can therefore be closed loop).  Is a visible WFS adequate or do we also need an IR WFS?

- Need ADC for narrow field science instruments (but not for dNIRI units)

- Need ADCs for tip/tilt sensors (in order to sense across J & H simultaneously)

- Need TT, TTFA sensors to either allow offset tracking for differential atmospheric refraction or track the unit)

- Differential tracking required for non-science sensors on dNIRI platform to allow for a non-sidereal science object on narrow field science instruments

- Acquisition camera in front of narrow field relay

- Need an accurate registration system between acquisition camera and all NGS & science sensors

- Would be good to have 1 TT sensor close to science instrument (e.g., on IR science camera)

TBD

- What is the requirement on the TTM?  Could it do dither (i.e., ~3”)?

Risks

- Unclear that dNIRI can fit close enough to el journal

- Unclear enough space for dNIRI & narrow field at same time.

- Lots of non-common path for LGS between D-NIRI platform and narrow field science instruments (separate rotators, separate DMs, separate TTMs, non-common ADC).  This impacts relative TT and relative WFE and PSF calibration (and note that this varies in time).

- Open loop LGS

- Open loop LOWFS

- Registration of acquisition camera to NGS & science sensors

- The above 4 items imply significantly more calibration effort required

Requirements Issues

- Refractive relay won’t pass all the way from J to N-band for interferometer

- Narrow field relay won’t pass large enough field to interferometer