KAON #TBD

Next Generation Adaptive Optics System

Preliminary laser architecture

DESCRIPTION

(DRAFT)

Prepared By Jason Chin, Viswa Velur

Sept 27, 2007

VersionV1.2

# Revision History

| Revision | Date | Author (s) |  | Reason for revision / remarks |
| --- | --- | --- | --- | --- |
| 1.0 | Sept 19, 2007 | JC |  | Initial release |
| 1.1 | Sept 25 2007 |  | VNV | Added some details to section 4 |
| 1.2 | Sept 27, 2007 |  | JC | Updated section 4 and both Concepts A and B. |

# TABLE OF CONTENTS

Revision History	2

TABLE OF CONTENTS	3

1	Introduction	4

2	References	5

2.1	Input Material for this document	5

2.2	Acronyms and Abbreviations	6

3	Methodology	7

3.1	Financial Budget	7

3.2	Performance	7

3.2.1	Photon Return	7

3.2.2	Laser Spot Quality	7

3.3	Laser Reliability	7

3.4	Laser Servicing Requirements	7

3.5	Laser Operating Cost	7

3.5.1	Service and Maintenance	7

3.5.2	Operations	8

3.6	Risk	8

3.6.1	Technical Risk	8

3.6.2	Financial Risk	8

3.7	System Scaling Abilities and Flexibility	8

3.8	Integration with Telescope Facility	8

3.9	Rankings	8

4	Concept A.  Moving Laser Platform (MLP)	10

4.1	Motivation for MLP	10

4.2	Laser	10

4.2.1	Laser Hardware	10

4.2.2	Laser Software and Control	10

4.2.3	Beam Transport Optical System	10

4.3	Keck 1 vs. Keck 2	11

4.4	Starfire Optical Range Laser	11

4.5	LCMTI Laser	12

4.6	Future Development	12

4.7	Risk	12

4.8	Cost	12

5	Concept B.  Stable laser platform (SLP)	12

5.1	Motivation for Concept B	12

5.2	Laser	12

5.2.1	Laser Hardware	12

5.2.2	Laser Software & Control	13

5.2.3	Beam Transport Optical System	13

5.3	Keck 1 vs. Keck 2	13

5.4	Starfire Optical Range Laser	13

5.5	LCMTI Laser	14

5.6	Future Development	14

5.7	Risk	14

5.8	Cost	14

6	Concept-C  Fiber Lasers	14

7	What NEXT?	14

# Introduction

The NGAO project will require a combination of nine sodium laser guide stars.  These laser guide stars may come as a result of a single laser or multiple lasers.  This document describes possible architectures to meet this requirement.  The amount of laser power required to produce these guide stars is not fully defined due to dependencies that are not well established.  These dependencies include the direction of pointing into the earth’s magnetic fields, the seasonal changes in the sodium layer, and laser characteristics such as pulse or CW.

This document provides initial concepts of laser architectures that may be able to produce the required laser powers and be integrated into the existing telescope facilities on Keck I or Keck II.  These concepts are based on existing lasers that are being developed at lower powers for the astronomical community.  This document also includes criteria to assist in development of laser requirements as well as guiding in the selection of the laser vendor.  It is assumed the laser will be procured from an outside vendor.

An also important goal for this document is to provide sufficient input to assist personnel in completion of conceptual designs for the Laser Facility (WBS 3.3).

# References

## Input Material for this document

| Ref. # | Document # | Revision or Effective Date | Source | Title |
| --- | --- | --- | --- | --- |
| 1 | GEM00417_B_CTI_Req.v02.91.pdf | 2.91 | Gemini | Requirements for Gemini South and Keck 1 Laser Systems |
| 2 |  |  |  | NGAO Error Budget Spreadsheet |
| 3 |  |  |  | NGAO Laser Guide Star Facility FRD |
|  |  |  |  | IEEE standards of definitions |

Table 1.  References

## Acronyms and Abbreviations

Table 5 defines the acronyms and abbreviations used in this document.

| Acronym/Abbreviation | Definition |
| --- | --- |
| ANSI | American National Standards Institute |
| AO | Adaptive Optics |
| CARA | California Association for Research in Astronomy |
| CW | Continuous Wave |
| HVAC | Heating, Ventilation, Air Conditioning |
| LGS | Laser Guide Star |
| LGS AO | Laser Guide Star Adaptive Optics |
| NFPA | National Fire Protection Association |
| NGS | Natural Guide Star |
| OSHA | Occupational Safety and Health Administration |
| LGS-LSE-XXX | Requirement Keck 1 Laser Service Enclosure |
| RH | Relative Humidity |
| TBD | To Be Determined |
| VAC | Voltage alternating current |
| WMKO | W. M. Keck Observatory |
| EBS |  |
| GS |  |
| GN |  |

Table 2.  Acronyms and Abbreviations

# Methodology

The methodology for determining the laser architecture will depend on the following criteria.  These criteria will need to be prioritized based on the requirements of the laser.

## Financial Budget

The nominal allocation for NGAO is $100,000/watt of 589 nm light with appropriate line width and pulse format.

## Performance

### Photon Return

The laser facility shall produce photon return > 0.12 photons/cm^2/ms/W at the telescope entrance from exciting the Mesospheric sodium layer. [from EBS]

### Laser Spot Quality

The laser M2 shall be <1.3 if it is capable of being mounted on the elevation ring (varying gravity vector configuration).  If the laser is situated on the Nasmyth platform it is suggested that the laser be specified to have an M2 <1.05 . Fiber coupling efficiency is a strong function of the laser M2.

## Laser Reliability

In addition to requirements for reliable operations, it is expected the lifetime of the laser facility to operate for a minimum of 10 years.  Based on K2 laser operations, it is a goal for the laser facility to achieve an uptime of 97% or higher.  One of the considerations for achieving a high uptime is to minimize the requirements on maintenance.  On K2, increasing the number of operational nights with the laser reduces the availability to days to service and improve the laser.  70 nights per semester was determined to be a limit based on resources and time allowed to keep the K2 laser at peak performance.

## Laser Servicing Requirements

Servicing of the laser has not only an impact on the cost of the service but also impact on observatory resources such as telescope.  The architecture can have a large impact on telescope resources.  If the laser head and pump diodes are on the elevation ring, it is necessary for the telescope to be in a configuration that generally has a negative impact on servicing of existing observatory instruments.  Design of the laser should include service and maintenance that minimizes effort as well as resources such as the telescope.

## Laser Operating Cost

### Service and Maintenance

For comparison with existing K2 laser operating cost, the laser will require an FTE that is a combination of an engineer and laser technician to service and maintenance once the laser is in operations.  Operations is defined as performing servicing and maintenance to keep the laser operating trouble free with minor upgrades if any. This number will increase depending on the power and complexity of the system.  For example, higher power will have significantly more power diodes to replace.  The system will need to be designed such that servicing is done with minimal impact to laser operations.  Ideas to consider are replacement of a bank of diodes instead of a single unit at a time; or diodes are design to minimize replacement effort.   Another possible solution is redundancy so part of the system can be down for service while another part is used for operations; this solution is less feasible considering the cost of the laser system.

Material cost will mainly be consumables such as laser pump diodes.  Although diodes are rated for 10,000 hours, the current laser systems are seeing significantly less hours from them.  Assuming a 50% derating and operating the laser at 70 nights/semester, diodes will have to be replaced every 2.4 years at $5K per diode.  The number of diodes will be dependent on the power of the laser system.

### Operations

The goal is for the laser facility to operate without the assistance of a laser operator.  Current K2 Laser operations require the need of a technician for startup of the laser and a laser operator to oversee laser status and controls. The laser operator also does planning to assist in compliance with FAA and Space Command Issues.   The effort estimate is approximately 18 hours / night of observation based on a 12 hour night.  As a minimum, the laser should be able to be operated remotely to minimize the inefficiencies of having someone on the summit for laser operations.

## Risk

### Technical Risk

Specifying a laser even after given a certain type of architecture (QCW, pure CW with resonant ring SFG, pulsed (fiber) laser etc.) is not trivial. There is a fine trade-off between the line width and the power output that needs to be fed into the laser specification and design process.

### Financial Risk

Specifying the laser (or estimating the return wrongly) means extra $$s for we need more laser power for the same return.

## System Scaling Abilities and Flexibility

Unless a laser is well established with a proven record of operations, it is likely the enormous risk and cost of the NGAO laser may have to be possibly scaled to achieve the overall requirements.  This will need to be confirmed as to the ability to scale the laser is acceptable to meet the current observing modes.   For this size project and the number of laser guide stars, design for scaling and redundancy can minimize the risk of having part of the system down at any one time.

## Integration with Telescope Facility

The ability to integrate such a laser on the Keck 1 or Keck 2 telescopes will be challenging.  The current laser systems are larger than what can be integrated onto the existing telescopes.  The next generation lasers based on existing lasers will need to redesign to minimize not just its size, but requirements on the facility such as power and cooling needs.  It is imperative that the laser manufacturer works closely with the telescope facility to minimize the impact of such a laser on an operating telescope.

## Rankings

| Criterion | Ranking | Comment |
| --- | --- | --- |
| Financial |  |  |
| Performance |  |  |
| Reliability |  |  |
| Servicing Requirements |  |  |
| Operating Costs |  |  |
| Risk |  |  |
| Scalability |  |  |
| Integration with Telescope Facility |  |  |

Table 3.  Criteria Rankings

# Concept A.  Moving Laser Platform (MLP)

## Motivation for MLP

MLP is driven by the requirement of maximizing throughput requirement of 75% of the Beam Transfer Optical System.  Unless a major breakthrough in fiber laser or some other form of sodium lasers becomes available, the cost of photon return will be expensive and improvements to throughput will in the end reduce this cost.  MLP is also flexible if the technology improves in other areas such as fiber transport.  Fiber transport’s efficiency has a strong dependency on the beam’s M2; using a free space transport at the elevation ring will reduce the risk on the laser’s beam quality’s requirement. MLP is also adaptable to both the K1 and K2 telescopes.

## Laser

### Laser Hardware

MLP is based on the ability of a laser platform that will be able to operate in a varying gravity vector.  The laser head is defined as the optical components necessary to generate the final 589nm laser beam and amplification; this may include the sum frequency mixing of other laser frequencies.  The laser head does not contain the larger components such as power supplies and possibly pump diodes.

Figure 1: Laser Architecture Locations Front View.

MLP design has the laser head on the elevation ring (Location III).  This concept has been proven with the use of a similar design on K2.  The K2 laser head that generates the final 15 watts of laser light is on the elevation ring.  The area for this enclosure is ample to provide the sufficient space for existing laser designs.  The volume of the K2 laser enclosure is 12’W x 7’H x 4’D.  The laser head must be ruggedized or hardened to operate in a changing gravity vector in this location.  Both K1 and K2 have similar locations for the laser head on the elevation ring.

Figure 2.  Laser Architecture.  Moving Laser Platform Side View.

To minimize the amount and weight of the laser head, it is expected the power supplies and possibly the pump diodes to be located on the Right Nasmyth Platform (Location I) or even off the telescope all together.  Because the power supplies and pump diodes generally have requirements for large amount of power and cooling, it’s highly recommended to have this equipment in a well insulated structure.  These structures can be similar to what currently exist in K1 with the Laser Service Enclosure.

If the power supplies are on or near the Right Nasmyth Platform, cabling can be run to transmit this power to the laser head on the elevation ring.  If the pump diodes are on the Right Nasmyth Platform, IR fibers can be used to transmit the power to the elevation ring.  Fibers that can transmit IR light are more accessible than fibers to transmit 589nm.

### Laser Software and Control

The laser software should be independent of the LGSF architecture.  It is highly recommended the laser software is able to operate with a distributed system similar to the existing EPICS system at WMKO.  The EPICS system may be obsolete by the time the laser is built, but the interface shall be suitable for use with the observatory’s distributed software control architecture.

### Beam Transport Optical System

With the laser head on the telescope, the throughput efficiency of the beam transport optical system is expected to similar to K2 at 85% or higher.  The throughput will heavily depend on the optical system for creating and steering of nine laser guide stars.  It is not expected the throughput to be significantly greater than 90% due to the losses in the launch telescope, optical surfaces for steering of the beam and optical components for dividing the laser power into 9 separate beams.

From the laser head, optical surfaces as well as sensors and actuators will have to steer the beam from the elevation ring to the launch telescope behind the secondary.  The ability of the laser head on the elevation ring will negate the need for compensation from a more stable platform to a moving structure.  Both the K2 laser and the Gemini North Laser have the laser head on the elevation ring.

The generation of 9 laser beams and the steering of each beam can be done on an optical bench that will reside behind the secondary mirror in the secondary module.  This bench as well as the launch telescope has to fit within the secondary module and still provide sufficient ample space to service the BTO equipment as well as other equipment in the vicinity.

## Keck 1 vs. Keck 2

There already exists an enclosure on the K2 telescope for the laser.  This enclosure is not a clean room environment.  The infrastructure for MLP is very similar between the two telescopes. Both telescopes will be able to support this concept.

Keck 1 has the existing laser service enclosure that can be used to house laser electronics such as power supplies and pump diodes.  It may not be large enough to house the entire laser system.

## Starfire Optical Range Laser

SOR currently operates a 50W CW laser at its base at Kirtland, NM.  The SOR laser has a laser head bench on the side a 3.5 meter telescope.  This optical bench does not rotate with gravity and sits flat with the telescope platform.  The beam comes out of the optical bench and is steered onto the elevation.  The output aperture is on the side of the telescope and not centrally projected as required by NGAO.  There are 3 large 19” racks of control computers that also reside on the telescope platform.  Both the racks and the bench rotates only with azimuth movements.

The SOR laser’s architecture can be applied to the MLP based on discussions with Dr. Craig Denman from SOR.  Currently, the power supplies are located on the base of the telescope at a distance of 200 feet from the laser head.  Current cabling requires two pairs of #8 gauge cables for the IR pump Yags.  It is likely, the power supplies can be as far as 400 ft away using larger gauge cables.  This will eliminate the need of a large area available for the power supplies and their cooling requirements on the telescope.  This can improve the scalability of the laser.  The 3 large 19” racks on the Right Nasmyth Platform can be repackaged using embedded computer systems instead of the current PC controllers.  This system will then be small enough to be placed on the elevation ring next to the laser head bench.

The current size of the GEN I 50W optical bench is 6’Wx2’H.  It is reasonable to assume the Gen II system will improve in its output power and repackaged into a smaller volume.  The GEN II system may also use optical pumping at two frequencies to maximize photon return improving the return by a factor of 1.6.  As mentioned before, the laser bench will have to be ruggedized to allow for a changing gravity vector which is expected in the GEN II System.  At the current SOR configuration, the K2 laser enclosure on the elevation ring will fit three 50W SOR optical benches.   Another advantage of the SOR laser is its lack of requirement for a Class 10K clean room, as well as a tightly controlled temperature enclosure.  This will minimize the impact on the size of this enclosure on the telescope as well as infrastructure related to a clean room.

Currently, SOR is planning to build two GEN II Systems, one for SOR and one for AMOS in the next three years.   It will be advantages for NGAO to work closely with AMOS to determine if the NGAO requirements can be included in the GEN II Systems.

Another consideration is the use of the GEN II’s optical pumping at two different frequencies.  The increase return by a factor of 1.6 can significantly reduce the required power output from the laser, thus reducing the overall cost and infrastructure cost related to a higher power laser system.

The SOR laser can be controlled remotely via its network.  The system is controlled using a PC running C++ code written by SOR.  If this does not change, NGAO will need to write a front end interface between the SOR system and the observatory EPICS system.

Based on the current GEN I system and the plans for the GEN II system, it is reasonable to assume the GEN II system will meet the requirements for the MLP architecture.  WMKO should also plan for scaling from delivery of individual 50+ watts laser system with a total of 3 lasers to meet the requirements of NGAO.  This can be reduced to two if the dual frequency laser comes to fruition.

WMKO is currently planning to meet with Craig Denman in the first quarter of FY08 to discuss in more detail the technical aspect of the NGAO LGSF.  Some effort will also need to be spent to determine whether SOR is able to provide a laser for the astronomical community.

## LCMTI Laser

LMCTI is currently developing a 50W Quasi CW laser for the Gemini South Observatory and has a 12W Quasi CW laser operating at the Gemini North Facility.  The current GN system has 2 x 19”W electronics rack with a 5’Wx3.5’Dx4’H laser bench and supports on the elevation ring.  Additional equipment is needed for the LMCTI system due to its requirement for a clean room environment.  In the GN system, the clean room increased the overall dimensions of the enclosure on the elevation ring.

The GS laser has a larger optical bench producing 50W that is not on the elevation ring.  Based on the GS system, it is difficult, but not impossible to place 3 to 4 optical benches of this size and electronics on the elevation ring.  If some scaling can be done to repackage the GS laser into two optical benches similar in size to the GN laser and produce the required power, they can also be placed on the elevation ring.  Discussion with LMCTI will need to take place once the GS laser has met the requirements of the 50W and determine how the system can be repackaged for MLP.

Similar to the SOR laser, the LCMTI laser power should be scaled based on the expected photon return.  The quasi-CW laser provides half the return of the SOR’s CW laser.

## Future Development

If fiber lasers come to fruition, it is reasonable to assume the lasers will be small enough to be located in the elevation ring enclosure.  Fiber lasers are less susceptible to a changing gravity vector.  The BTO system can still be used to transmit the beam to the secondary module.  Some modifications will have to be made since it is likely that multiple fiber lasers will be built to generate the 9 beams needed to be transmitted through the BTOB.

If photonic crystal fibers improve their throughput, power handling, and the laser’s M2 is sufficient to benefit the use of fibers, it is reasonable to assume the MLP laser can use fibers to transport the beam from the elevation ring to the secondary.  However, it would be lower risk to install the laser on a stable platform such as the Right Nasmyth if fibers can be used.  The drawback is the losses in the fiber coupling which increases the power requirement of the lasers.

## Risk

## Cost

# Concept B.  Stable laser platform (SLP)

## Motivation for Concept B

SLP is driven by the requirement of the laser system that it is too large to be located on the elevation ring or that it is not able to withstand a moving gravity vector to meet the performance requirements.  Because of the additional losses related to a fiber transport or additional transport optics, the power requirement for the laser will be increased to meet the NGAO photon returns.

## Laser

### Laser Hardware

SLP will assume the laser head is not on the elevation or varying gravity platform.  The Beam Transport Optical System will provide additional controls to transfer the beam from the laser head to the secondary module.  A possible platform for SLP is the Right Nasmyth Platform; combination of Location I and Location II.  There may be some differences applied to this concept between Keck I and Keck II due to the existing HIRES instrument location on K1.  For now, SLP will assume the laser system will be on or below the Right Nasmyth Platform with appropriate beam transport for each location.

Figure 3.  Laser Architecture.  Stable Laser Platform.  Front View..

There are significant advantages for the laser system since it does not have to harden to operate with a varying gravity vector.  By being off of the telescope, there is less of a requirement to minimize the size of the laser system.  It is reasonable to assume a large enough structure at Location II will be fabricated to house a laser large enough to produce the power needed for NGAO.  Unlike MLP where the burden of meeting the photon return requirement is placed mainly on the laser, SLP divides the burden into both the laser and the Beam Transport Optical System.  In order for the laser to be on a stable platform, the laser power must be increased to compensate for additional losses in the BTOS.  The BTOS will also be required to meet higher throughput in order to reduce the laser power.  The ability to use fibers for beam transport will greatly impact the success of SLP since it minimizes the dependency of the laser system locale.

Figure 4.  Laser Architecture.  Stable Laser Platform.  Side View.

### Laser Software & Control

It is assumed the laser software will be a distributed system similar to EPICS.

### Beam Transport Optical System

With the laser head on a stable platform, there are two likely options for transporting the beam to the secondary.  The first is the use of fibers and the second is the use of free space transport optics.  Fibers will be the preferred design for the BTOB if they are able to meet the requirements for photon return.  Fibers also put additional requirements on the laser spot quality.

The BTOB’s complexity will be determined by the locale of the laser head and the ability to send the laser beam onto the varying gravity platform such as the elevation ring.  A simple solution is to send the beam into the optical axis between the Right Nasmyth Platform and the Tertiary mirror.  This will allow for a simple design with throughput that may be similar to the MLP.   If this is not possible to do, a more complex BTOB will be needed to transport the beam to the secondary with reduced throughput.

## Keck 1 vs. Keck 2

A major consideration for K1 vs. K2 is the ability to access the optical axis for the BTOS.  In K1, HIRES resides on the optical axis making it difficult to access this location.  K2 uses moveable instruments which allow the BTOS to have access to the optical access.  This availability makes K2 more suitable for this concept.  A possible idea to consider also is the possibility to repackage HIRES so it is not directly on the optical axis of K1.

Keck 1 has the existing laser service enclosure that can be used to house laser system if it can be sized to fit into this enclosure.  This room is a clean room and suitable for the LMCTI laser requirements.  The Keck 1 telescope also had its Nasmyth Platform extended for the laser service enclosure.  If Keck 2 requires a similar sized enclosure at the same location, the Nasmyth Platform will also need to be extended.

## Starfire Optical Range Laser

Based on the existing SOR laser, the system is likely to be able to fit in an enclosure similar to the existing K1 LSE with some repackaging in the GEN II system to reduce the overall volume.   Additional power will have to be considered to compensate for the laser on a stable platform.

The power supplies for the GEN II system will be packaged to be off the telescope as in MLP.  This will allow the laser head to be built in the space currently provided by the K1 LSE.  There is room in the existing K1 LSE for some electronics but reducing the overall packaging will provide additional market if more real estate is needed for the additional laser power.

Assuming a similar LSE to K1 can be built on K2, the LSE should be able to fit 3 to 4 existing SOR optical benches.  The benches will be mounted onto the wall without a changing gravity vector which it is currently able to do.

## LCMTI Laser

With the existing LMCTI 50W laser configuration, it will be difficult to fit the 3 to 4 lasers in the existing K1 LSE.  It is likely an entire new enclosure will have to be design and fabricated possibly below the Right Nasmyth Platform.  This should provide sufficient space to accommodate the LMCTI lasers.  Similar to the SOR design, the prefer method will be fiber transport if the throughput requirement can be met.  Otherwise, a free space transport will be necessary to accommodate the moving elevation ring.

## Future Development

Having fiber lasers will minimize the requirement to put the lasers on a stable platform.  Due to their robustness, there is a greater advantage to install them on the elevation ring to increase throughput.  SLP can also applied to fiber lasers since the volume/watt and $/watt are likely to be reduced with fiber lasers.  Additional fiber lasers can be purchased and installed minimizing the need for a high BTOS throughput.

Improved photonic crystal fibers will make SLP more viable as it will reduce the overall cost of the laser.  Other than improvements to lasers, photonic crystal fibers will likely reduced the overall risk and cost to the project.

## Risk

## Cost

# Concept-C  IR Fibers

A third concept may use of hollow core fibers to transmit IR pump light from a stable platform to the SFG.  In this scenario, the IR pump lasers will be housed in a thermal enclosure such as the K1 LSE on the Right Nasmyth Platform.  The SFG is relatively small compared to the pump yags required to generate the IR frequencies (1319nm and 1064nm).  Fibers will transport the pump light to the SFG which can fit in the secondary module.  This would significantly reduce the risk and effort for the BTOS.

The risk of this concept is that fibers for transporting 1064nm and 1319nm light are not readily available, especially at higher CW powers.   This concept will require significant R&D to develop these fibers.

# What NEXT?