Keck Next Generation Adaptive Optics

Opto-mechanical Design Pros and Cons

Donald Gavel

December 3, 2007

The following is a tabular comparison of the two options for KNGAO opto-mechanical design, with pros and cons indicated. The 2-tier design is Brian Bauman’s initial layout as reported at the completion of the AO System Architecture phase.  The 1-tier design is Reni Kupke’s layout, possibly with some suggested changes by Peter Wizinowich.

Refer to the AO System Design web page for Zemax prescriptions and details of these layouts.

|  | 1-tier | 2-tier |
| --- | --- | --- |
| 1st fold mirror conjugate | Con: At ~20km | Pro: At ~10km, a possible location for a 2nd MCAO DM |
| Clearance between K-mirror derotator (M2) and 1st OAP | Pro: ~130mm | Con: Closer. There is an orientation of the K-mirror where the clearance is about 50mm. This assumes the K-mirror must rotatate through a 360 degree angle |
| OSIRIS feed | Pro: almost the same feed can be accomplished from the 1st relay | Con: beam from the AO relay is not at the same vertical height as the OSIRIS beam line |
| Tip/Tilt Performance | Con: larger beam will require larger tip/tilt mechanism; may eat in to tip/tilt bandwidth | Pro: smaller beam means smaller tip/tilt mechanism and less mass, easing the ability to meet the bandwidth requirement. Note: bandwidth requirement is high (~90Hz) driven by the need to control for telescope vibration. |
| Interferometer feed | Pro: reflections/polarizations and field rotation into the interferometer are identical to the current system | Con: reflections/polarizations and field rotation angle are probably different and would require extra reflections for compensation |
| Overall footprint | Con: 2.6 x 2.8 meters. There may or may not be a tight squeeze for instruments on the Nasmyth, depending on instrument volume. | Pro: 2.2 x 1.8 meters. More room for instruments. |
| Beam size in 1st relay and deformable mirror actuator pitch | Con: Must be 140 mm beam. Either 7mm pitch or 5mm pitch DM can be used, but 5mm pitch will require 2x actuators than 7mm pitch. | Pro: Either 100mm or 140mm beam is ok. 140 mm beam will work, but layout will scale in size accordingly |
| Deformable mirror | Pro: Could re-use the existing Keck AO DM (340 actuator 7mm pitch). Con: This DM has considerable hysteresis. If used, there is an unknown effect of actuator hysteresis on the hybrid open-loop/closed-loop control law. | Con: Potentially costly 5 mm pitch DM must be purchased Pro: Can use newer technology low-hysteresis actuators. |
| d-NIRI focus location | Con: up-facing d-NIRI has limited room for instrument under the AO optical bench (focus is located ~300mm below Nasmyth beam line). | Pro: 2-tier design allows d-NIRI focus to be higher, however limited lateral space will demand d-NIRI have a “snout.” |
| Accessibility | Pro: 1-tier design allows for easier assembly and maintenance on a standard optical bench | Con: 2-tier design will demand a space-frame or other additional structure to hold second level of optical elements. More costly design and more difficult to maintain. |