| Architecture | Tel. | NF AO | WF AO | K Mirror | Na Dichroic | 2nd relay Pickoff | ADC | Ent. Win. | Sci. Fold | Total | Total KAON 499 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cascaded Relay | 3 | 3+4 | 4 | 3 | 2 | 1 | 6? | 4 | 1 | 31 | 31 |

Table 14.2, KAON 499

Surface count, sky to narrow field science instruments. 1 and 2 tier are identical.

Table 14.3 KAON 499

| Architecture | Tel. | NF AO | WF AO | K Mirror | Na Dichroic | 2nd relay Pickoff | ADC | Ent. Win. | Sci. Fold | Total | Total KAON 499 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cascaded Relay | 3 | - | 4 | 3 | 2 | 0* | 6? | 4 | - | 22 | 14 |

Surface count, sky to dFIS, 1 and 2-tier are identical

| Architecture | Tel. | NF AO | WF AO | K Mirror | Na Dichroic | 2nd relay Pickoff | ADC | Ent. Win. | LGS Fold | Focusing Element | Total | Total KAON 499 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cascaded Relay, 1 tier | 3 | - | 3* | 3 | 1 | - | - | 4 | - | 2** | 16 | 16 |
| Cascaded Relay, 2 tier | 3 | - | 3* | 3 | 1 | - | - | 4 | 1 | 1 | 16 |  |

* This assumes that the 2nd relay dichroic is removed for dFIS operation.

Table 14.4 KAON 499

Surface count, sky to LGS WFS

* Dichroic for LGS pick-off lies before OAP2.

** Focusing element assumed to be simple lens.

Unknowns:

- Is acquistion camera pick-off to be a removable mirror, or a beamsplitter?  With a dichroic, add 2 surfaces to dNIRI for 2-tier layout (not sure where acq. camera is for 1 tier layout).

- I have not included a NGS pickoff dichroic in the narrow field relay.

- ADCs are assumed to have same number of surfaces as KAON 499 – I don’t have new numbers yet.