Dichorics required for NGAO:

Requirements as suggested by point design of splitter farm (http://www.astro.caltech.edu/twiki_oir/bin/view.cgi/Keck/NGAO/SystemsEngineering):

1st dichroic: (IR Vis. splitter)

Diameter = 34 cms.

R 400-900 nm

R 400-1000 nm

mirror

2nd dichroic:(IR LOWFS and science)

Diameter = 25 cms

R J band

R H band

R K-H band

[R Z band]

open

3rd dichroic: (LGS, SWFS and Vis. science)

Diameter = 20 cms.

R I band

R R band

[R V band]

R < 100 nm

The OAP design of the optical relay is shown below:

Specifications of dichroics for RFQs

1. Name: Infra-red - visible light splitter 1

Diameter of optic = 33 cms

Angle of incidence = 10 degrees

Reflects 400-900 nm and transmits all IR wavelengths

2. Name: Infra-red - visible light splitter 2

Angle of incidence = 10 degrees.

diameter of optic = 33 cms

Reflects 400-1000 nm and transmits all IR wavelengths

-----------------------------------------------------------------

(To first IR instrument)

3. IR LOWFS and Science splitter A1

Angle of incidence = 22.5 degrees

Diameter of optic = 25 cms

Reflects 1.13-1.37 microns (J-band) and transmits over 1.37 microns

4. IR LOWFS and Science splitter A2

Angle of Incidence = 22.5 degrees

Diameter of optic = 25 cms

Reflects 1.5-1.80 microns (H-band) and transmits over 1.80 microns

5. IR LOWFS and Science splitter A3

Angle of incidence = 22.5 degrees

Diameter of optic = 25 cms

Reflects 1.5-2.42 microns (k and H-band) and transmits over 2.42 microns

6.IR LOWFS and Science splitter A4

Angle of incidence = 22.5 degrees.

Diameter of optic = 25 cms or greater

Reflects .6-.99 microns (R to Z-band) and transmits over 1 micron

- - - - - - - - - - - - - -  - - - - - - - - - - - - - - - -

(To second IR intrument)

7. IR LOWFS and Science splitter B1

Angle of incidence = 45 degrees

Diameter of optic = 25 cms

Reflects 1.13-1.37 microns (J-band) and transmits over 1.37 microns

8. IR LOWFS and Science splitter B2

Angle of Incidence = 45 degrees

Diameter of optic = 25 cms

Reflects 1.5-1.80 microns (H-band) and transmits over 1.80 microns

9. IR LOWFS and Science splitter B3

Angle of incidence = 45 degrees

diameter of optic = 25 cms

Reflects 1.5-2.42 microns (k and H-band) and transmits over 2.42 microns

10.IR LOWFS and Science splitter B4

Angle of incidence = 45 degrees.

Diameter of optic = 25 cms or greater

Reflects .81-.99 microns (Z-band) and transmits over 1 micron

-------------------------------------------------------------------

LGS, SWFS and Vis. science splitters

8. LGS, SWFS and Vis. science splitter 1

Angle of incidence = 45 degrees

Diameter of optic = 20 cms

Reflects .7-.9 microns (I-band) and transmits over .9 micron

9.  LGS, SWFS and Vis. science splitter 2

Angle of incidence = 45 degrees

Diameter of optic = 20 cms

Reflects .58-.66 microns (R-band) and transmits over .66 micron

10. LGS, SWFS and Vis. science splitter 3

Angle of incidence = 45 degrees

Diameter of optic = 20 cms

Reflects .49-.59 microns (V-band) and transmits over .9 micron

11. LGS, SWFS and Vis. science splitter 4

Angle of incidence = 45 degrees

Diameter of optic = 20 cms

Reflects < .1 microns and transmits over .1 micron