Observing Operations Concept Document

at the Preliminary Design Phase

D. Le Mignant, E. McGrath et NGAO team

W. M. Keck Observatory

1.	Introduction	4

2.	Scope and Applicability	4

3.	Related Documents	4

4.	Revision History	4

5.	Background	4

5.1.	On science requirements	5

5.2.	On observatory requirements	5

5.3.	Criteria for observing operations success	5

5.4.	Observing efficiency budget	5

6.	Example observations	6

6.1.	Galaxy Assembly and Star Formation History	6

6.1.1.	Pre-observing phase	6

6.1.2.	Observing phases:	8

6.1.3.	Post-Observing	9

6.2.	Key science case 2	10

6.3.	Key science case2	10

6.4.	Etc	10

6.5.	Interferometry science	10

7.	Use Cases	10

7.1.	Search and selection of AO natural guide stars	10

7.2.	AO performance estimation	10

7.3.	Planning the observing sequences with “INST”	10

7.4.	Planning the observing sequences with IF	10

7.5.	Exposure time estimation with “INST”	10

7.6.	Observing efficiency estimation with “INST”	10

7.7.	Loading the observing sequence	10

7.8.	Field acquisition and centering on “INST” pointing origin	10

7.9.	AO acquisition	10

7.9.1.	NGS AO acquisition	12

7.9.2.	LGS AO acquisition: acquiring the NGS (LOWFS and TWFS)	12

7.9.3.	LGS AO acquisition: acquiring the LGS	12

7.10.	Science target acquisition and fine-centering in narrow field instrument	12

7.11.	Science target acquisition and fine-centering in wide field instrument	12

7.12.	Observing sequences execution	12

7.12.1.	Initiating an observing sequence in NGS mode	12

7.12.2.	Initiating an observing sequence in LGS mode	12

7.12.3.	Dither sequences with the d-IFS	12

7.12.4.	Dither sequences with the narrow field science instruments	12

7.12.5.	Dither sequences in NGS AO	12

7.12.6.	Offset sequences with the d-IFS	12

7.12.7.	Offset sequences with the narrow field science instruments	12

7.12.8.	Offset sequences in NGS AO	12

7.12.9.	Filter sequences with the science instrument	12

7.13.	Calibration observing sequences	12

7.13.1.	PSF sequences	13

7.13.2.	Telluric and photometric standard sequences	13

7.13.3.	Background emission sequences	13

7.13.4.	Twilight sequences	13

7.14.	Switching AO modes and science instruments	13

7.14.1.	From wide to narrow field science	13

7.14.2.	Switching AO modes	13

7.14.3.	Switching to backup instrument	13

7.15.	Troubleshooting common observing situations	13

7.15.1.	Variable counts on NGS sensors	13

7.15.2.	Variable counts on LGS sensor	13

7.15.3.	Laser interrupt from LTCS (including LCH)	13

7.15.4.	Observing interrupt for sub-system fault (AO, Laser, instrument, DCS, etc)	13

7.15.5.	Etc	13

7.16.	Post-observing	13

7.16.1.	Data products	13

7.16.2.	Data storage and archive	13

7.16.3.	PSF reconstruction	13

7.17.	AO/Laser calibrations & alignment  sequences	13

7.17.1.	Day time calibrations	13

7.17.2.	Night time calibrations	13

8.	Observing support effort and technical operations	13

8.1.	Science instruments extra-operational support	13

8.1.1.	Access to the system for maintenance	13

8.1.2.	Preventive maintenance needs	13

8.2.	AO/Laser extra-operational support	13

8.2.1.	Access to the system for maintenance	14

8.2.2.	Preventive maintenance needs	14

8.2.3.	Access to the system for development	14

8.3.	Ancillary systems operational support	14

8.3.1.	Access to the system for maintenance	14

8.3.2.	Preventive maintenance needs	14

8.3.3.	Access to the system for development	14

9.	Glossary	14

9.1.	Acronyms and abbreviations	14

9.2.	Jargon and definitions	14

10.	Appendix	14

LIST OF FIGURES

Figure 1: Example of an observing efficiency budget	5

LIST OF TABLES

Table 1: Summary of the observing parameters for the “galaxy assembly and star formation history” science case	6

Table 2: Generic acquisition steps for NGAO	12

# Introduction

As defined in the W. M. Keck Observatory instrument development program (KOR xxx), the OCD is written during the second design phase for a new instrument, preliminary design.  It is the third major design document written for a new instrument and follows the creation of the System Requirements Document (SRD – KAON 455 and 548) and the System Design Manual (SDM – KAON 511), both of which were written during the system design phase.

# Scope and Applicability

The OCD describes how the features and functions of the design described in the SDM will be used to carry out the observations described in the science case.

# Related Documents

This sections lists the documents that are keys to developing the example observations:

- KAON 455: In the following sections, the references to the KAON 455 requirements are noted in italic e.g., (reqt #1.2) refers to the science case requirement #1.2 found in the KAON 455 tabulated requirements.

- KAON 548 presents a summary for the science requirements in support of the architecture evaluations.

- KAON 456: The requirements for  the science operations are described in the NGAO System Requirement Document (SRD – KAON 456) under two sections: Sec. 6.1.4 for Science Operations Requirements from the Science Cases and Sec. 6.2.5 for the Observatory Operational Requirements.

- KAON 511: the System Design Manual presents an overview of the system design (AO system, laser facility, controls hardware and software, science operations and science instruments), performance budgets and requirements flow down. The detailed requirements can be found in most of the referenced design KAONs as well as in the Functional Requirements Document (KAON 573).

In addition, we have made some assumptions in developing the example observations, based on the lessons learned for LGS operations at Keck (KAON 463):

  - the fraction of LGS science time lost to weather is 25%.

  - only 55% of the nights year-round are considered fully photometric

  - the software architecture will allow for parallel operations of subsystems, reducing the overall overhead

# Revision History

| Author | Revision Date | Version | Remarks |
| --- | --- | --- | --- |
| D. Le Mignant | June 4, 2008 | 0 | Initial outline based on TMT and GPI OCD |
| D. Le Mignant | July 24, 2008 | 0.1 | Discussed outline with SA and modified along new OCD guidelines for Keck Included content from draft document on Observing Scenarios |
| D. Le Mignant | August 4, 2008 | 0.2 | Revising content to fit the revised Keck OOCD standards from SA |

# Background

## On science requirements

## On observatory requirements

## Criteria for observing operations success

## Observing efficiency budget

We have developed a simple MS excel-based tool to estimate the observing efficiency for the NGAO science cases. Each science case has its own spreadsheet where open shutter time and overhead have been estimated for the science target and the calibration standard. We have made the following assumptions:

- Open shutter time on the sky-calibration (telluric and flux standards, sky background) is considered science time.

- The main contributors to the observing overhead will be the telescope slews, telescope pointing adjustment and field registration, NGAO and science field acquisitions, fine centering and re-centering, dither/offset + instrument setup + readout.

- For each contribution (open shutter and every overhead), we estimate the minimum, median and maximum values and calculate a weighted average with the following weights (1/6, 4/6, 1/6).

- Estimates for the overhead contributions from telescope slews, pointing adjustments and NGS acquisitions are derived from KAON 567.

- Additional overhead has been included for LTCS interrupts in form of an integer number of individual science integration (hence a higher impact for long integration) and subsequent time for re-centering the science target(s). Note that LTCS interrupts will be greatly reduced by the use of the “first-on-target” rule and the use of planning tools to avoid science observing during planned interrupt request from the Laser Clearinghouse.

- The overhead for dither/offset will be reduced compared to the current system if we adopt a different strategy for dither/offset (KAON 558)

- No time loss has been included for NGAO system failure at this point as we know very little about the NGAO failure rate. This could be considered overly optimistic, yet it is counterbalanced by the allocation of maximum values for every overhead.

Figure 1: Example of an observing efficiency budget sheet

The Observing Efficiency Tool can be downloaded from:

http://www.oir.caltech.edu/twiki_oir/bin/view/Keck/NGAO/NGAOObservingScenarios

# Example observations

This section presents the example observations for each of the Key science cases, the science cases as well as other science cases, such as ASTRA, that drive very specific observing modes.

## Galaxy Assembly and Star Formation History

The NGAO Science Case on Galaxy Assembly and Star Formation History is one of the five Key Science Drivers, which place the most restrictive or technologically challenging constrains on the NGAO system.

The requirements for this science case are described in Section 2.1.1 in KAON 455.  The following sub-sections document its example observations.

### Pre-observing phase

During the pre-observing planning process, the astronomer will want to perform the following actions from her/his home institute:

- Select a list of galaxy fields that contain the galaxies of interest

  - Prioritize the list in terms of scientific interest and observability for the allocated night

- Search for guide stars within each field

  - For each field, the tool will search catalogs available on the internet (ADS, NASA, etc) and  extract the coordinates, brightness (visible and NIR) and color information for a list of star and point-like source.

  - This list can be edited by the astronomer to add point-like source coming from other catalogs, observations or to edit the information on the star parameters.

  - The guide star list is saved in a format easily readable by the user, conform to the Keck telescope star list syntax and common to all NGAO planning tools.

- Simulate the NGAO performance on the galaxy field:

  - A simulation tool will load the natural guide star location and the location of the science targets with respect to the guide stars, as well as all relevant flux/extension/background information.

  - The tool will identify and recommend different possible configurations for the LOWFS guide stars

  - The tool will identify and recommend one or two possible configuration for the laser asterisms

  - The tool will request the user to enter the anticipated airmass for the observations

  - The tool will offer various options for the seeing value (Cn2 profiles?)

  - For each options, the simulation tool will output a simulated performance at the location of the science target. The performance will be in TBD format.

- Simulate the instrument performance

  - The instrument performance will take as inputs:

    - The output PSF profile from the NGAO simulation tool

    - A spatial (and spectral?) model for the object (format TBD)

    - A total flux expressed in TBD photometry system (mJ, ergs.s-1)

    - A sky emission flux model (airglow + continuum)

    - A background emission model for the instrument + telescope

    - A model for the detection noise, depending on detector readout options

    - TBD

  - The instrument performance simulation will  output:

    - SNR/resolution (spatial and spectral) element (expressed in TBD units; 5 sigma, 1 hour, dn/sec, etc)

#### General observing strategy and anticipated observing efficiency:

The observing goal for this science case is to survey more than 200 galaxies over a few years (reqt #1.2) . This is possible by using a deployable NIR spectrograph over a field of ~ 2 arcmin diameter. The number of targets per  field of regard depends on the  number density for the class of objects being studied and varies between 0.1 to 40 arcmin-2  (see KAON 455, table 3). Depending on the object brightness, the NGAO + dIFS will dedicate ~ 1.5 to 4 hours of open shutter time on a set of targets (reqt #1.2).

| Parameters | Value | Comments |
| --- | --- | --- |
| Science instrument | d-IFS |  |
| Instrument setup | IFU FoV: 1 x 3 arcsec J-K wavelength range 70 mas/spaxel 35 mas/pixel TBD |  |
| Total integration time on target | 1.5 - 4 hours | reqt #1.2 |
| Individual integration time on target | 15 - 30 min |  |
| Individual integration time on calibration stars | 20 - 60 sec |  |
| Positioning precision of IFU with respect to the NGS | < 100 milli-arcsec | goal of 20 milli-arcsec ? between LOWFS arms and IFU arms? |
| Background estimation | Dither on IFU | Observer are likely to use a dithering script on the IFU |
| Instrumental calibration | Using arc lamps and integrating sphere | Performed during the day |
| PSF calibration | Using an IFU ? | Telemetry-based PSF reconstruction |
| Telluric standard | Using an IFU ? Dedicated observing sequence |  |
| Dedicated background | Yes, by offsetting off the IFU at least once – TBD |  |

Table 1: Summary of the observing parameters for the “galaxy assembly and star formation history” science case

In estimating the observing efficiency, we have made the following assumptions:

- In parallel to observing a set of extragalactic target, we anticipate the observer will dedicate one of the deployable units on a point-like source to calibrate for the PSF, and possibly for the telluric absorption (depending on the spectral class for the point-like source). This point-like source will also be used to monitor the centering of the field.

- Yet, for every set of targets, we anticipate the observer will calibrate for the telluric absorption after the science spectroscopy observations and within a certain interval in air mass and time. This should add approximately 10 min of observing time to the observations of the set of targets. In addition, we anticipate the observer will want to calibrate the flux by observing flux standards, twice trough the night.

- We assumed individual integration time between 15 and 30 min on the d-IFS for the science targets and between 1 and 3 min for the standards.

- For an observation on a set of targets, we find that it will take 12 min on average for the full setup from initiating the telescope slew till first start of science-quality exposure. The open shutter time will be 155 min on average with 3 min of overheads for dither/readout and 5.3 min for re-centering. The observation of the telluric of flux standard will take approximately 9 min including ~ 55% total overhead (telescope slew, acquisition, dither, readouts).

- We anticipate 27 min loss due to LTCS interrupts, mainly a consequence of the long integration time.

Using these assumptions, we derive that it will be possible to observe 3.14 science fields including the telluric standards and two flux standards for an average night of 10.25 hours numbers.

The total observing efficiency (open shutter on science, including standards) is 83%.

Assuming 3, 5 and 6 deployable units used (minimum, median, maximum, respectively) on the science targets, this leads to 9 to 19 observed targets (16 median) per full observing night.

#### Number of allocated nights

The PI astronomer and her/his team would need 10 to 21 full observing nights to collect data for ~ 200 objects. Assuming a weather-loss fraction of 0.25 (KAON 463), this requires 14 to 28  allocated nights (spread over a few semester) to collect the needed sample of data.

#### Observing model

astronomers are requesting half to full night(s) for this science case in classical observing mode. The astronomer(s) will be performing the observations remotely (either from Waimea or elsewhere) and assessing the data quality on-the-fly. The observations require less than 1 mag of extinction in the V band (for the use of lasers). The PI and the science team are responsible for the backup observing program.

### Observing phases:

The observations with the d-IFS will require the LGS AO mode with the medium to wide configuration for the LGS asterism. The configuration for the LGS asterism will be optimized as a function of the location of the science targets for the d-IFS and the location of the NGS. The planning tools will help the astronomer to select the LGS and NGS configuration that optimizes an observational quantity (SR, EE or target selection)

The AO rotator will be in position angle mode: observers will set the orientation of the North axis, as projected on the d-IFS for each observation of a set of targets.

In order to get an accurate estimate for the PSF, the astronomer will select to dedicate one of the d-IFS unit to observe a faint star in the field, while brighter natural stars from the field will be used for the LOWFS and the truth WFS.

The observer may choose to observe a set of bright photometric or telluric calibrators with the d-IFS. We anticipate the use of the d-IFS in NGS AO mode to maximize the observing efficiency (better SNR, minimal overhead).

#### Science instrument configuration:

The science instrument configuration will be detailed during the Preliminary and Detailed Design phases for NGAO in parallel to the studies for the d-IFS.

KAON 548 provide details for the d-IFS: a spaxel size of 70 milli-arcsec (2 pixels of 35 mas/spaxel), a field-of-view of ~ 1.0 x 3.0 arcseconds per units (#reqt 1.8). Six spectrograph units deployable over a field of regard of at least 120 arcseconds in diameter.

#### Acquisition

The science targets require to be centered on the science array with an accuracy of less than 10% of the field of view (#reqt 1.12) corresponding to  100 – 200 milli-arcsecond.

The targets to acquire during the acquisition are the three NGS for the LOWFS/TWFS and the astronomical sources for the d-IFS. The d-IFS targets include galaxies  plus maybe, simultaneous science field calibrators (point-like sources that can be used to estimate the PSF, the telluric absorptions and/or the photometry).  Subsequently, there are two main scenarios for this science case:

- Except for the 3 NGS, there are not point-like sources in the science field that can be used to check the d-IFS centering. All science targets are faint and their final positioning will rely on the  position from the literature and the system pointing accuracy. This is a “blind acquisition & centering” scenario.

- In addition to the 3 NGS, there are point-like sources in the science field that can be used to reference the centering for the d-IFS (probe arms + MEMs + internal optics).  The centering requirements can be loosen for this case at the expense of the observing efficiency.

The NGAO acquisition  is being designed to meet the requirement for case a).  Yet we have included some time losses due to re-centering in the estimation of the observing efficiency.

#### Science observing sequences

Several scenarios are anticipated for the science observing sequences:

- Dithering and micro-dithering without moving the telescope and opening the AO loops:

For this sequences, the observer will enter a set of n positions in instrument (x, y pixel or spaxels) or sky (RA, DEC offsets) coordinates. The OSC Tools will be coordinating the sequences. Dithering without moving the telescope and opening the loops minimizes the overhead, but it is restricted to small amplitude dither of less than 3 arcsecond if the dithering is performed by the individual MEMs  (See KAON 558).  This allows the observer to recenter quickly the reference objects on the d-IFS and possibly run independent dithering scripts for each IFU.  A typical dithering script will have the object positioned in two or more locations of the IFU, with minimum spatial overlap, allowing to record the target and the sky on one IFU.

Need more information regarding micro-dithering (spaxel or sub-spaxel level).

We also anticipate moving the science probe arms independently for any move beyond the range of the MEMs. The restrictions on the motion of the probe arms (depending on the deployment space and the variations in wavefront error) is yet to be defined. While the field would stay registered, a subset of probe arms could be switching from targets to sky (and targets to targets – TBC) within these constrains. In this scenario, the sky is recorded separately from the science target.

The dither flexibility illustrated by these observing scenario might fit very well the case where sources of very different brightness are observed (e.g., K=19 point-like source calibrator observed in parallel to a set of high-z  galaxies).

- Offsetting:

Offsetting requires moving the telescope, pausing the AO loops (at least for the LOWFS/TWFS), repositioning the LOWFS and LGS probe arms and resuming the AO loops. Offset will apply to all targets at once.

We do anticipate to use the offset mode with d-IFS if a different set of targets are observed for the same science field. This saves the time to re-acquire the NGS and perform a new LGS acquisition.

- Filter and instrument sequences

The d-IFS instrument will allow the observer to  run different observing sequences such as filter, pixel, read modes, etc. The observer should be able to set the priority for the instrument sequences with respect to the dithering sequences.

#### Science Calibrations:

- Flat-field and spectral calibrations:

Flat field and spectral calibrations are performed on the internal light source (integrating sphere) by the Observatory support staff. The output of the calibrations is integrated in the rectification matrix, as part of the  data reduction pipeline.  The requirements for these calibrations will be detailed during the study and design of the d-IFS.

- PSF calibrations:

Calibrating the PSF is critical to the quantitative analysis of the galaxies to derive e.g., the ensquared energy per spaxel. This analysis is required per spectral channel; it is then required to estimate the PSF per spectral channel.

The quantitative requirements for the PSF calibrations have yet to be understood. Different methods are being assessed: 1) one IFU is used as a PSF monitoring, and/or 2) a model PSF is reconstructed from the telemetry (WFC and Cn^2 data). The first scenario is costly to science as it requires dedicating one probe arm to the observations of a point-like source but provides a simultaneous PSF calibration for one location in the field. Extrapolating the PSF measured on one location to any location in the corrected field has been demonstrated (Britton 2006). The second scenario still requires some significant development and has yet to be demonstrated with LGS.

- Photometry

- Astrometry

### Post-Observing

#### Data products

The generic data product includes:

  - The raw science and calibration data

  - The reduced science and calibration data

  - The PSF reconstruction data from the WFC and Cn2 profiler telemetry, if applicable

  - The image quality monitoring data (SR, EE, flux on LOWFS and laser return, fault events log, etc)

The d-IFS will include its own data reduction pipeline. The requirements for the DRP will be developed during the design phase for the d-IFS.

#### Science user data

#### Observatory data management

The observatory will save and store at least one copy of the entire set of the data product, defined with the observer. The requirements for the automated archive of the NGAO data will be studied and developed during the Preliminary Phase for NGAO.

The archived data will be proprietary and the release mechanism for technical or scientific purpose after a certain delay will be defined in the Preliminary Design phase of NGAO.

## Key science case 2

## Key science case2

## Etc

## Interferometry science

# Use Cases

## Search and selection of AO natural guide stars

## AO performance estimation

## Planning the observing sequences with “INST”

## Planning the observing sequences with IF

## Exposure time estimation with “INST”

## Observing efficiency estimation with “INST”

## Loading the observing sequence

## Field acquisition and centering on “INST” pointing origin

## AO acquisition

Three types of targets needs to be acquired on the NGAO photo-sensors:

- the NGS for the LOWFS, the TWFS and/or the WFS

- the astronomical sources for the science instruments:

  - for the d-IFS: target galaxies plus simultaneous science field calibrators

  - for the narrow field science instruments: the science target

- the Laser Guide Stars for the HO WFS.

The LGS acquisition section will be detailed in the next iterations of the example observations, depending of the options selected for the LGS asterisms and the point-and-shoot mode.

The table below presents a generic description for the acquisition sequence (see KAON 567).

| Step | Observing Step | Parallel Steps | Remarks |
| --- | --- | --- | --- |
| 0 | Select next target: - assess science priority - check target elevation range - check observing conditions - check LTCS conditions | Complete integration on current science target or calibrator. When target selected from Planning Tools, then information is loaded in OSC Tools. | It is not clear yet whether the astronomer will have to run these checks manually or whether it will be automated. |
| 1 | Upon completion of readout of science array, LGS is shuttered, AO loops open and key-system feedback parameters are saved then the Science Operations (ScOp) Tools triggers the telescope slew. | OSC Tools parses information, and get ready for execution: - NGS parameters for acquisition - AO configuration - Instrument configuration |  |
| 2 | Telescope slews | The OSC Tools sends command to the subsystem sequencer and setup sequences are executed as appropriate by the AO, Laser, Science Instrument subsequencers. |  |
| 3 | Telescope Pointing Adjustment on one of the NGS (brightness allowing). This steps is automatically performed by the NGS acquisition subsystem with the visual check of the Observing Assistant (OA). Upon success, pointing corrections are applied and next telescope slew is commanded from the OSC. |  | This step is required when the NGS are fainter than R=18 mag. (TBC) or when the telescope slews by more than x deg in elevation/azimuth range. |
| 4 | Telescope coarse registration on the science field. NGS acquisition subsystem runs an automated routine to record and process image, ID the NGS in the field wrt catalog data then compute required offset. Visual check of process by OA. Upon success, position offsets are applied to telescope. | LGS propagation and acquisition steps initiated. If small telescope centering offset (<5 “ – TBC), uplink TT correction closed with very low gain. Pickoff positioning and LOWFS setup complete including background. | Need to implement the acquisition for the vibration/wind shake reference. Need addit’al study for telescope guiding. Not clear if the HOWFS will require a background. |
| 5 | Telescope fine registration on the science field: If photons are not detected at the expected SNR on the LOWFS (or NGS not on Pointing Origin), then NGS acquisition subsystem runs a second iteration. Visual check of process by OA. Iterate if necessary (to be detailed). Upon completion adjust telescope pointing model. | Pick-off mirrors for science and TWFS in position | Need to check the conditions for the dichroic during this step. Need to be able to adjust pointing model for telescope even though PO != REF. |
| 6 | AO subsystem control: 1) low gain on woofer and MEMs, 2) increase gain on UT, 3) start telescope guiding, 4) adjust woofer and MEMs gain, 5) initiate TWFS + tomography optimization | Science instrument is setup: optics and read modes are set and confirmed. May record first exposure to check centering with point-source and expected SNR/coadd. Monitor image quality and assess optimization progress. | UTT control (all/individual?) Assuming redundant information from USNO-B, GSC-II and SDSS, uncertainty in the field centering should be ~0.2” (TBC). |
| 7 | Science integration starts |  |  |

Table 2: Generic acquisition steps for NGAO

The main contributions to the centering error budget during NGAO acquisition in LGS mode are:

- The accuracy for the knowledge of the separation distance and PA between the stars and the galaxies from the literature. This information is provided by the astronomer, and can be less than 10  milli-arcsecond  if the field has been observed (recently) with HST cameras. The USNO-B on-line catalog  provides an astrometric accuracy of 200 milli-arcsecond, and the astrometric solution for the USNO-B, GSC-II and SDSS catalogs are improved as the proper motions are being calibrated using GSC-I (KAON 467).

- The accuracy in positioning the pickoff arm for each science target w/ respect  to the TT closed-loop reference position for the LOWFS, which is the total of:

  - The internal positioning accuracy and position stability for each individual pick-off  arm (science and LOWFS) – the requirement is less than 5 milli-arcsecond (KAON 548)

  - Registration accuracy and stability between LOWFS and science arms including TT stage positioning accuracy.

- The differential atmospheric refraction between the LOWFS and the science

- The total contribution from the optical distortions  due to thermal gradient, alignment error, woofer and MEMs positioning between the science array and the LOWFS.

KAON 559 provides a conceptual study report for the “Interim LOWFS and LGS Object Selection Mechanism” and shows that the requirement for the positioning accuracy can be met (contribution #2 above). Yet, contribution # 3 and #4 will need further analysis during the PD or DD phases.

### NGS AO acquisition

### LGS AO acquisition: acquiring the NGS (LOWFS and TWFS)

### LGS AO acquisition: acquiring the LGS

## Science target acquisition and fine-centering in narrow field instrument

## Science target acquisition and fine-centering in wide field instrument

## Observing sequences execution

The observing sequences will be coordinated at the Observing Sequence Tool level to minimize the  overhead during dither and offset scripts: as soon as the instrument reports that the readout is complete, the OS tool will command the dither or offset either in instrument pixel or sky coordinates. KAON 558 proposes possible scenarios for dither and offset. We anticipate that any repositioning of the observing reference frame of less than 5” (TBC) will be performed by repositioning internal optics and will not require to move the telescope (hence not require to open any loops). These we propose to define as “dither” and should take less than 5 seconds.

Conversely, any move that require to move the telescope and open the AO loops are defined as offset, and may require  longer overhead (~ 15 -30 sec - TBC).

### Initiating an observing sequence in NGS mode

### Initiating an observing sequence in LGS mode

### Dither sequences with the d-IFS

### Dither sequences with the narrow field science instruments

### Dither sequences in NGS AO

### Offset sequences with the d-IFS

### Offset sequences with the narrow field science instruments

### Offset sequences in NGS AO

### Filter sequences with the science instrument

## Calibration observing sequences

### PSF sequences

### Telluric and photometric standard sequences

### Background emission sequences

### Twilight sequences

## Switching AO modes and science instruments

### From wide to narrow field science

### Switching AO modes

### Switching to backup instrument

## Troubleshooting common observing situations

### Variable counts on NGS sensors

### Variable counts on LGS sensor

### Laser interrupt from LTCS (including LCH)

### Observing interrupt for sub-system fault (AO, Laser, instrument, DCS, etc)

### Etc

## Post-observing

### Data products

#### Science data product

#### AO & ancillary data product

### Data storage and archive

### PSF reconstruction

## AO/Laser calibrations & alignment  sequences

### Day time calibrations

### Night time calibrations

# Observing support effort and technical operations

## Science instruments extra-operational support

### Access to the system for maintenance

### Preventive maintenance needs

## AO/Laser extra-operational support

### Access to the system for maintenance

### Preventive maintenance needs

### Access to the system for development

## Ancillary systems operational support

### Access to the system for maintenance

### Preventive maintenance needs

### Access to the system for development

# Glossary

## Acronyms and abbreviations

The following acronyms and abbreviations  were used in this document:

- KAON: Keck Adaptive Optics Notes, available from the NGAO Twiki site or from KeckShare

- NGS: Natural Guide Star

- LGS: Laser Guide Star

- TAC: Time Allocation Committee

- LOWFS: Low Order Wavefront Sensor, sensors looking at a NGS used for tip-tilt / focus and astigmatism.

- TWFS: True Wavefront Sensor, sensor looking at a NGS used for providing a true reference for the average residual aberrations.

- d-IFS: deployable Integral Field Spectrograph

- IFU: Integral Field Unit

## Jargon and definitions

The following terms were used with the corresponding definitions:

- Dither:

- Offset

# Appendix