Keck Adaptive Optics Note 455

Keck Next Generation Adaptive Optics

Science Case Requirements Document

Release 2

Version 11

June 29, 2007

Keck Adaptive Optics Note 455

Keck Next Generation Adaptive Optics

Science Case Requirements Document

Release 2

1	Revision History	5

2	Introduction	5

2.1	Background	5

2.2	JWST Capabilities	6

2.3	ALMA Capabilities	6

2.4	Next Generation AO Projects at Other Observatories	7

3	Science Cases	9

3.1	Multiplicity, Size, and Shape of Minor Planets	9

3.1.1	Scientific background and context	9

3.1.2	Scientific goals	9

3.1.3	Proposed observations and targets	9

3.1.3.1	Size and shape	9

3.1.3.2	Orbits of multiple asteroidal systems	10

3.1.4	AO requirements	13

3.1.4.1	Wavefront error	13

3.1.4.2	Encircled energy	13

3.1.4.3	Contiguous field requirement	14

3.1.4.4	Photometric precision	14

3.1.4.5	Astrometric precision	14

3.1.4.6	Contrast	14

3.1.4.7	Polarimetric precision	14

3.1.4.8	Backgrounds	14

3.1.4.9	Overall transmission	15

3.1.5	Other key design features	15

3.1.5.1	Required observing modes	15

3.1.5.2	Observing efficiency	15

3.1.6	Instrument requirements	15

3.1.6.1	Required instruments	15

3.1.6.2	Field of view	16

3.1.6.3	Field of regard	16

3.1.6.4	IFU multiplicity	16

3.1.6.5	Wavelength coverage	16

3.1.6.6	Spectral resolution	16

3.1.6.7	Requirements Summary	16

3.1.7	References	20

3.2	Moons of the Giant Planets	21

3.2.1	Summary of Requirements	21

3.3	Imaging and characterization of extrasolar planets around nearby stars	22

3.3.1	Scientific background and context	22

3.3.2	Scientific goals	22

3.3.2.1	Planets around low-mass stars and brown dwarfs	24

3.3.2.2	Very young planets in the nearest star-forming regions	25

3.3.3	Proposed observations	25

3.3.4	AO and instrument requirements	25

3.3.5	Performance Requirements	26

3.3.5.1	Wavefront error	26

3.3.5.2	Encircled energy	26

3.3.5.3	Need for large contiguous fields	26

3.3.5.4	Photometric precision	26

3.3.5.5	Astrometric precision	26

3.3.5.6	Contrast	26

3.3.5.7	Polarimetric precision	26

3.3.5.8	Backgrounds	27

3.3.5.9	Overall transmission	27

3.3.6	Other key design features	27

3.3.6.1	Required observing modes	27

3.3.6.2	Observing efficiency	27

3.3.7	Instrument requirements	27

3.3.7.1	Instruments needed	27

3.3.7.2	Field of view	27

3.3.7.3	Field of regard	27

3.3.7.4	IFU multiplicity	27

3.3.7.5	Wavelength coverage	27

3.3.7.6	Spectral resolution	28

3.3.8	Summary of Requirements	28

3.3.9	References	31

3.4	Precision Astrometry: Measurements of General Relativity Effects in the Galactic Center	32

3.4.1	Scientific background and goals	32

3.4.2	General Relativistic Effects	32

3.4.3	R0 and the dark matter halo	34

3.4.4	Proposed observations and targets	34

3.4.5	Observing plan for Astrometric Imaging:	35

3.4.6	Observing plan for Radial Velocity Measurements (IFU spectroscopy):	35

3.4.7	Current issues and limitations that could be further explored with existing data sets	36

3.4.8	AO requirements	37

3.4.9	General comments on astrometric accuracy in an AO system with multiple deformable mirrors	37

3.4.10	Radial Velocity	38

3.4.11	Instrument requirements	38

3.4.12	Summary of Requirements	39

3.4.13	References	40

3.5	Nearby Active Galactic Nuclei	42

3.6	QSO Host Galaxies	43

3.7	Galaxy Assembly and Star Formation History	45

3.7.1	Introduction	45

3.7.2	Scientific background and context: galaxies at 2 ≤ z ≤  3	46

3.7.3	Scientific goals	46

3.7.4	Proposed observations and targets	48

3.7.5	AO and instrument requirements	49

3.7.5.1	AO requirements	49

3.7.5.2	Instrument Requirements:	51

3.7.6	Summary of Requirements	52

3.7.7	References	53

3.8	Gravitational Lensing	54

3.9	Roll-up of AO and Instrument Requirements	58

4	Appendix A.  Table of potential infrared tip-tilt stars close to the Galactic Center	60

# Revision History

| Version | Date | Author | Reason for revision / remarks |
| --- | --- | --- | --- |
| Release 1 v 10b | June 6, 2007 | Max | Official “Release 1”, KAON 455 |
| Release 2 v 1 | June 29, 2007 | Max | Added Science Requirements tables that have been developed to date |

# Introduction

## Background

The Science Team of the Keck NGAO project is charged with 1) identifying the science requirements for the Next-Generation Adaptive Optics (NGAO) system, and 2) when design trade-offs must be made, ensuring that the NGAO system will be built with capabilities that enable key science cases to the greatest extent possible.

This document, which will be referred to as the Science Case Requirements Document (SCRD), is a “living document”, and will be updated as the science case is developed with increasing fidelity.  Initially, the SCRD will rely on and heavily reference the science cases developed for the Proposal to the Keck Science Steering Committee prepared in June 2006.  Key issues are (a) the importance of the science enabled by the AO system and its accompanying instruments; (b) the advances offered by NGAO relative to AO systems being developed on other telescopes (discovery space); and (c) complementarity to the James Webb Space Telescope (JWST) and the Atacama Large Millimeter Array (ALMA), which will be commissioned on the same timescale as Keck NGAO.

Since June 2006, the Project Scientist has met with subgroups of the Science Team in order to re-examine the science cases, to develop more solid requirements, and to look in more detail at the associated Instrument and Observatory requirements.  This document, Release 1 of the SCRD, describes AO and instrument requirements for a subset of NGAO science that is anticipated to have impact on the following AO system error budgets and instrument requirements:

- AO and instrument background requirements

- wavefront error

- astrometric error budget

- field of view and field of regard

- spectral resolution and multiplicity for the deployable integral field unit instrument

- AO capabilities at visible wavelengths

## JWST Capabilities

The James Webb Space Telescope (JWST) is a cryogenic 6.5-m space telescope that is currently scheduled to be launched in 2013.  It will have considerably higher faint-source sensitivity than Keck NGAO due to its low backgrounds.  Its NIRCAM instrument will image in 14 filters in the 0.6-2.3 µm wavelength range that overlaps with Keck NGAO.  NIRCAM will have a 2.2 x 2.2 arc-minute field of view, a pixel scale of 0.035 arc sec for 0.6-2.3 µm wavelengths, and coronagraphic capability.  NIRCAM has diffraction-limited imaging for wavelengths between 2.4 and 5 µm, but not below 2 µm due both to the primary mirror quality specification and to the undersampled pixel scale (0.035 arcsec) within NIRCAM.  Thus there is an interesting part of parameter space in which Keck NGAO can complement JWST’s imaging capabilities: diffraction limited imaging at wavelengths below 2 µm, over a field of order 2 arc-minutes on a side.

NIRSpec is a near infrared multi-object spectrograph for JWST in the 0.6 - 5 μm band. The primary goal for NIRSpec is enabling large surveys of faint galaxies (1<z<5) and determining their metallicity, star formation rate, and reddening. The NIRSpec design provides three observing modes: a low resolution R=100 prism mode, an R=1000 multi-object mode, and an R=2700 mode. In the R=100 and R=1000 modes NIRSpec provides the ability to obtain simultaneous spectra of more than 100 objects in the 3.4 x 3.5 arcmin field of view. Spatial pixel size will be 0.1 arc sec.  There will be an integral field spectrograph with field of view 3” x 3”, using 0.1 arc sec pixels, and a spectral resolution of R=2700.

Areas of parameter space in which Keck NGAO could complement JWST’s spectroscopic capabilities include the following: 1) Spectroscopy (either slit or IFU) with spatial resolution better than 0.1 arc sec; 2) multi-IFU spectroscopy; 3) spectroscopy (slit or IFU) near the Keck diffraction limit at wavelengths 0.6 - 2 μm.  It would be very difficult for Keck NGAO to compete with JWST at wavelengths longer than K band, because JWST will have far lower backgrounds.  Even at the long-wavelength end of K band where the thermal background is important, NGAO will have difficulty competing in sensitivity with JWST’s NIRSpec.

## ALMA Capabilities

ALMA is a powerful new facility for mm and sub-mm astrophysics that is currently scheduled to begin science operations in 2012.  It will consist of 54 12-m and 12 7-m antennas located at an altitude of 5000m (16,500 feet) in the Atacama desert of Chile.  ALMA will observe, with very high sensitivity and resolution, the cold regions of the Universe which are optically dark, yet shine brightly in the millimeter portion of the electromagnetic spectrum. With baselines ranging from 150 m to 18 km, it will have spatial resolution down to 0.01 arc-seconds (0.004 arc-seconds at the highest frequencies), with typical resolution of 0.1 arc-second or better.  It is expected to operate within atmospheric windows from 0.35 to 9 mm.

The design of the ALMA is being driven by three key science goals:

1. The ability to detect spectral line emission from CO or CII in a normal galaxy like the Milky Way at a redshift of z = 3, in less than 24 hours of observation.

2. The ability to image the gas kinematics in protostars and in protoplanetary disks around young Sun-like stars at a distance of 150 pc (roughly the distance of the star-forming clouds in Ophiuchus or Corona Australis), enabling the study of their physical, chemical and magnetic field structures and to detect the tidal gaps created by planets undergoing formation in the disks.

3. The ability to provide precise images at an angular resolution of 0.1 arcsec. Here the term "precise image" means being able to represent, within the noise level, the sky brightness at all points where the brightness is greater than 0.1% of the peak image brightness.

ALMA will excel at the study of chemical evolution in star-forming regions at z ~ 3, dust-gas interactions, molecules surrounding stars, and molecular clouds.  With its high sensitivity it will detect redshifted continuum dust emission out to z=10.  It will reveal kinematics of obscured AGNs and quasi-stellar objects on spatial scales of 10 - 100 pc.  It will use line emission from CO to measure the redshift of star-forming galaxies throughout the universe.  It will image the formation of molecules and dust grains in the circumstellar shells and envelopes of evolved stars, novae, and supernovae.

ALMA’s spatial resolution in the mm and sub-mm bands will be competitive with Keck’s diffraction limit at wavelengths 0.6 – 2.4 μm.  ALMA will be observing regions that are colder and more dense than can be seen in the visible or near-infrared with Keck.  However Keck NGAO observations of H2 and atomic hydrogen emission lines at H and K bands will complement ALMA by characterizing the warmer outer regions of molecular clouds and circumstellar disks.  ALMA images and spectra of debris disks will complement the higher spatial resolution NGAO images at shorter wavelengths.

## Next Generation AO Projects at Other Observatories

The NGAO team has done a survey of current and future AO systems worldwide.  Within the scope of our science goals we would prefer to position Keck NGAO to take a leadership role in AO, rather than building the second or third version of a specific type of next-generation AO system.

The VLT and Gemini Observatories are planning Ground Layer AO and Extreme AO systems.  Gemini South and (eventually) the LBT plan to have MCAO systems.  By contrast precision AO, as in the planned Keck NGAO system, has not been emphasized in the plans of the other 8-10 meter telescopes.

Below in Table 1 we give an overview of plans of other observatories for what we call “next-generation AO systems” on 8 – 10 meter telescopes.  By next-generation AO we mean those systems that go beyond single-conjugate AO with one laser guide star, or that aim for a special-purpose application such as high-contrast imaging or interferometry.  We obtained our information from published papers, from web sites, and from the May 2006 SPIE meeting in Orlando FL.

| Next-Generation AO Systems Under Development for 8 - 10 meter Telescopes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Telescope | GS | Next-Generation AO Systems for 8 to 10 m telescopes | Capabilities | Dates |
| High-contrast | Subaru | N/LGS | Coronagraphic Imager (CIAO) | Good Strehl, 188-act curvature, 4W laser | 2007 |
| High-contrast | VLT | NGS | Sphere (VLT-Planet Finder) | High Strehl; not as ambitous as GPI | 2010 |
| High-contrast | Gemini-S | NGS | Gemini Planet Imager (GPI) | Very high Strehl | 2010 |
| Wide-field | Gemini-S | 5 LGS | MCAO | 2’ FOV | 2007 |
| Wide-field | Gemini | 4 LGS | GLAO | Feasibility Study Completed | ? |
| Wide-field | VLT | 4 LGS | HAWK-I (near IR imager) + GRAAL GLAO | 7.5' FOV, AO seeing reducer, 2 x EE in 0.1'' | 2012 |
| Wide-field | VLT | 4 LGS | MUSE (24 vis. IFUs) + GALACSI GLAO | 1' FOV; 2 x EE in 0.2" at 750nm | 2012 |
| Narrow-field | VLT | 4 LGS | MUSE (24 vis. IFUs) + GALACSI GLAO | 10” FOV, 10% Strehl @ 650 nm | 2012 |
| Inter-ferometer | LBT | NGS | AO for LINC-NIRVANA (IR interferometer) | Phase 1: Single conj., 2 tel’s Phase 2: MCAO 1 telescope Phase 3: MCAO both telescopes | Phase 1 in 2008 |

Table 1

Next-generation AO systems under development for 8 – 10 meter telescopes.

# Science Cases

## Multiplicity, Size, and Shape of Minor Planets

Author: Franck Marchis

Editor: Claire Max

### Scientific background and context

While space missions largely drove early progress in planetary astronomy, we are now in an era where ground-based telescopes have greatly expanded the study of planets, planetary satellites, and the asteroid and Kuiper belts.  Ground-based telescopes can efficiently perform the regular observations needed for monitoring planetary atmospheres and geology, and can quickly respond to transient events.

The study of the remnants from the formation of our solar system provides insight into the proto-planetary conditions that existed at the time of solar system formation.  Such information has been locked into the orbits and properties of asteroids and Kuiper Belt objects.  The study of binary (and multiple) minor planets is one key path to revealing these insights, specifically by studying their kinematics and geological properties.  There are no space missions currently planned to study these binaries.  This important inquiry is only accessible to ground-based telescopes with AO.

### Scientific goals

High angular resolution studies are needed of large samples of binary asteroids to understand how their enormous present-day diversity arose from their formation conditions and subsequent physical evolution, through processes such as disruption and re-accretion, fragmentation, ejecta capture, and fission.  Specifically one can study:

- Formation and interiors of minor planets by accurate estimates of the size and shape of minor planets and their companions

- Mass, density, and distribution of interior material by precise determination of the orbital parameters of moonlet satellites

- Chemical composition and age, by combining high angular resolution with spectroscopic analysis

### Proposed observations and targets

#### Size and shape

Spatially resolved imaging of large asteroids is critical in order to derive reliable statistical constraints on large collisions throughout the Main Belt.  Observations of the 15 or 20 largest asteroids would provide the statistics necessary to put much stronger constraints on the frequency of major collisions.  We estimate that 20 Main Belt asteroids will be resolved with sufficient resolution with NGAO in R-band (33 in V-band) to obtain mapping comparable to that already done for 4 Vesta.

Table 2 summarizes the number of asteroids resolvable from visible to near-IR, categorized by domain and population.  Thanks to NGAO’s high angular resolution in V and R bands, ~800 main-belt asteroids could be resolved and have their shape estimated with a precision of better than 7%.  With the current AO system ~100 asteroids, located only in the main-belt, can be resolved.  The determination of the size and shape of even a few Trojan asteroids will be useful to estimate their albedo.  For Near Earth Objects (NEAs), the large number of resolvable objects is a result of very close approaches to Earth.

Table 2

Number of asteroids resolvable with Keck NGAO in various wavelength ranges and populations (assuming on-axis observations).

| Populations by brightness (numbered and unnumbered asteroids) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Orbital type | Total number | V < 15 | 15 < V < 16 | 16 < V < 17 | 17 < V < 18 |
| Near Earth | 3923 | 1666 | 583 | 622 | 521 |
| Main Belt | 318474 | 4149 | 9859 | 30246 | 88049 |
| Trojan | 1997 | 13 | 44 | 108 | 273 |
| Centaur | 80 | 1 | 1 | 2 | 2 |
| TNO | 1010 | 1 | 2 | 0 | 2 |
| Other | 3244 | 140 | 289 | 638 | 870 |

This research program will have even higher impact if it is combined with the study of binary asteroids.  Recent studies suggest that the primary asteroid of most binary asteroid systems has a rubble-pile structure, indicating that they have weak shear strength (Marchis et al., 2006).  Consequently their shape is directly related to the angular momentum at their formation (Tanga et al., 2006).  One can obtain their mass through determination of moonlet orbits combined with a good shape determination, by direct imaging in the visible (which provides the best angular resolution).  Assuming an R band Strehl > 20% so that there is a clear diffraction-limited peak in the PSF, we estimate that between 1000-4000 new binary asteroids could be discovered with NGAO.  An accurate shape estimate for ~300 of them (an order of magnitude more than the number of asteroids with currently known shapes) can be attained with NGAO in R band.  Six observations taken at various longitudes are enough to accurately reconstruct the 3D-shape of the asteroid.  Twelve nights of observations should be considered for the completion of such a program.  Dedicated nights are not necessary since this program can be combined with the study of satellite orbits of asteroids using the same instrumentation.

#### Orbits of multiple asteroidal systems

Study of main-belt multiple systems: One of the main limitations of current AO observations for a large search for binary asteroids and for characterization of their orbits is the limited quantity number of asteroids observable considering the magnitude limit on the NGS wavefront sensor. The Keck NGS AO system can use guide stars down to 13.5 magnitude, so ~1000 main-belt asteroids (to perihelion >2.15 AU and aphelion <3.3 AU) can be observed.

With NGAO providing an excellent correction up to tip-tilt star magnitudes V = 17 or J = 19, 10% of the known main-belt population can be searched, corresponding to the potential discovery of 1000 multiple systems assuming the current multiplicity rate of 6% - 15%. This is a lower limit on the detection rate of new moonlets, because the NGAO system will provide a more stable correction than current Keck LGS AO and the halo due to uncorrected phase errors will be significantly reduced.

Closer and fainter satellites should be detectable, as will be explained below.  At the time of this writing, the orbits of ~15 visual binary systems are known and display considerable diversity. To better understand these differences, we propose to focus our study on 100 new binary systems in the main-belt discovered elsewhere, by light-curves or snap shot programs on HST and/or previous AO systems.  The increase by an order of magnitude of known orbits will help us to understand how they formed as members of a collisional family, their distance to the Sun, their size and shape, and other parameters.

To reach a peak SNR~1000-3000 on an AO image, the typical total integration times assuming 140 nm of wavefront error are 5 min and 15 min for 13th or 17th V-magnitude targets respectively.  Considering a current overhead of 25 min (Marchis et al. 2004b) to move the telescope onto the target and close the AO loop, the total telescope time per observation is ~30 min.   This overhead time should be significantly improved by careful design of the NGAO system.  The orbit of an asteroid can be approximated (P, a, e, i) after 8 consecutive observations taken over a period of 1-2 months to limit the parallax effect, corresponding to the need for 0.3 nights per object. Thirty nights of observation would be requested for this program over 3 years.

To illustrate the gain in quality expected with NGAO, we generated a set of simulated images of the triple asteroid system 87 Sylvia. The binary nature of this asteroid was discovered in 2001 using the Keck NGS AO system. Marchis et al. (2005) announced recently the discovery of a smaller and closer moonlet. The system is composed of a D=280 km ellipsoidal primary around which two moons describe a circular and coplanar orbit: “Romulus”, the outermost moonlet (D=18 km) at 1356 km (~0.7”) and “Remus” (D = 7 km) at 706 km (~0.35”). In our simulation we added artificially two additional moonlets around the primary: “S1/New” (D=3.5 km) located between Romulus and Remus (at 1050 km) and “S2/New” (D=12 km) closer to the primary (at 480 km). This system is particularly difficult to observe since the orbits of the moons are nearly edge-on. We blurred the image using the simulated NGAO and Keck NGS AO PSFs (with an rms error of 140 nm) and added Poisson and detector noise to reach a S/N of 2000 (corresponding to 1-3 min integration time for a V=12 target). We then estimated whether the moonlets could be detected and their intensity measured by aperture photometry. Figure 1 displays a comparison for one observation between the current Keck NGS AO, NGAO in two wavelengths, and HST/ACS. The angular resolution and thus the sensitivity of the NGAO R-band is a clear improvement and permits detection of the faintest moon of the system.

Figure 1

Simulation of pseudo-Sylvia observed with various AO systems.

We assumed that NGAO has a wavefront error of 140 nm in this simulation.

Table 3 summarizes the 2-s detection rate for the pseudo-Sylvia system moonlets. The photometry was done using the same technique as for real observations (aperture photometry + fitting/correction of flux lost). The detection rates for NGAO- R band are 100% for all moons. One can also notice a very good photometric recovery with this AO system. The chances to discover multiple systems and to analyze them are significantly improved with NGAO. It should be also emphasized that because the astrometric accuracy is also better, determination of the orbital elements of the moons will be also more accurate (e.g., a significant eccentricity or small tilt of the orbit).

Table 3

Detection rate and photometry on the moons of pseudo-Sylvia,

assuming an NGAO system with 140 nm of wavefront error.

|  | Romulus |  | Remus |  | S_New1 |  | S_New2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Det. rate | m | Det. Rate | m | Det. Rate | m | Det. Rate | m |
| Perfect image | 100% | 6.6 | 100% | 8.1 | 100% | 6.9 | 100% | 9.6 |
| NIRC2-H | 82% | 6.40.04 | 70% | 8.30.3 | 11% | 6.90.2 | 0% | N/A |
| NGAO-H | 100% | 7.00.1 | 70% | 8.50.5 | 40% | 7.10.2 | 0% | N/A |
| NGAO-R | 100% | 6.600.01 | 100% | 8.30.1 | 100% | 6.91.1 | 100% | 10.10.3 |

### AO requirements

#### Wavefront error

A wavefront error of 140 nm would provide excellent angular resolution in the visible, better than HST and adequate for our program.  We expect excellent sensitivity for point source detection. Table 14 of the Keck NGAO Proposal to the SSC (June 2006) indicates that the point source limiting magnitude for such AO system (5s, 1hr integration) is 29.0 in R band. For comparison, recent observations of Pluto-Charon recorded with ACS/WFC at 0.61 m (Weaver et al. 2006) allowed the detection of 2 new moons with R = 23.4 (SNR=35). With NGAO in R band with 140 nm of wavefront error, these moons could have been discovered with SNR~47.  Such gain in sensitivity will help find more multiple systems, and also to find out if around these multiple systems there is still a ring of dust left over from the catastrophic collision that formed the multiple system.  We are currently carrying out simulations to characterize the science that could be done with 170 nm and 200 nm of wavefront error.  Our expectation is that there will not be a “cliff” in science output as the wavefront error degrades, but rather a gradual decrease in the number of moonlets detected and in the number of primary asteroids whose shapes can be measured.  Future releases of this Science Case Requirements Document will compare the science performance for 140, 170, and 200 nm of wavefront error.

#### Encircled energy

N/A

#### Contiguous field requirement

Required FOV is  2 arc sec.  There is no requirement for a larger contiguous field.

#### Photometric precision

Accurate photometry will lead to a better estimate of the size and shape of the moonlets, which will give strong constraints on their formation mechanism (e.g. one would be able to tell if the moonlet is synchronized and displays an equilibrium shape under tidal forces).  The proposed method is to detect photometric changes due to its potential lack of sphericity over the moonlet’s orbit, as we see different faces of the moonlet.  With current AO systems, the photometric accuracy on the moonlet is rather poor.  The accuracy of the flux estimate of the 22 Kalliope moonlet, orbiting at 0.6 arc sec with Dm=3, was only ~20% with Keck LGS AO.  Assuming the same sky background and detector noise as with current Keck LGS AO, NGAO in the near IR is predicted to yield a photometric accuracy of 5% or better for the same observing situation.

#### Astrometric precision

The astrometric measurements for our program are relative to the primary.  The maximum angular separation between the secondary and the primary is 0.7 arc sec.  We require the visible instrument to provide images with at least Nyquist sampling.  The relative position of the secondary, estimated by a Moffat-Gauss fit, cannot be better than a 1/4 of a pixel (since the primary is resolved).  The residual distortion over the field of the detector should not be more than 1.5 mas.  Uncharacterized detector distortion will be the limiting factor in these astrometric measurements.

#### Contrast

At the current time the faintest and closest moonlet discovered around an asteroid is Remus, orbiting at 0.2-0.5” (350-700 km) around 87 Sylvia with Dm (peak-to-peak) = 3.5. The detection of this moonlet is challenging with current Keck AO, and also with the VLT NACO system.  For instance, it was detected (SNR > 3) on 10 images out of 34 recorded over 2 months with the VLT.  A better contrast will increase the detection rate, allowing us to see fainter and closer moonlets  but also to get a better photometric measurement on those already known.  Coronagraphic observations cannot be considered in our case: the central source is not point-like so the effect of the mask will be negligible.  It is assumed that the distance to the primary of a satellite is driven by tidal effects, but at the moment theoretical work fails to agree on the age of an asteroid and the position of its moonlet. This is mostly due to the lack of observed systems in which a moonlet orbits at less than 1000 km (a / Rp <8). Two orders of magnitude gain in the detection limit (Dm = 5.5 at 0.5 arc sec) would lead to the possibility of detecting a half-size moonlet around (87) Sylvia.

#### Polarimetric precision

N/A

#### Backgrounds

Any background equal to or better than current Keck AO will be acceptable.  Lower backgrounds are always better.

#### Overall transmission

Comparable to or better than with current LGS AO system.

### Other key design features

#### Required observing modes

The capability of efficiently observing moving targets must be included in the design of NGAO, so that implementation of differential guiding when the tip-tilt source is not the object itself (and is moving relative to the target) is possible.  The maximum relative velocity to be expected is 70 arc sec per hour.

We also point out that for this science case, the scientific return of the Keck telescope and the NGAO system would greatly improve if some sort of flexible or queue scheduling or service observing were to be offered. With an error budget of 140 nm the NGAO system will achieve a Strehl of ~20% in R- band under moderate seeing conditions. Bright targets like the Galilean satellites (V~6) can be observed even if the seeing conditions are lower than average in the near IR (at separations > 1.2”). Other difficult observations, such as the study of multiple TNOs (V>17) could be scheduled when the seeing conditions were excellent (< 0.7”). Finally, frequent and extremely short (half hour) direct imaging observations of a specific target such as Io, to monitor its activity over a long period of time, would be extremely valuable and are not available on HST. All these programs could be done more easily if flexible or queue or service observing were available at Keck. It would also relax the constraints on the NGAO error budget since it would be possible to take advantage of excellent atmospheric conditions to observe the faintest objects.

#### Observing efficiency

Current observations with Keck AO have a ~25 minute overhead when switching between targets for an on-axis LGS observation of an asteroid.  It is very desirable to reduce this overhead.  A goal of 10 minutes setup time when switching between LGS targets is desirable.  There is no firm requirement, but observing efficiency suffers in direct proportion to the time it takes to switch from one target to the next, particularly when the observing time per target is relatively short.  This is an important constraint for this science case, since numerous targets must be observed per night.

### Instrument requirements

#### Required instruments

Primary: Visible imager, on-axis, diffraction limited, narrow field, with coronagraph

Secondary: Near IR imager, on-axis, diffraction limited, narrow field, with coronagraph

Secondary: Visible IFU, on-axis, narrow field, R~100

Secondary: Near infrared IFU, on-axis, narrow field, R~1000-4000

#### Field of view

No more than 4 arc sec.

#### Field of regard

Should be determined by the requirement to find adequate tip-tilt stars.

#### IFU multiplicity

Single object mode only.  Density of asteroids on the sky is not high enough for multi-object observing.

#### Wavelength coverage

Imaging: Wavelengths 0.7 – 2.4 m

Spectroscopy: Wavelengths 0.8 – 2.4 m

#### Spectral resolution

There are spectroscopic features at visible wavelengths (e.g. the absorption bands of pyroxene at 0.85 - 1 m).  For these bands, which are relatively broad, a spectral resolution of R~100 is desirable.  This could be accomplished either with a low resolution IFU spectrograph or with narrow-band filters.  There are also bands in the near IR.  SO2 frost (bands at 1.98 and 2.12 m) can be best observed with R~1000.  However R~4000 would be acceptable.

#### Requirements Summary

##### The requirements for the asteroid companions survey science case are summarized in the following table (see also the Multiple Asteroids 1: Survey mode to find new systems Observing Scenario).

Table 1. Asteroid Companions Survey driven requirements

General comment about this table: need to clarify and motivate choice of wavelength band at which contrast requirements are stated.  Current version switches from H to J to I to R bands frequently; hard to understand why.

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 1.1 | The companion sensitivity shall be ΔH ≥ 5.5 mag at 0.5” separation for a V ≤ 17 asteroid (J≤ 15.6) (asteroid size < 0.2”) with a proper motion of ≤ 50 arcsec/hour [ScRD §2.1.4.6, RollUp_v1 E3] | The asteroid can be used as tip/tilt guidestar (proper motion of ≤ 50 arcsec/hour). The AO system has sufficient field of view for objects and for their seeing disks (>3 arcsec, see # 1.8). The tip-tilt residual error will be less than 10 mas (limited by resolved primary) while guiding on one V=17 (J =15.6) object at 50 arcsec/hr (14 mas/sec). The AO system has sufficient Strehl to achieve this contrast ratio and sensitivity in 15 min exposure time. (need further simulations to state wavefront error requirement. ~170nm?) | Near-IR imager |
| 1.2 | H-band relative photometric accuracy (between primary and companion) of 5% at 0.6” for ΔH = 3 for a V ≤ 17 (J≤ 15.6) asteroid (asteroid size < 0.2”) with a proper motion of ≤ 50 arcsec/hour [ScRD §2.1.4.4, RollUp_v1 H3] | System has facilities and tools for calibrating the PSF to within xxx percent during one observation. (This is likely the width of the peak for astrometry. For photometry?) | Near- IR imager (no coronagraph because many asteroids will be resolved) |
| 1.3 | Target sample ≥ 300 asteroids in ≤ 4 yr [SCRD §2.1.3.2 ¶4] Insert in SCRD doc. Leads to requirement of ≥ 25 targets per 11 hour night [SCRD §2.1.5.2, RollUp_v1 M3] | Assumes 3 good nights per year. Needs high observing efficiency: Able to slew to new target and complete the entire observation within 26 minutes on average. |  |
| 1.4 | Observing wavelengths = J and H-band (R-band pending simulations) [SCRD 2.1.6.5, RollUp_v1 B3] | Transmit R through z to visible imager, z through H to IR imager. | Visible and IR imagers. |
| 1.5 | Spatial sampling ≤ Nyquist at the observing wavelength. [SCRD §2.1.6.1] Insert in SCRD |  | Spatial sampling ≤ Nyquist at the observing wavelength (l/4D to l /6D is optimal). |
| 1.6 | Field of view ≥ 3” diameter [SCRD §2.1.6.2] | AO system passes a >3” unvignetted field of view | Imager fields of view ≥ 3” |
| 1.7 | The following observing preparation tools are required: guide star finder for asteroids too faint to use as the only TT star, PSF simulation as function of wavelength and seeing conditions. [Need to retrofit into SCRD] | Guide star finder tool. PSF simulation tool (predict energy and width of central core to within 10%). |  |
| 1.8 | The following data products are required: Access to archive with proper identification in World Coordinate System (to within 1 arc sec or better) and with associated PSF calibrated to within xxx %. [Need to retrofit into SCRD] | Calibrated PSF capability (to what accuracy? Why) Ability to collect AO telemetry data to support the required PSF calibration. | FITS header system is capable of handling non-sidereal offsets in reporting object coordinates in the World Coordinate System to within 1 arc sec or better. |
| 1.9 | Observing requirements: Observer present either in person or via remote observing rooms, because real-time observing sequence determination is needed. | Classical observing mode or service mode with active observer participation. Remote observing capabilities must allow frequent real-time decisions by observer. |  |

The requirements for the asteroid companions orbit determination science case are summarized in the following table (see also the Multiple Asteroids 2: Orbits determination for the discovered system Observing Scenario).

Table 2. Asteroid Companions Orbit Determination driven requirements

need to clarify what wavelengths are needed and why

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 2.1 | Same as #1.1 | Same as #1.1 | Near-IR imager |
| 2.2 | The companion sensitivity shall be ΔI ≥ 7.5 mag at 0.75” separation for a V ≤ 17 asteroid (asteroid size < 0.2”) with a proper motion of ≤ 50 arcsec/hour [need source] |  | Imager [need simulations for wavelength range] |
| 2.3 | Photometric accuracy: Same as #1.2 | Same as #1.2 |  |
| 2.4 | H-band relative astrometric accuracy of ≤ 1.5 mas for a V ≤ 17 (J ≤ 15.6) asteroid (asteroid size < 0.2”) with a proper motion of ≤ 50 arcsec/hour [SCRD §2.1.4.5 RollUp_v1 I3] | Non-sidereal tracking accuracy <10 mas. Thoughput, emissivity to the guider sufficient to achieve SNR ≥ 400 on the primary in <1 second. [Do we need this additional requirement on SNR if the non-sidereal tracking accuracy is < 10 mas? And where does the <1 sec requirement come from?] | Uncalibrated detector distortion < 1.5 mas (Is this an edge to edge number? Over xxx pixels? Need to be more explicit.) |
| 2.5 | Target sample size of ≥ 100 asteroids in ≤ 4 years [SCRD §2.1.3.2 ¶4]. Leads to requirement of ≥ 25 targets in an 11 hour night. [SCRD §2.1.5.2, RollUp_v1 M3] | Assumes 3 good nights per year, need 3 epochs per target. Needs high observing efficiency: Able to slew to new target and complete the entire observation within 25 minutes on average. Will generally only observe at one wavelength (the one that gives the best astrometric information). |  |
| 2.6 | Observing wavelengths = J and H-band [SCRD §2.1.6.5, RollUp_v1 B3] | Same as #1.6 (Note: R-band may become a future requirement if R-band Strehl > 15%) | Near-IR imager (R-band imager) |
| 2.7 | Spatial sampling same as #1.5 |  | Same as #1.5 |
| 2.8 | Field of view ≥ 3” diameter [SCRD §2.1.6.2] | AO system passes a >3” unvignetted field of view | Imager fields of view ≥ 3” |
| 2.9 | Same as #1.7 | Same as #1.7 |  |
| 2.10 | Same as #1.8 | See #1.8 |  |
| 2.11 | Observing requirements: 8 epochs per target per semester. [SCRD §2.1.3.2 ¶4 6-orbital elements+ time. Need to insert in SCRD.] |  |  |

The requirements for the asteroid size and shape (characterize surface and orbital parameters) science case are summarized in the following table.  In addition to the requirement of a high resolution visible imager, the slope of the visible spectrum is needed to determine the asteroid age or surface type.  This case requires a spectral resolution of R ~ 100 for 0.7 – 1.0 µm wavelength with Nyquist sampling.  Since the slope of the spectra is of importance this could be accomplished with a visible IFU, a visible slit spectrograph, or narrow-band filters.

Table 3. Asteroid size, shape, and composition derived requirements

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 3.1 | The companion sensitivity shall be ΔI ≥ 7.5 mag at 0.75” separation for a V ≤ 17 asteroid (asteroid size < 0.2”) with a proper motion of ≤ 50 arcsec/hr [RollUp_v1 F2] | The asteroid can be used as the tip/tilt guidestar (proper motion of ≤ 50 arcsec/hr). The AO system has sufficient field of view for both objects and their seeing disks (>3 arcsec, see requirement 1.8). The tip-tilt residual error will be less than 10 mas while guiding on one V=17 (specify J mag) object at 50 arcsec/hour (14 mas/sec). The AO system has sufficient Strehl to achieve this contrast ratio and sensitivity in 15 minutes of exposure time. (need further simulations to state wavefront error requirement.) | I-band imager. |
| 3.2 | Target sample size of ≥ tbd asteroids in ≤ tbd years [needs to be specified]. ≥ 10 targets in an 11 hour night [SCRD §2.1.5.2, RollUp_v1 M2] | #2.5 is stricter requirement. |  |
| 3.3 | Observing wavelengths 0.7 – 1.0 µm [SCRD §2.1.6.5, RollUp_v1 B2] | AO system must pass 0.7 to 1.0 micron wavelengths with sufficient sensitivity to satisfy #3.1 | Visible imager (R through I band) with narrow-band filters or slit spectrograph (R~100), or possibly visible IFU (R~100). |
| 3.4 | Spatial sampling same as #1.5 | Same as #1.5 | Same as #1.5 |
| 3.5 | Field of view ≥ 2” diameter [SCRD §2.1.6.2] | Same as #2.8 | Same as #2.8 |
| 3.6 | Ability to measure the spectral slope with R ~ 100 at 0.85-1.0 m [SCRD §2.1.6.6, §2.5] |  | Spectroscopic imaging at R ~ 100 across R and I-band, potentially with narrow-band filters (how many? at what spacing?) |
| 3.7 | Ability to measure the SO2 frost bands at R=1000 (R=4000 is acceptable) at 1.98 and 2.12 m [SCRD §2.1.6.6, §2.5] |  | Spectroscopic imaging at R ~ 1000 or 4000 in the K band. Are there other lines of interest possibly in H or J? |
| 3.8 | Same as #1.7 | Same as #1.7 |  |
| 3.9 | Same as #1.8 | Same as #1.8 |  |

### References

Marchis, F., J. Berthier, P. Descamps, et al. 2004b. Studying binary asteroids with NGS and LGS AO systems, SPIE Proceeding, Glasgow, Scotland, 5490, 338.

Marchis, F., Descamps, P., Hestroffer, D. et al., 2005. Discovery of the triple asteroidal system 87 Sylvia, Nature 436, 7052, 822.

Marchis, F.; Kaasalainen, M.; Hom, E. F. Y.; Berthier, J.; Enriquez, J.; Hestroffer, D.; Le Mignant, D.; de Pater, I. 2006. Shape, size and multiplicity of main-belt asteroids, Icarus 185, 39.

Tanga, Paolo; Consigli, J.; Hestroffer, D.; Comito, C.; Cellino, A.; Richardson, D. C. 2006, Are Asteroid Shapes Compatible With Gravitational Reaccumulation? American Astronomical Society, DPS meeting #38, #65.06

Weaver, H. A.; Stern, S. A.; Mutchler, M. J.; Steffl, A. J.; Buie, M. W.; Merline, W. J.; Spencer, J. R.; Young, E. F.; Young, L. A. 2006, Discovery of two new satellites of Pluto, Nature, 439, 943.

## Moons of the Giant Planets

### Summary of Requirements

The requirements for the moons of giant planets science case are summarized in the following table.

Table 4. Moons of giant planets derived requirements

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 4.1 | Able to map intensity distribution with a spatial resolution ≤ 20 mas at 0.7 µm (are 0.8 or 0.9 microns OK? what science would be lost?) for a target with V = 5 to 8 with proper motion ≤ 30” per hour Need to lay out in SCRD what unique info about each moon can be gained with NGAO in the visible, as opposed to the NIR. | The moon will be used as the tip/tilt and NGS guidestar (at a non-sidereal rate of 30 arcsec/hr). Tracking (tip-tilt) accuracy must be better than 5 mas. Need to simulate and calculate what wavefront error would give the required “spatial resolution” at 0.7 or 0.8 or 0.9 microns. | Visible imager. |
| 4.2 | Able to acquire Io within 5” of Jupiter and to track it to within 2.5” of Jupiter. Note that this is a goal but perhaps not a rigid requirement: we know we can acquire within 10” today. | May require either a diaphragm or a filter to attenuate the light from Jupiter. | See AO derived requirement at left. |
| 4.3 | Relative photometric accuracy of ≤ 0.05 mag on target. (What science is possible at a variety of levels of accuracy? Need to rephrase photometric requirement as a function of observing wavelength, and take into account possible cirrus) | Puts a requirement on Strehl and PSF knowledge at each wavelength. What is this requirement? | Detector flat-fielding requirements, linearity, etc will flow down from required photometric accuracy. Need to specify. |
| 4.4 | Absolute photometric accuracy ≤ 0.05 mag on target. (What science is possible at a variety of levels of accuracy? Need to rephrase photometric requirement as a function of observing wavelength, and take into account possible cirrus) | Puts a requirement on Strehl and PSF knowledge at each wavelength. What is this requirement? | Detector flat-fielding requirements, linearity, etc will flow down from required photometric accuracy. Need to specify. |
| 4.5 | Relative astrometric accuracy ≤ 2 mas on target to determine positions of features within disk of a moon (diameter ≤ 1.2 arc sec). |  | Well-calibrated detector distortion (see # 2.4) |
| 4.6 | Observing wavelengths 0.7 to 2.4 µm (how crucial is 0.7 microns to the science, compared with 0.8 or 0.9?) | AO system must pass these wavelengths to science instruments. | Visible imager. Near-IR imager. May require a number of (how many?) specific narrow-band filters; implies needed flexibility in filter wheels. |
| 4.7 | Spatial sampling ≤ Nyquist at all wavelengths |  |  |
| 4.8 | Moons are very bright: do not allow saturation. Typical brightness: 5 mag per square arc sec. Check this. |  | Either need to use neutral density filters, or have a fast shutter, or have a detector with large wells or very short exposure times (and low read noise). Note: these observations will have high overhead. |
| 4.9 | Field of view ≥ 2” diameter. | Must pass ≥ 2” | Minimum FOV 2” |
| 4.10 | The following observing preparation tools are required: PSF simulation, target ephemeris, exposure time calculator to enable choice of ND filter and exposure time. |  |  |
| 4.11 | The following data products are required: Calibrated PSF. (to what accuracy?) |  |  |
| 4.12 | Io and Titan are time domain targets; Io requires ≤ 1 hr notification of volcano activity. Typical timescales for clouds on Titan are of order days or weeks. |  |  |

#### Uranus and Neptune

Text and tables not yet written

Table 5.  Uranus and Neptune derived requirements

## Imaging and characterization of extrasolar planets around nearby stars

Author: Michael Liu

Editor: Claire Max

### Scientific background and context

The unique combination of high-contrast near-IR imaging (K-band Strehl ratios of 80-90%) and large sky coverage delivered by NGAO will enable direct imaging searches for Jovian-mass planets around nearby young low-mass stars and brown dwarfs.  Both the Gemini Observatory and ESO are developing highly specialized planet-finding AO systems with extremely high contrast for direct imaging of young planets.  These "extreme AO" systems are very powerful, but their design inevitably restricts them to searches around bright, solar-type stars (I=8 to 9 mag).

NGAO will strongly distinguish work at WMKO from all other direct imaging searches planned for large ground-based telescopes.  By number, low-mass stars (M0.5 MSun) and brown dwarfs dominate any volume-limited sample, and thus these objects may represent the most common hosts of planetary systems.  Such cool, optically faint targets will be unobservable with specialized extreme AO systems because their parent stars are not bright enough to provide a high-order wavefront reference.  But thousands of cool stars in the solar neighborhood can be targeted by NGAO.  Direct imaging of extrasolar planets is substantially easier around these lower mass primaries, since the required contrast ratios are smaller for a given companion mass.

### Scientific goals

Direct imaging of extrasolar planets by NGAO would allow us to measure their colors, temperatures, and luminosities, thereby testing theoretical models of planetary evolution and atmospheres.  NGAO spectroscopic follow-up will be an important means to characterize the atmospheres of extrasolar planets, which are otherwise essentially inaccessible to spectroscopy.  Figure 2 summarizes the relative parameter space explored by NGAO and extreme AO.  The complementarity of the two systems is very important: establishing the mass and separation distribution of planets around a wide range of stellar host masses and ages is a key avenue to understanding the planet formation process.  The optical faintness of low-mass stars, brown dwarfs and the very youngest stars make them inaccessible to extreme AO systems but excellent targets for NGAO.

Figure 2 Schematic illustration of the parameter space of Keck NGAO and of the Gemini Planet Imager for direct imaging of extrasolar planets.

#### Planets around low-mass stars and brown dwarfs

Direct imaging of substellar companions (brown dwarfs and extrasolar planets) is substantially easier around lower mass primaries, since the required contrast ratios are smaller for a given companion mass. Indeed, the first bona fide L dwarf and T dwarfs were discovered as companions to low-mass stars (Becklin & Zuckerman 1988, Nakajima et al. 1995).  Thus, searching for low-mass stars and brown dwarfs is an appealing avenue for planet detection and characterization.  Given that low-mass stars are so much more abundant than higher mass stars, they might constitute the most common hosts of planetary systems.  Figure 3 shows an estimate of the planet detection sensitivity for NGAO.

Figure 3

Estimated NGAO sensitivity for direct imaging of planets around low-mass stars (red lines) and brown dwarfs (green lines).   NGAO will be able to search for Jovian-mass companions around large numbers of low-mass stars and brown dwarfs in the solar neighborhood

[Need to specify wavefront error for these graphs, and compare for several different values of the WFE (e.g. 140, 170, 200 nm).]

Spectroscopic follow-up of the coldest companions will be an important path in characterizing the atmospheres of objects in the planetary domain.  Strong molecular absorption features from water and methane provide diagnostics of temperature and surface gravity at modest (R~100) spectral resolution.  Below ~500 K, water clouds are expected to form and may mark the onset of a new spectral class, a.k.a. ”Y dwarfs”.  Such objects represent the missing link between the known T dwarfs and Jupiter, but are probably too faint and rare to be detected as free-floating objects in shallow all-sky surveys such as 2MASS and SDSS.  Furthermore, the coolest/lowest mass objects may not exist as free-floating objects if there is a low-mass cutoff to the initial mass function of the star formation process, e.g., from opacity-limited fragmentation of molecular clouds (Mmin ~ 5-10 MJup; Silk 1977).  Even cooler/lower mass objects might only form via fragmentation, akin to the formation of binary stars, and may only be found as companions.

#### Very young planets in the nearest star-forming regions

Imaging searches and characterization at the very youngest (T Tauri) stages of stellar evolution provide a unique probe of the origin of extrasolar planets, by constraining their formation timescales and orbital separations. Young stars and brown dwarfs can be enshrouded by substantial dust extinction, both from the natal molecular cloud and their own circumstellar material.  Thus most young (T Tauri) stars are too optically faint for current NGS AO systems or future ExAO systems.  Keck NGAO imaging will probe physical separations of ≥ 5-10 AU around these stars.

It is still an open question whether giant planets form extremely rapidly (≤ 104 yr) due to disk instabilities (e.g. Boss 1998) or if they first assemble as ~10 Mearth rocky cores and then accrete ~300 Mearth of gaseous material over a total timescale of ~1-10 Myr (e.g. Lissauer 1998).  Potentially both mechanisms may be relevant, depending on the range of orbital separations and circumstellar disk masses.  In addition, imaging searches of both young T Tauri stars with disks (classical TTS) and without disks (weak TTS) can help to constrain the formation timescale.  In particular, weak T Tauri stars with planetary companions would suggest that planet formation could occur even when disk evolution/dissipation happens rapidly.

### Proposed observations

See above description.

### AO and instrument requirements

The high contrast near-IR (0.9-2.5 micron) imaging required for planet imaging will require coronagraphy to suppress PSF diffraction features.

In many cases, near-IR tiptilt sensing is required given the intrinsic redness of the science targets (e.g. brown dwarfs) or the high extinction of the science regions (e.g. star-forming regions). Both on-axis and off-axis tip-tilt sensing will be needed, depending on the optical/IR brightness of the primary stars. For off-axis science applications, sky coverage of >30% (as an areal average over the entire sky) is needed at the highest image quality over a corrected field of view < 5" in size.

Low-resolution (R~100) near-IR (0.9-2.4 micron) spectroscopy is essential to follow-up planet discoveries, in order to determine their temperatures, surface gravities, and masses. The relevant spectral features have broad wavelength ranges, e.g. the broad-band SEDs of circumstellar dust needed to diagnose grain composition and sizes and the broad molecular absorption band of H2O and CH4 present in the atmospheres of ultracool brown dwarfs and extrasolar planets.  Thermal (L-band) imaging would be desirable to help measure the SEDs of the planets, but is not essential for this science.

### Performance Requirements

#### Wavefront error

The key performance driver for this science case is contrast, not wavefront error.  Initial simulations with an RMS wavefront error of 140 nm indicated that the required contrast could be achieved.  Simulations are under way to understand how the science would degrade for wavefront errors of 170 and 200 nm (relative to 140 nm).  Good control and calibration of internal static wavefront calibration errors will also be required of the AO system.

#### Encircled energy

N/A

#### Need for large contiguous fields

Required FOV is only a few (< 5) arc sec.  A larger contiguous field is not required.

#### Photometric precision

Probably not a key requirement.  Require relative photometry of planetary companion to primary star of better than 0.05 mag, or absolute photometry of planetary companion to the same accuracy.

#### Astrometric precision

Probably not a key requirement.  Astrometric accuracy to ~1/10 of the PSF FWHM would suffice for proper motion confirmation that candidate planets are physically associated to their primaries.

#### Contrast

From a science standpoint, the required contrast can be set by the need to directly image Jupiter-mass planets around a large sample of (1) field low-mass stars and brown dwarfs at ages of <~200 Myr and (2) young stars in the nearest star-forming regions.  A benchmark value of DH=13 magnitudes at 1 arc sec separation is required.

#### Polarimetric precision

N/A

#### Backgrounds

Thermal L-band photometry is desirable, but not a key requirement.  Backgrounds lower than those on the current Keck AO system would be helpful for L-band photometry.

#### Overall transmission

For some targets with low-mass primaries (brown dwarfs) and relatively old ages, high sensitivity will be a benefit at separations of ≥ 1 arc sec.  The baseline sensitivity numbers from the NGAO proposal of H=25 mag (5-sigma) in 20 minutes of on-source integration time are suitable for these purposes.

### Other key design features

#### Required observing modes

Imaging with both on-axis and off-axis tip-tilt stars, and single-object spectroscopy will be needed.  Coronagraphic imaging is required, and will need additional design consideration because one must be able to center the science target on the focal plane mask and to keep it there during the observations.

#### Observing efficiency

Good efficiency is required, i.e. 10 min or less overhead per target, since we want to be able to observe many (several dozen) targets per night.

### Instrument requirements

#### Instruments needed

Primary: Near-IR diffraction limited imager, narrow field, 0.9-2.4 microns, coronagraph

Secondary: Near-IR IFU, R~100, diffraction limited, 0.9-2.4 microns, coronagraph?

Secondary: L-band imager

#### Field of view

No more than 5 arc sec for both the near IR imager and the R~100 IFU.

#### Field of regard

Determined by the need for tip-tilt stars.  All-sky average sky coverage for off-axis observations should be >30%.

#### IFU multiplicity

One object at a time only.

#### Wavelength coverage

0.9-2.4 microns (extension to L-band desirable, but not essential).

#### Spectral resolution

R~100 at 0.9-2.4 microns.

### Summary of Requirements

The requirements for the planets around low-mass stars science case are summarized in the following table.  The key area in which NGAO will excel is the detection of planets around low-mass stars and brown dwarfs because Keck, unlike GPI, will be able to use a laser guide star.  NGAO will also be able to search for planets around young solar-type stars where dust extinction is significant.  JWST will have coronagraphic capability in the 3 to 5 m window, but will have significantly lower spatial resolution than Keck NGAO.  In terms of the types of solar systems that can be studied, this means that JWST will focus on older, nearby main sequence stars (since older giant planets will remain visible in 3 to 5 m for a longer time).  JWST may be more limited than NGAO in doing large surveys, because of its longer slewing time and possibly a lifetime limit on the total number of slews.

Table 6. Planets Around Low Mass Stars derived requirements

|  | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 6.1 | Target sample 1: Old field brown dwarfs out to distance of 20 pc. Sample size several hundred, desired maximum survey duration 3 yrs (practical publication timescales). | Observe 20 targets per night (each with e.g. 20 min integration time). Need a requirement on tip-tilt stars (what magnitude are the parent stars?) | Near infrared imager (possibly with coronagraph). Survey primary stars at J- and H-band. |
| 6.2 | Target sample 2: Young (<100 Myr) field brown dwarfs and low-mass stars to distance of 80 pc. Sample size several hundred, desired maximum survey duration 3 yrs. | Observe 20 targets per night (each with e.g. 20 min integration time). | Near infrared imager (possibly with coronagraph). Survey primary at J- and H-band. Could benefit from dual- or multi-channel mode for rejecting speckle suppression, but not essential for this program. |
| 6.3 | Target sample 3: solar type stars in nearby star forming regions such as Taurus and Ophiuchus, and young clusters @ 100 to 150 pc distance. Bright targets (on-axis tip-tilt generally possible: V=14-15, J=10-12). Sample size several hundred, desired maximum survey duration 3 yrs. | (May not require LGS if there is a good enough near-IR wavefront sensor available). | Possible dual- or multi-channel mode for speckle suppression. Alternatively an IFU would help, provided it is Nyquist sampled at H and has FOV > 1 arc sec. Min. IFU spectral resolution is R~100. May need IR ADC for imaging or coronagraphic observations (J or H bands); typical airmass is 1.7 for Ophiuchus. |
| 6.4 | Companion Sensitivity Sample 1: assume no companions beyond 15 AU. Targets at 20 to 30 pc; companion distribution peaks at 4 AU = 0.2"; this yields 2 MJupiter planets at a 0.2" separation with contrast ΔJ = 10. Will be retrofitted into SCRD. Should put in apparent magnitudes | Excellent (5 nm rms? needs simulation) calibration of quasi-static non-common path aberrations, especially at mid-spatial-frequencies. Needs algorithms such as phase retrieval or speckle nulling (on a fiber source + good stability). Small servo-lag error (how small?) to avoid scattered light at 0.2 arc sec. Source: Bruce Macintosh’s error budget and simulations. Need to document. | 6 to 10 λ/D general-purpose coronagraph. Need to describe the coronagraph more explicitly. Speckle suppression capability (multi-spectral imaging) Simulations needed. |
| 6.5 | Companion Sensitivity Sample 2: a 1 MJupiter planet is at 300K, J=22, ΔJ = 11 (2 MJupiter is 2.5 mags brighter). This distribution could have a wider distribution of binaries a) 0.1" separation, ΔJ = 8.5 (2MJ) b) 0.2" separation, ΔJ = 11 (1MJ) c) Goal ΔJ = 11 at 0.1" separation (1MJ) based on properties of the planets you want to look for. Put in apparent magnitudes | Excellent (5 nm rms? needs simulation) calibration of quasi-static non-common path aberrations, especially at mid-spatial-frequencies. Needs algorithms such as phase retrieval or speckle nulling (on a fiber source + good stability). Small servo-lag error (how small?) to avoid scattered light at 0.2 arc sec. Source: Bruce Macintosh’s error budget and simulations. Need to document. | a) 6 λ/D general-purpose coronagraph b) 6 or 10 λ/D general-purpose coronagraph c) (Goal) Not achievable with a general purpose coronagraph May need small Inner Working Distance coronagraph. Non-redundant aperture masking is an interesting approach for this, limits currently unknown, probably requires low read noise in science detector. |
| 6.6 | Goal: Companion Sensitivity Case 3 (need to do simulations): at 5 Myr , 1 Msun primary; a) goal ΔJ = 13.5 to see 1 MJupiter or b) goal ΔJ = 9 for 5 MJupiter. 0.2" is uninteresting, 0.07" is needed. Should put in apparent magnitudes (probably J = 22 or 23) | Excellent (5 nm rms? needs simulation) calibration of quasi-static non-common path aberrations, especially at low-spatial-frequencies. Needs algorithms such as phase retrieval or speckle nulling (on a fiber source + good stability). Small servo-lag error (how small?) to avoid scattered light at 0.2 arc sec. Source: Bruce Macintosh’s error budget and simulations. Need to document. | a) Not achievable with a general purpose coronagraph (requires a very small IWA coronagraph, non-redundant masking – maybe have a conversation with Ireland, Lloyd or Tuthill). b) 6 λ/D general-purpose coronagraph |
| 6.7 | Sensitivity of H=25 for 5-sigma detection in 20 minutes, at 1 arcsec separation from primary star. [RollUp_v1 G5] | Sufficiently high throughput and low emissivity to permit detecting H=25 in 20 minutes at 5 sigma above background. (need to derive throughput and emissivity numbers) Sufficiently low wavefront error to permit the coronagraph to suppress scattered light from primary to this level. TBD. 140nm meets goal; simulations by Ralf Flicker of 170nm underway. May be in KAON. [SCRD §2.2.5.1] |  |
| 6.8 | H-band relative photometry (between primary and companion): accuracy ≤ 0.1 mag for recovered companions; goal of measuring colors to 0.05 mags (0.03 mag per band) [where did this requirement come from? Needed to measure temperatures and surface gravities sufficiently accurately. For what?] | Diagnostics on AO data to measure Strehl fluctuations if it takes a while to move on and off the coronagraph (a possible more attractive solution is a specialized coronagraph that simultaneously images the primary) | Induced ghost images of primary; or rapid interleaving of saturated and unsaturated images; or a partially transparent coronagraph |
| 6.9 | Requirement: Astrometric precision 2 mas (~1/10 PSF) relative between primary and planet, for initial rejection of background objects. [SCRD §2.2.5.5, RollUp_v1 I5] Goal: For measuring orbits of nearby field objects, want 0.5 mas to measure masses to 10%. Note this gives you mass of primary star. Difficult goal: To get mass of companion you need absolute astrometry at very low level (0.1 mas?). It’s unclear if we can meet this goal. Could be combined with Doppler measurements if that’s practical for the brighter objects. Need to elaborate this. All of this has to be retrofitted into SCRD | Ways to do this: a) Position stability requirement for star behind coronagraph. 2) Induced ghost image method. Needs a wire grating ahead of the coronagraph, or use DM to induce ghost images. (papers by Marois et al. and Sivaramakrishnan et al ApJL 2006). Get reference | Distortion requirement TBD (similar to asteroid science case). Also want ghost images of primary (as for photometry #5.7) in order to locate it accurately relative to planet. |
| 6.10 | Efficiency: 20 targets per night (30 goal) [SCRD §2.2.6.2] | AO system must be able to absolutely steer objects so they land on the coronagraph. This implies 0.005" reproducibility of field steering –or lock the tip/tilt to this accuracy relative to coronagraph field stop. How did we calculate this required accuracy number? |  |
| 6.11 | Observing wavelengths JHK bands (strong goal: Y and z for companion characterization) [SCRD §2.2.4, §2.2.7.1, RollUp_v1 B5] | Transmit JHK to science instrument. Goal: Y and z. | Methane band filters for rapid discrimination, Y and z, and/or a custom filter for early characterization. |
| 6.12 | Able to register and subtract PSFs (with wavelength, time, etc.) for post-processing to get rid of residual speckles. (To what accuracy do you need to subtract PSFs?) | What is the derived requirement here? | 1.5 x better than Nyquist sampled at J (goal Y) |
| 6.13 | Field of view: must see companions at 100 AU scales at 30 pc (goal 20 pc) [SCRD §2.2.5.3] |  | Field of view 3" radius (goal 5" radius) |
| 6.14 | Characterization of companion [SCRD §2.2.4, §2.5] |  | a) R ~150 IFU, sub-Nyquist sampling spectrograph, or if above not available, b) Nyquist spatial sampling IFU, R ~ 4,000, OH suppressing). Both must be sensitive to J = 22 or 23 (in how long an integration time?). |
| 6.15 | Sky Coverage >30%. How was this derived? [SCRD §2.2.7.3, RollUp_v1 N5] | Technical field for low-order wavefront guidestar pickoff at least 150 (180?) arcseconds to achieve this at high galactic latitude. Ability to acquire and track 3 tip/tilt stars. (More lenient if parent star can be used as one of the three TT stars.) |  |
| 6.16 | The following observing preparation tools are required: guide star finder for high proper-motion stars |  |  |

### References

Becklin, E. & Zuckerman, B. 1988, Nature 336, 656

Boss, A. 1998, ApJ 503, 923

Lissauer, J. 1998, in “Origins,” ASP Conf. Series vol. 148, 327

Nakajima, T. et al. 1995, Nature 378, 463

Silk, J. 1977, ApJ 214, 152

## Debris Disks and Young Stellar Objects

### Debris Disks

Text and tables not yet written.  Will be a driver for coronagraph design.

Table 7.  Debris Disks derived requirements

### Young Stellar Objects

Text and tables not yet written.  May be a driver towards having an infrared wavefront sensor.

Table 8.  Young Stellar Objects derived requirements

## Astrometry Science in Sparse Fields

Text and tables not yet written.  Will be a driver for low and/or very well calibrated instrument distortions, compensation for atmospheric differential refraction, and good temperature control of the AO system.

Table 9.  Astrometry Science in Sparse Fields derived requirements

## Precision Astrometry: Measurements of General Relativity Effects in the Galactic Center

Authors: Andrea Ghez and Jessica Lu

Editor: Claire Max

### Scientific background and goals

The proximity of our Galaxy's center presents a unique opportunity to study a massive black hole (BH) and its environs at much higher spatial resolution than can be brought to bear on any other galaxy. In the last decade, near-IR observations with astrometric precisions of < 1 mas and radial velocity precision of 20 km/s have enabled the measurement of orbital motions for several stars near the Galactic center (GC), revealing a central dark mass of 3.7 x 106 MSun (Ghez et al. 2003, Ghez et al. 2005; Schodel et al. 2002; Schodel et al. 2003). Radio VLBA observations have now resolved the central object to within several multiples of the event horizon, indicating that the central mass is confined to a radius smaller than 1 AU (Shen et al. 2005). These observations provide the most definitive evidence for the existence of massive BHs in the centers of galaxies. The orbital motions now also provide the most accurate measurement of the GC distance R0, constraining it to within a few percent (Eisenhauer et al. 2003).

### General Relativistic Effects

Due to the crowded stellar environment at the GC and the strong line-of-sight optical absorption, tracking the stellar orbits requires the high angular resolution, near-IR imaging capabilities of adaptive optics on telescopes with large primary mirrors, such as Keck. Though the current orbital reconstructions are consistent with pure Keplerian motion, with improved astrometric and radial velocity precision deviations from pure Keplerian motion are expected. With Keck NGAO we will be able to detect the deviations from Keplerian motion due to a variety of effects. These will provide a unique laboratory for probing the dynamics of galactic nuclei, the properties of exotic dark matter, and the mass function of stellar-mass black holes. They will also provide the first tests of general relativity in the high mass, strong gravity, regime. Keck NGAO will measure these non-Keplerian motions to precisions that will not be greatly surpassed even in the era of extremely large (~30m) telescopes.

Of the theories describing the four fundamental forces of nature, the theory that describes gravity, general relativity (GR), is the least tested. In particular, GR has not been tested in the strong field limit, on the mass scale of massive BHs. The highly eccentric 15 yr orbit of the star S0-2 brings it within 100 AU of the central BH, corresponding to ~1000 times the BH's Schwarzschild radius (i.e., its event horizon). Studying the pericenter passage of S0-2 and the other high eccentricity stars therefore offers an opportunity to test GR in the strong gravity regime.

With Keck NGAO, the orbits can be monitored with sufficient precision to enable a measurement of post-Newtonian general relativistic effects associated with the BH. This includes the prograde precession of orbits. As Figure 1 illustrates, the General Relativistic prograde precession can be measured even for single orbits of known stars (e.g., S0-2, K=14.1 mag) if we have an astrometric precision of  ~ 100 μas coupled with a radial velocity precision of 10 km/s.

With Keck NGAO, the orbits can be monitored with sufficient precision to enable a measurement of post-Newtonian general relativistic effects associated with the BH. This includes the prograde precession of orbits. As Figure 4 illustrates, the General Relativistic prograde precession can be measured even for single orbits of known stars (e.g., S0-2, K=14.1 mag) if we have an astrometric precision of  ~ 100 μas coupled with a radial velocity precision of 10 km/s.

Figure 4

Required astrometric precision for detecting, from top to bottom, GR effects associated with relativistic prograde precession, extended mass within the stellar orbits, and frame-dragging effects due to BH spin (based on Weinberg et al. 2005). Estimates are based on measurements of stellar orbits and positions from Keck diffraction-limited images (thick, solid lines), and assume radial velocity measurement errors of 10 km/s. The stellar orbits include 16 stars within 0.5” of Sgr A* with orbital fits obtained from speckle imaging measurements and 142 stars within 1” of Sgr A* with stellar positions obtained with new, deep AO maps. For comparison, we also show estimates based on measurements from just the short-period star S0-2 (thin, dashed line). Results are for a 10-year baseline with 10 integrations per year. Low-order GR and extended matter effects are detectable at the >7 level if an astrometric  precision of ~100 as can be achieved.  Detection of BH spin requires either better precision or, at ~100 as precision, improved SNR obtained by observation of multiple as-yet-undiscovered high-eccentricity, short-period stars over multiple orbits.

Keck NGAO will bring several important improvements to measurements at the Galactic Center:

1) Current measurements are strongly confusion limited, because the Galactic Center is a very crowded field.  Higher Strehl at K-band will improve contrast and therefore reduce the confusion, improving both photometric and astrometric accuracy because the previously undetected faint star population will cause less of a bias in the positions and magnitudes of brighter stars.

2) Higher Strehl at K-band will allow the detection of new stars, some of which may pass close enough to the black hole to contribute to the obtainable accuracy and precision of General Relativistic effects.  (See figure caption.)

3) NGAO’s use of multiple laser guide stars and multiple IR tip-tilt stars will decrease the field dependence of the PSF, thereby increasing both photometric and astrometric accuracy.  This effect needs be quantified.

4) The accuracy of current radial-velocity measurements is limited by signal to noise.  NGAO’s higher Strehl and lower sky background will materially improve the radial-velocity contribution to orbit determinations.

### R0 and the dark matter halo

Since the orbital periods are proportional to R03/2Mbh-1/2 and the radial velocities are proportional to R0-1/2Mbh1/2, where R0 is the heliocentric distance to the BH and Mbh its mass, the two parameters are not degenerate and can be determined independently (Salim & Gould 1999). As shown in Figure 4, by complimenting high precision astrometric measurements with high precision radial velocity measurements with accuracies of ~10 km s-1, we can measure R0 to an accuracy of only a few parsecs (i.e., ~0.1% accuracy) with Keck NGAO.  Today’s radial velocity precision for the observations in hand is about 20 km/s.  This could be improved to 10 km/sec with higher signal to noise observations, either from longer integration times or lower backgrounds.

Since R0 sets the scale within which is contained the observed mass of the Galaxy, measuring it to high precision enables one to determine to equally high precision the size and shape of the Milky Way's several kpc-scale dark matter halo (Olling & Merrifield 2000). The halo shape tells us about the nature of dark matter (e.g., the extent to which it self-interacts) and the process of galaxy formation (how the dark matter halo relaxes following mergers). Currently the shape is very poorly constrained.

### Proposed observations and targets

Target: Central 10 arc sec of the Galactic Center, centered on SgrA*.  Note that this is a low-elevation target from Keck (RA 17 45 40   DEC -29 00 28).

Observing wavelengths: K band (2.2 microns)

Observing mode: Imaging for astrometry purposes, and spectroscopy for radial velocities

### Observing plan for Astrometric Imaging:

Based upon the way things are done today using 1st-generation Keck AO:

- Guide Star Aquisition:

Current visible-wavelength guide star is USNO-A2.0 0600-28577051 (R=14.0, Separation = 19.3’’)

There are a great many possible IR tip-tilt stars, as shown in Appendix A.  The addition of multiple IR-corrected tip-tilt stars is anticipated to improve astrometric accuracy considerably, although more work is needed in order to understand the specific limitations of today’s observations.

- 1-minute K’ exposures, continuing for 3.5 hours elapsed time

- Dither pattern is random over a 0.7’’ box (small box used to minimize distortion)

- Construct 40 arcsec mosaics to tie to radio astrometric reference frame (radio masers)

- After the Galactic Center has set, move to a dark patch of sky at a similar airmass to obtain sky exposures.

Standard stars: none (astrometry)

Data Analysis:

- Image reduction is standard, including distortion correction using the NIRC2 pre-ship review distortion solution.  Improved distortion solution is needed and appears to be possible with data in hand.

- Individual exposures are shifted (translations only) and added together for an entire night to produce a final map.  Information from >1000 stars is included in the solution.

- Individual exposures are also divided into 3 subsets of equal quality to produce 3 images used for determining the astrometric and photometric RMS errors.

- Source extraction is performed using StarFinder (Diolaiti et al 2000) which iteratively estimates the PSF from several bright stars in the image and then extracts all source positions and photometry.

- Star lists from different epochs are aligned by matching all the stars and minimizing the quad-sum of their offsets allowing for a 2nd order transformation between epochs.

### Observing plan for Radial Velocity Measurements (IFU spectroscopy):

K-band IFU spectroscopy, one field

20 or 35 mas plate scale, R~4000

FOV at least 1.0’’ x 1.0’’

Exposure times are currently 15 minutes.

Sky frames of the same duration are obtained in the same mode after the Galactic Center sets in order to remove OH lines.

Obtain standards stars of A and G spectral type to remove telluric lines.

Data analysis performed with a provided pipeline to do wavelength calibration.

### Current issues and limitations that could be further explored with existing data sets

1) Improved geometric distortion map for narrow camera on NIRC2.  At present we know that the map from pre-ship review is incorrect at the half-pixel level.

2) Effect of differential tip-tilt error across 10 arc sec field.  Present data show the expected decrease in astrometric errors as the stars get brighter (K=20 -> 15), due to photon noise improvement.  However for stars brighter than K=15, the astrometric error hits a plateau and does not improve further as the stars get brighter.  This is illustrated in Figure 5.  The Galactic Center Group at UCLA has three hypotheses for the existence of this floor:  differential tip-tilt anisoplanatism across the field, differential high-order anisoplanatism across the field, and/or lack of a good enough distortion solution for the narrow camera.  At present the Galactic Center Group thinks the most likely cause is differential tip-tilt anisoplanatism; they plan to test this hypothesis by further analysis of existing data.

3) In principle chromatic and/or achromatic atmospheric refraction could be adversely affecting current accuracy.  These effects will also be analyzed further.

Figure 5

Left panel: positional uncertainty vs. stellar magnitude for stars near Galactic Center.  Pixel scale is 0.01 arc sec/px.  Right panel: average values of positional uncertainties for two different data sets.  The positional uncertainty of the “floor” changes from about 1 mas to 0.25 mas between the two data sets shown.

### AO requirements

Astrometry:

Astrometric precision		100 micro arc sec or better

Wavefront error 170 nm or better

Tip-tilt correction	IR tip-tilt needed; due to very strong reddening in Galactic Center, available J-band stars will be fainter than at H or K band, so either H or K is desirable for tip-tilt sensors. The total astrometric precision should be better than 0.1 mas. Therefore the required contribution from differential tip-tilt must be smaller than this. Further investigation into the astrometric error budget is required to determine the exact requirement.

Photometric precision		na

Polarimetric precision		na

Backgrounds			na (confusion dominated)

### General comments on astrometric accuracy in an AO system with multiple deformable mirrors

One of the point design concepts for Keck NGAO specifies a large-stroke deformable mirror within the main optical relay, plus a high-order MEMS deformable mirror either located on-axis or in multiple deployable IFU arms.  Any AO system with multiple DMs must consider the impacts on astrometric accuracy.  The following is a quote from the TMT Science-Driven Requirements Document that seems relevant to Keck NGAO design as well (this should be regarded as a place-holder for future NGAO-specific analysis of the same topic):

“An astrometric MCAO system must constrain Zernike modes 4-6 using either a single natural guide star (NGS) which is bright enough to sense defocus and astigmatism or provide two additional tip-tilt stars, making their total number 3. The differential tilts between the three tip-tilt stars constrain these modes. This requirement occurs because the tip and tilt of laser guide stars (LGS) are undetermined. As a consequence, the information brought by them is insufficient for a full solution of the tomographic problem. In addition to tip and tilt, differential astigmatism and defocus between the two DMs is unconstrained. These three unconstrained modes do not influence on-axis image quality, but produce differential tilt between the different parts of the field of view.

If multiple tip-tilt sensors are used, the MCAO system must provide for a facility to align them. If the tip-tilt sensors for the three NGSs are misplaced, the MCAO system will compensate these errors in the closed loop, hence the field will be distorted. For example, the plate scale will change if the upper DM has a static defocus. Calibration procedures must be applied to ensure that these errors do not compromise the astrometric performance of an MCAO system (e.g., flattening of the upper DM before closing the loop).

The limitations on astrometric accuracy imposed by the atmosphere are discussed in detail in TMT technical report #XX (Graham 2003)”.

### Radial Velocity

IFU with 20 or 35 mas slitlets/spaxels

The required radial velocity accuracy is 10 km/s which is a factor of 2 improvement over current observations with OSIRIS-LGSAO. Current accuracy is limited by:

1) Signal-to-noise:

This will be improved by higher Strehl ratios.

2) Differential atmospheric refraction (chromatic):

Should be compensated for by an infrared atmospheric dispersion corrector.

3) PSF estimation:

Need to investigate how to improve PSF estimation for fields without good 	PSF stars.

4) Local background subtraction (diffuse Brg gas over the entire field):

Higher Strehls will yield sky estimates that are less contaminated by the 	halos of bright stars.

5) Spectral resolution (many lines are blends):

Brg (2.166 microns) and He (2.112 microns) lines are blends at R~4000. Higher spectral resolution would resolve the individual lines. Further investigation of the ideal spectral resolution is needed.  In particular, if the NGAO system allows IFU spectroscopy of fainter stars, one may be able to obtain radial velocities from unblended spectral lines other than Brg (2.166 microns) and He (2.112 microns).

### Instrument requirements

Essential: High contrast near-IR imager with excellent astrometric performance (better than 0.1 mas).

Essential:  Infrared integral field spectrometer, R  3000 that can achieve 10 km/sec radial velocity accuracy for stars near the Galactic Center.

Desirable but not absolutely essential: High resolution (R~15000) IFU spectroscopy. With this spectral resolution, radial velocity accuracies are improved to ~1 km/s and the radial velocity measurements may themselves constrain General Relativistic effects.

Imager:

Field of view: 		at least 10 x10 arc sec

Field of regard: 	IR tip-tilt stars available 1-20’’ from imaging field center. 					Tip-tilt pickoff is required to be able to deal with multiple tip-				tilt stars separated by only a few arcseconds.

IFU multiplicity: 	one is sufficient

Wavelength coverage: K-band

IFU Spectrometer:

Field of view: 			at least 1 x 1 arc sec (more is desirable but not essential)

Field of regard: 		as needed to meet tip-tilt correction requirements

IFU multiplicity: 		one is sufficient

Wavelength coverage: 	K-band

Spectral resolution: 	(in addition, optional R~15,000)

Type and depth of required data pipeline: IFU pipeline for wavelength/flux calibration

### Summary of Requirements

The requirements for the Measurement of General Relativity Effects in the Galactic Center science case on both precision astrometry and radial velocities are summarized in the following two tables, respectively.

Table 10a. General relativity effects in the Galactic Center derived requirements

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 10a.1 | Astrometric accuracy ≤ 100 µas for objects ≤ 5” from the Galactic Center [SCRD §2.3.8, RollUp_v1 I6] | High Strehl to reduce confusion limit: rms wavefront error ≤ 170 nm. IR tip/tilt sensors. Lower background than current AO system (not quantified how much lower) |  |
| 10a.2 | Observing wavelengths K-band [SCRD §2.3.11, RollUp_v1 B6] | Transmit K band to science instrument |  |
| 10a.3 | Spatial resolution ≤ Nyquist at H and K Not called out. Perhaps derived from #6.1 |  |  |
| 10a.4 | Spatial resolution stable over field of view to 100 mas [SCRD §2.3.9] | Means of aligning tip/tilt sensors so that their mean reference position distorts the field by less than 100 mas in closed loop operation. Means of preventing WFS-blind field-distortion modes (if multi-DMs are in series). Error budgets are needed for these. |  |
| 10a.5 | Field of view ≥ 10” x 10” for imaging [SCRD §2.3.11] | Science path shall allow an unvignetted 10” x 10” field. Note: there is no specification on allowable isoplanatic error |  |
| 10a.6 | Target drift should be ≤ TBD mas Not sure what this means. See #6.4. |  |  |
| 10a.7 | Ability to construct 40x40” mosaic to tie to radio astrometric reference frame [SCRD §2.3.5] To what accuracy? |  |  |
| 10a.8 | The following observing preparation tools are required: not specified |  |  |
| 10a.9 | The following data products are required: Not specified |  |  |

Table 10b. Radial velocity measurements derived requirements

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 10b.1 | Radial velocity accuracy ≤ 10 km/sec for objects ≤ 5” from the Galactic Center [SCRD §2.3.10, RollUp_v1 J6] | High Strehl, low background, differential atmospheric refraction correction, PSF estimation [suggestions from SCRD §2.3.10] (need to derive required accuracies. Need to make an error budget for ADC accuracy across band) | Spectral resolution ≥ 4000 |
| 10b.2 | Observing wavelengths K-band [SCRD §2.3.6] |  |  |
| 10b.3 | Spatial resolution ≤ 20 or 35 mas [SCRD §2.3.6] |  | 20 and 30 mas plate scales, R=4000 IFU spectrometer with… |
| 10b.4 | Field of view ≥ 1” x 1” [SCRD §2.3.6] |  | …1” x 1” spatial field of view |
| 10b.5 | Target drift should be ≤ TBD mas Not sure what this means. See #6.4. |  |  |
| 10b.6 | The following observing preparation tools are required: not specified |  |  |
| 10b.7 | The following data products are required: IFU pipeline for wavelength/flux calibration [SCRD §2.3.6] |  |  |

### References

Broderick, A. E., & Loeb, A. 2005, MNRAS, 363, 353

Diolaiti, E., 0. Bendinelli, D. Bonaccini et al. 2000, A&AS 147, 335

Eisenhauer, F., Schodel, R., Genzel, R., Ott, T., Tecza, M., Abuter, R., Eckart, A., & Alexander, T. 2003, ApJL, 597, L121

Genzel, R., Schodel, R., Ott, T., Eckart, A., Alexander, T., Lacombe, F., Rouan, D., & Aschenbach, B. 2003, Nature, 425, 934

Ghez, A. M., Salim, S., Hornstein, S. D., Tanner, A., Lu, J. R., Morris, M., Becklin, E. E., & Duchene, G. 2005, ApJ, 620, 744

Ghez, A. M., et al. 2004, ApJL, 601, L159

Ghez, A. M., et al. 2003, ApJL, 586, L127

Gondolo, P., & Silk, J. 1999, Physical Review Letters, 83, 1719

Miralda-Escude, J., & Gould, A. 2000, ApJ, 545, 847

Morris, M. 1993, ApJ, 408, 496

Olling, R. P., & Merrifield, M. R. 2000, MNRAS, 311, 361

Peebles, P. J. E. 1972, ApJ, 178, 371

Salim, S., & Gould, A.1999, ApJ, 523, 633

Schodel, R., Ott, T., Genzel, R., Eckart, A., Mouawad, N., & Alexander, T. 2003, ApJ, 596, 1015

Schodel, R., et al. 2002, Nature, 419, 694

Shen, Z.-Q., Lo, K. Y., Liang, M.-C., Ho, P. T. P., & Zhao, J.-H. 2005, Nature, 438, 62

Weinberg, N. N., Milosavljevic, M., & Ghez, A. M. 2005, ApJ, 622, 878

## Resolved Stellar Populations in Crowded Fields

Text and tables not yet written.

Table 11.  Resolved Stellar Populations in Crowded Fields derived requirements

## Nearby Active Galactic Nuclei

The requirements for the Nearby AGN science case are summarized in the following table (see the Nearby AGN section of KAON 455 (V1)).  The typical AGN that we are considering is at redshift <0.05, and if a Seyfert 1 galaxy has a magnitude yyy point source in the center, with a host galaxy of magnitude zzz per square arcsec.

The region of interest for spatially resolved spectroscopy is within the gravitational sphere of influence of the central black hole: generally we will need at least two resolution elements across this distance.  [convert to arcsecs as function of z]

The scientific goals are the following: to measure the black hole mass using stellar kinematics in the cores of AGNs.  In order to accomplish this, PSF subtraction will be crucial for Seyfert 1 galaxies.  [This is a derived requirement.]

Table 12. Nearby AGNs derived requirements

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 12.1 | Number of targets required: sample size of TBD galaxies in TBD nights or years | Requirement on sky coverage fraction may be implied here |  |
| 12.2 | Required wavelength range 0.85 – 2.4 microns |  |  |
| 12.3 | Required spatial sampling at least two resolution elements across gravitational sphere of influence | 80% enclosed energy radius < ½ gravitational sphere of influence. This implies a total wavefront error of not more than TBD nm over a range from TBD to TBD microns. | Spectral and imaging pixels/spaxels < ½ gravitational sphere of influence (in the spatial dimension) |
| 12.4 | Required field of view for both spectroscopy and imaging > 10 radii of the gravitational sphere of influence [fill this in] |  | Will need to get sky background measurement as efficiently as possible. For IR, consider using d-IFU on the sky; for visible, need solution |
| 12.5 | Required SNR for spatially resolved spectroscopy of the central black hole region using stellar velocities > 30 per resolution element | AO Strehl ratio > TBD at 0.85 microns (Ca infrared triplet). This implies a total wavefront error of TBD nm at 0.85 microns. PSF stability and knowledge, temporal and field of view [uniformity trade]; | Spectral resolution R ~ 3000-4000 (TBD) with two pixels per resolution element; detector limited SNR performance; Spatial sampling at least two resolution elements across the gravitational sphere of influence |
| 12.6 | Required signal to noise ratio for imaging of the region around the central black hole [is this a contrast requirement?] | AO Strehl ratio > TBD, this implies a total wavefront error of not more than TBD nm over a range from TBD to TBD microns. PSF stability and knowledge, temporal and field of view [uniformity trade]; | Spatial sampling at least two resolution elements across the gravitational sphere of influence |
| 12.7 | Photometric accuracy required for imaging the central point source and possible cusp: TBD at TBD wavelengths | AO Strehl ratio > TBD, this implies a total wavefront error of not more than TBD nm over a range from TBD to TBD microns. AO background level < TBD; over TBD wavelength range; PSF stability and knowledge, temporal and field of view [uniformity trade]; | Calibration stability and accuracy, Zero-point stability and knowledge, Quality of flat-fielding; |
| 12.8 | Velocity determined to ≤ TBD km/sec for spatial resolutions of TBD mas | PSF intensity distribution known to ≤ xxx% per spectral channel. Spatial and spectral model fitting valid to ≤ TBD |  |
| 12.9 | Required observation planning tools (e.g. guide stars); PSF simulation tools to plan for observations of Seyfert 1 galaxies which have strong central point sources |  |  |
| 12.10 | Required data reduction pipeline for IFU |  |  |

## QSO Host Galaxies

The requirements for the QSO Host Galaxy science case are summarized in the following table (see the QSO Host Galaxy section of KAON 455 (V1)).  The typical QSO that we are considering is at redshift xxx, had a magnitude yyy point source in the center, with a host galaxy of magnitude zzz per square arc sec.  The galaxy is TBD arc sec across.  [Say something about the profile of the galaxy]  The scientific goals are the following: 1) measure colors and magnitudes for the point source; 2) measure morphology and surface brightness profile for the galaxy; 3) obtain spectrum of point source; 4) obtain spatially resolved spectrum of galaxy in order to study its kinematics and stellar populations.  In order to accomplish these things, PSF subtraction will be crucial.  [This is a derived requirement.]

Table 13. QSO Host galaxies derived requirements

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 13.1 | Number of targets required: sample size of TBD galaxies in TBD nights or years | Requirement on sky coverage fraction may be implied here, particularly if data from space missions or radio surveys is required |  |
| 13.2 | Required signal to noise ratio for imaging of central point source: TBD at TBD wavelength | AO Strehl ratio > TBD AO background level < TBD; over TBD wavelength range | Instrument background level; Need to be background limited (implications for detector) |
| 13.3 | Photometric accuracy required for imaging the central point source: TBD at TBD wavelengths. | PSF stability and knowledge, temporal and field of view [uniformity trade]; Target drift should be ≤ TBD mas per hour | Calibration stability and accuracy, Zero-point stability and knowledge, Quality of flat-fielding |
| 13.4 | Required SNR for spatially resolved spectroscopy of the point source > TBD at TBD wavelengths | AO Strehl ratio > TBD AO background level < TBD; over TBD wavelength range; PSF stability and knowledge, temporal and field of view [uniformity trade]; Target drift should be ≤ TBD mas per hour | [What spatial sampling is optimum for good PSF subtraction?], spectral resolution R ~ TBD; detector limited SNR performance; FOV = TBD arc sec [e.g. larger for sky subtraction] |
| 13.5 | Required signal to noise ratio for imaging of host galaxy in presence of point source: TBD at TBD wavelength at TBD distance from the point source | AO Strehl ratio > TBD AO background level < TBD; over TBD wavelength range; PSF stability and knowledge, temporal and field of view [uniformity trade]; | Spatial sampling TBD |
| 13.6 | Photometric accuracy required for imaging the host galaxy: TBD at TBD wavelengths at TBD distance from point source. | AO Strehl ratio > TBD AO background level < TBD; over TBD wavelength range; PSF stability and knowledge, temporal and field of view [uniformity trade];Target drift should be ≤ TBD mas per hour | Calibration stability and accuracy, Zero-point stability and knowledge, Quality of flat-fielding; FOV = TBD arc sec [e.g. larger for sky subtraction] |
| 13.7 | Required SNR for spatially resolved spectroscopy of the host galaxy > TBD; spatial resolution = TBD; velocity resolution and accuracy = TBD; wavelength range TBD | AO Strehl ratio > TBD AO background level < TBD; over TBD wavelength range; PSF stability and knowledge, temporal and field of view [uniformity trade]; AO 80% enclosed energy radius = TBD; Target drift should be ≤ TBD mas per hour | FOV = TBD; spectral resolution = TBD; spatial sampling (spaxel size); detector-limited performance; Calibration stability and accuracy, Zero-point stability and knowledge, Quality of flat-fielding; Requirement on persistence? Required minimum trade between single-shot spectral coverage and field of view |
| 13.8 | Required observation planning tools (e.g. guide stars); PSF simulation tools to plan for whether PSF subtraction will be good enough to see the host galaxy |  |  |
| 13.9 | Required data reduction pipeline for IFU |  |  |

## Galaxy Assembly and Star Formation History

Authors: D. Law. C. Steidel, J. Larkin

Editor: Claire Max

### Introduction

Within the last decade the near infrared has become crucial for understanding the early universe and the evolution of galaxies.  At redshifts above 1, galaxies have shrunk to angular sizes of approximately 1 arc sec making seeing based observations almost useless at uncovering morphologies and internal kinematics.  At the epoch of greatest star formation and AGN activity around a redshift of 2.5, the traditional optical lines of H, OIII and OII are nicely shifted into the K, H and J bands respectively.  The combination of the Keck LGS AO system with OSIRIS spatially resolved infrared spectroscopy is just now starting to dissect some of the brightest galaxies at this epoch.  But with the factor of ~10 in sensitivity gain possible with Keck NGAO, a wealth of science topics can be addressed.  These include the relationship between AGN and their host galaxies: radio galaxies and quasars have very strong emission lines and complex kinematics.  In more “normal” galaxies, the redshift range from 1.5 to 2.5 is the key era for the birth of their first stars and the formation of the major architectural components, namely the bulge and disk.  Measuring the morphology of star formation, the kinematics of proto-disks, the internal velocity dispersions and metallicity gradients (from things like the NII/H ratio) will allow us to witness the birth of galaxies like the Milky Way.

Table 4 gives a sample of which lines are available as a function of redshift.

Table 4

| Redshift | J band | H band | K band |
| --- | --- | --- | --- |
| ~ 1.2 | H and NII |  |  |
| ~ 1.5 | OIII | H and NII |  |
| ~ 2.5 | OII | OIII | H and NII |
| ~ 3.2 |  | OII | OIII |
| ~ 4.1 |  |  | OII |

Because JWST is optimized for faint-object IR spectroscopy and imaging, for this science case we will have to seek specific “sweet spots” in which Keck NGAO can make a significant contribution in the age of JWST.

Here we address science requirements flowing from one of these redshift ranges: 2 ≤ z ≤  3.

### Scientific background and context: galaxies at 2 ≤ z ≤  3

At high redshifts z ~ 2-3 galaxies are thought to have accumulated the majority of their stellar mass (Dickinson et al. 2003), the rate of major galaxies mergers appears to peak (Conselice et al. 2003), and instantaneous star formation rates and stellar masses range over two decades in value (Erb et al. 2006).  Given the major activity at these redshifts transforming irregular galaxies into the familiar Hubble sequence of the local universe, it is of strong interest to study these galaxies in an attempt to understand the overall process of galaxy formation and the buildup of structure in the universe.

The global properties of these galaxies have recently received considerable attention, and the star formation rate, stellar mass, gaseous outflow properties, etc. have been studied in detail (e.g. Steidel et al. 2004, Papovich et al. 2006, Reddy et al. 2006 and references therein).  Beyond these global properties however, little is known about their internal kinematics or small-scale structure, particularly with regard to their mode of dynamical support or distribution of star formation.  Previous observations with slit-type spectrographs (e.g. Erb et al. 2004, Weiner et al. 2006) and seeing-limited integral field spectrographs (Flores et al. 2006) suggest that kinematics are frequently inconsistent with simple equilibrium disk models.  However these studies are too severely constrained by slit misalignment, spatial resolution, and the size of the atmospheric seeing halo relative to the size of the typical sources (less than one arcsecond) to obtain conclusive evidence.  It is therefore unknown whether the majority of star formation during this epoch is due to rapid nuclear starbursts driven by major merging of gas-rich protogalactic fragments, circumnuclear starbursts caused by bar-mode or other gravitational instabilities, or piecemeal consumption of gas reservoirs by overdense star forming regions in stable rotationally-supported structures.

Here we investigate the general capabilities of Keck NGAO for the study of these high-redshift galaxies, via simulations of the integral field spectrographs used to dissect these galaxies and to study their kinematics and chemical composition.

### Scientific goals

The study of high-redshift galaxies is a powerful driver for multiplexed observations, for example via deployable integral field unit (IFU) spectrographs.  Given the areal densities of 1 to 10 targets per square arcminute on the sky (depending on the target selection criteria, Table 5), multiplexing multi-conjugate or multi-object adaptive optics (MCAO/MOAO) systems would be capable of simultaneously observing ~ 10 targets within a several square arcminute field, permitting the compilation of a large representative sample with a minimum of observing time.  In order to take best advantage of the high areal densities of targets, it is desirable to be able to deploy of order 6-12 IFUs over a ~ 5 square arcminute field of view.

Table 5

Space Densities of Various Categories of Extragalactic Targets.

| Type of Object | Approx density per square arc minute | Reference |
| --- | --- | --- |
| SCUBA sub-mm galaxies to 8 mJy | 0.1 | Scott et al. 2002 |
| Old and red galaxies with 0.85 < z < 2.5 and R < 24.5 | 2 | Yamada et al. 2005; van Dokkum et al. 2006 |
| Field galaxies w/ emission lines in JHK windows 0.8 < z < 2.6 & R < 25 | > 25 | Steidel et al 2004; Coil et al 2004 |
| Center of distant rich cluster of galaxies at z > 0.8 | > 20 | van Dokkum et al 2000 |
| All galaxies K < 23 | > 40 | Minowa et al 2005 |

Such observations would permit the study of the chemical composition and distribution of star formation within the target galaxies (e.g. through mapping the measured [N II]/Ha ratios), in addition to mapping the velocity fields of the galaxies.  Velocity data will enable us to detect AGN through chemical signatures and broadening of nuclear emission lines, to differentiate chaotic major mergers from starbursting galaxies in dynamical equilibrium, to determine the location of major star forming regions within any such rotationally supported systems, and to distinguish between chaotic and regular velocity fields to help ascertain whether observed star formation is commonly a consequence of major tidal interaction as predicted in current theories of galaxy formation.

With current-generation instruments, it is extremely challenging to observe a representative sample of sources due to the uncertainties inherent in long-slit spectroscopy (i.e. slit misalignment with kinematic axes), seeing-limited integral field spectroscopy (i.e. loss of information on scales smaller than the seeing disk), or a single-object IFU with current-generation adaptive optics (for which integration times are prohibitive for obtaining a large sample).  A high-Strehl NGAO system with multi-object IFU capability would represent a major advance towards obtaining reliable kinematic and chemical data for a large sample of high redshift galaxies which could be productively integrated with the known global galaxy properties to further our understanding of galaxy formation in the early universe.

### Proposed observations and targets

At redshifts z = 0.5-3, major rest-frame optical emission lines such as H, [N II], and [O III] fall in the observed frame near-IR, and in order to study the evolution of galaxies across this range of cosmic times it is important to have wavelength coverage extending from 1 to 2.5 microns.  H line emission from the well-studied redshift z ~ 2-3 galaxy sample falls in the K band, emphasizing the importance of optimizing observations at these wavelengths by reducing backgrounds and inccreasing throughput as much as possible.

Typical observing strategy would entail simultaneous observation of approximately 10 high-redshift galaxies in a given field using a dithered set of exposures designed to move each object around on the detectors permitting maximum on-source integration time whilst simultaneously measuring accurate background statistics for sky subtraction.  Based on the numerical simulations of Law et al. (2006) and the observed performance of the OSIRIS spectrograph, we anticipate that typical observations (assuming a K-band Strehl of roughly 60-70% from the NGAO system) would last approximately 1-2 hours per set of targets (for bright star-forming galaxies at redshift z ~ 2) permitting a sample of approximately 50 targets in a given night of dedicating observing.

### AO and instrument requirements

#### AO requirements

Using the Gemini model of the Mauna Kea near-IR sky background coupled with a mathematical model of the thermal contributions from warm optical surfaces in the light path, Law et al. (2006) have demonstrated that the current K-band performance of AO-fed instruments is limited primarily by thermal emission from the warm AO system (which constitutes the majority of the total interline K band background).  It is therefore a priority to reduce this emission to a lower fraction of the intrinsic background from the night sky and thermal radiation from the telescope itself.  Using a combination of high-throughput optical components and AO system cooling, ideally we would like the thermal radiation from the AO system contribute less than 10-20% to the total K-band background.  In Figure 6 we plot the AO cooling required (according to the Law et al. 2006 models) as a function of the throughput to achieve this goal.

Figure 6 Required AO system temperature as a function of AO throughput to minimize impact of thermal radiation on the net K-band background.

Sky subtraction is key to obtaining quantitative spectral information.  There are two ways to achieve this: 1) Given the typical size of the target galaxies (less than or of order an arcsecond), the field of view of each IFU could be suitably large to permit accurate sky subtraction (via on-IFU dithering) while sampling the target on the smallest scales permitted by detector noise characteristics.  In this approach, each IFU should have a field of view measuring at least 3 x 1 arcseconds in order to avoid costly dedicated sky exposures.  2) The deployable integral field spectrgraph could include one or more arms that would be dedicated to taking spectra of the sky at the same time that galaxy spectra are being obtained on the other IFU arms.  In this case each unit would only need to subtend 1 x 1 arcsecond on the sky.  In the process of designing the deployable IFU spectrograph, there needs to be a trade study to evaluate and compare these two approaches to sky subtraction, as well as any other concepts that appear to be viable.  It is not yet clear to us that option 2), using separate IFU heads in order to measure sky backgrounds, will yield accurate enough sky measurements.

The individual IFU units should be sampled on scales of order the diffraction limit (~ 50 mas) to permit accurate characterization of the structure on small scales without introducing excessive instrumental contribution to the total noise budget.  Spectral resolution should be greater than R~3000 in order to effectively resolve out OH sky features and distinguish Ha from [N II] emission.

Specifying AO requirements such as spatial resolution and encircled energy is not straightforward for the high-z galaxy science case.  This is because the width of the core of the PSF will be limited by the availability of adequately bright tip-tilt stars.  One can obtain excellent spatial resolution and encircled energy over that small fraction of the sky where excellent tip-tilt stars are available, or more modest spatial resolution and encircled energy over a larger fraction of the sky where tip-tilt stars are dimmer and/or farther away.  AO requirements that deal with resolution alone are less useful than those that can be phrased in terms of “spatial resolution of xx achieved over a sky coverage fraction larger than yy”.  As we have not yet worked through this type of specification, we present the resolution and encircled energy requirements in terms of the desired IFU spaxel size, 50 mas.

AO Requirements:				Near-IR

Spaxel size (IFU spectroscopy)		50 mas

Field of view of one IFU unit

Two options				1) 3 x 1 arc sec or greater

(object size ~ 1”, but need 3” in at least

one dimension to get good sky meas’t)

2) one or more dedicated IFU units for 							sky measurement. with each unit 1 x 1”

Backgrounds					Goal: < 10-20% above telescope + sky

Field of regard					As large as needed to get tip-tilt stars

Is a contiguous field required?		no

Encircled energy				50 mas with optimal tip-tilt stars

Sky coverage fraction				at least 30% with encircled energy

radius < 75 mas

(Photometric accuracy)

(Astrometric accuracy)

(Polarimetry)

(Contrast sensitivity)

#### Instrument Requirements:

Field of view (spectroscopy)			3 x 1 arcsec or greater if sky subtraction 							is to be done within each IFU head.  If 							there are separate IFU heads dedicated 							to sky subtraction, then field of view of 							1 x 1 arcsec is adequate.

Field of regard					As large as needed for good tip-tilt

IFU or imager multiplicity			6-12

Wavelength coverage				JHK

Spectral resolution				3000 - 4000

Data reduction pipeline			Required

Other considerations	Atmospheric dispersion: may be able to avoid a dispersion corrector through appropriate data reduction pipeline software.  The performance needs to be compared with that of a “real” ADC.

### Summary of Requirements

The requirements for the high-z galaxy science case are summarized in the following table.

Table 14. High-Redshift Galaxies derived requirements

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 14.1 | Sensitivity. SNR ≥ 10 for a z = 2.6 galaxy in an integration time ≤ 3 hours for a spectral resolution R = 3500 with a spatial resolution of 50 mas [SCRD §2.4.4] This is not explicitly called out like this in the SCRD, but is reasonably consistent with it. The SNR of 10 is not called out. | Sufficiently high throughput and low emissivity of the AO system science path to achieve this sensitivity. Background due to emissivity less than 20% of sky + telescope. [SCRD §2.4.5.1 and SCRD Figure 6] |  |
| 14.2 | Target sample size of ≥ 200 galaxies in ≤ 3 years (assuming a target density of 4 galaxies per square arcmin) [SCRD §2.4.3] Again, not explicitly stated in this manner, but consistent with 10 targets / night over 20 observing nights. | Multi-object AO system: one DM per arm, or an upstream MCAO system correcting the entire field of regard. 6-12 arms on 5 square arc minutes patrol field. Minimum Spacing: TBD | Multiple (6-12) IFUs, deployable on the 5 square arc minute field of regard |
| 14.3 | Observing wavelengths = J, H and K (to 2.4 µm) What portions of these bands must be detected in a given single observation? [SCRD §2.4.5.2, RollUp_v1 B11] | AO system must transmit J, H, and K bands | Infrared IFUs designed for J, H, and K. |
| 14.4 | Spectral resolution = 3000 to 4000 [SCRD §2.4.5.2] |  | Spectral resolution of >3000 in IFUs |
| 14.5 | Spatial resolution 50 mas [SCRD §2.4.5.1] RollUp_v1 refers to 70mas when stating encircled energy spec. Is this consistent? | See encircled energy spec. #9.5 | 50 mas IFU spaxal scale |
| 14.6 | Encircled energy 50% in 70 mas [RollUp_v1 E11, N11] Note SCRD §2.4.5.1 seems garbled. (50% instead of 50mas (typo?)? What is meant by optimal tip/tilt stars?) Is this consistent with #9.5. Is there an EE spec for 50 mas? | Wavefront error sufficiently low (~170 nm) to achieve the stated requirement in J, H, and K bands. |  |
| 14.7 | Velocity determined to ≤ TBD km/sec for spatial resolutions of 50 to 70 mas (not specified in SCRD) | PSF intensity distribution known to ≤ 10% per spectral channel. Spatial and spectral model fitting valid to ≤ TBD (how did this get derived?) |  |
| 14.8 | Field of view ≥ 1” x 3” [SCRD §2.4.5.1] | Each MOAO channel passes a 1”x3” field plus a seeing disk diameter (~1”), assuming uncorrected field is at multiobject pickoff plane. Can be tightened appropriately if plane is partially or fully AO corrected. | Each IFU unit’s field of view is 1” x 3” |
| 14.8opt | Field of view ≥ 1” x 1” [SCRD §2.4.5.1] | Each MOAO channel passes a 1”x1” field plus a seeing disk diameter (~1”), assuming uncorrected field is at multiobject pickoff plane. Can be tightened appropriately if plane is partially or fully AO corrected. An additional MOAO channel is dedicated to sky observations. | Each IFU unit’s field of view is 1” x 1” One extra IFU for dedicated sky observations |
| 14.9 | Simultaneous sky background measurements within a radius of 3” with the same field of view as the science field | See choice of #9.8 or #9.8opt above. |  |
| 14.10 | Relative photometry to ≤ 5% for observations during a single night [RollUp_v1 H11] (left blank in SCRD) | Knowledge of ensquared energy in IFU spaxel to 5%. Telemetry system that monitors tip/tilt star Strehl and other real-time data to estimate the EE vs time. |  |
| 14.11 | Absolute photometry ≤ 0.3 mag (can’t find this requirement) |  |  |
| 14.12 | Sky coverage ≥30% to overlap with data sets from other instruments and telescopes [SCRD §2.4.5.3, RollUp_v1 N11] | Infrared tip/tilt sensors with AO correction of tip/tilt stars (specify order of correction?) |  |
| 14.13 | Should be able to center a galaxy to ≤ TBD of science field of view |  |  |
| 14.14 | Should know the relative position of the galaxy to ≤ TBD of spaxel size |  |  |
| 14.15 | Target drift should be ≤ TBD of spaxel size |  |  |
| 14.16 | The following observing preparation tools are required: TBD |  |  |
| 14.17 | The following data products are required: calibrated spectral data cube in World Coordinate System with absolute TBD accuracy |  |  |

### References

Coil, A. L. et al. 2004, ApJ, 617, 765

Conselice. C. et al. 2003, AJ, 126, 1183

Dickinson, M. et al. 2003, ApJ, 587, 25

Erb, D. K. et al. 2004, ApJ, 612, 122

Erb, D. K. et al. 2006, ApJ, 647, 128

Flores, H. et al. 2006, A&A, 455, 107

Law, D. et al.  2006, AJ, 131, 70

Minowa, Y. et al. 2005, ApJ, 629, 29

Papovich, C. et al. 2006, ApJ, 640, 92

Reddy et al. 2006, ApJ, 644, 792

Scott, S. E. et al. 2002, MNRAS, 331, 817

Steidel, C. C. et al. 2004, ApJ, 604, 534

van Dokkum, P. G. et al 2000, ApJ, 541, 95

van Dokkum, P. G. et al. 2006, ApJ, 638, L59

Weiner, B. J. et al. 2006, ApJ, 653, 1027 and ApJ, 653, 1049

Yamada et al. 2005

## Gravitational Lensing

The requirements for the gravitational lensing science case are summarized in the following two tables.

Spectroscopic studies of distant galaxies lensed by galaxies

Table 15a. Spectroscopic studies of distant galaxies lensed by galaxies

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 15a.1 | SNR ≥ 10 for a z = 1 – 2 galaxy in an integration time ≤ 3 hours for a Gaussian width 20 km/sec Gaussian width (50 km/sec FWHM) with a spatial resolution of 50 mas |  | R ~ 5000 (or whatever is needed to achieve 20 km/sec sigma on these targets) |
| 15a.2 | Target sample size of ≥ 50 galaxies, with density on the sky of 10 per square degree. Survey time ~ 3 years. | Number of IFUs: at least one, plus preferably one to monitor the PSF and one to monitor the sky. The extra two IFUs could be dispensed with if there were other ways to monitor the PSF and the sky background. |  |
| 15a.3 | Observing wavelengths = J, H and K (to 2.4 µm). Emphasis is on shorter wavelengths. Would use z and I bands if available. |  |  |
| 15a.4 | Spectral resolution: whatever is needed to get 20 km/sec radial velocity Gaussian sigma |  | Spectral resolution |
| 15a.5 | Spatial resolution 50 mas | [need to make the spatial resolution and the enclosed energy requirements consistent with each other] |  |
| 15a.6 | Velocity determined to ≤ 20 km/sec Gaussian sigma for spatial resolutions of 50 mas | PSF intensity distribution known to ≤ TBD% per spectral channel. |  |
| 15a.7 | Field of view > 4” diameter (lenses are bigger than galaxies). Typical lens is 3 to 4 arc sec diameter. |  |  |
| 15a.8 | Simultaneous sky background measurements |  |  |
| 15a.9 | Relative photometry to ≤ 0.1 mag for observations during a single night |  |  |
| 15a.10 | Absolute photometry ≤ 0.3 mag |  |  |
| 15a.11 | Sky coverage at least 50% with enclosed energy radius within TBD mas. Sky coverage should be better than with current LGS. |  |  |
| 15a.12 | Should be able to center a galaxy to ≤ TBD of science field of view |  |  |
| 15a.13 | Should know the relative position of the galaxy to ≤ TBD of spaxel size. (Whatever works for high z galaxy case is OK here as well) |  |  |
| 15a.14 | Target drift should be ≤ TBD of spaxel size (Whatever works for high z galaxy case is OK here as well) |  |  |
| 15a.15 | The following observing preparation tools are required: |  |  |
| 15a.16 | The following data products are required: calibrated spectral data cube |  |  |

Imaging studies of distant galaxies lensed by galaxies.

Goal: screen potential lensed-galaxy targets for more detailed and lengthy spectroscopic study.

Table 15b. Imaging studies of distant galaxies lensed by galaxies.

Goal: screen potential lensed-galaxy targets for more detailed and lengthy spectroscopic study. (there is no write-up on distant lensed galaxies in the present version of the SCRD).

| # | Science Performance Requirement | AO Derived Requirements | Instrument Requirements |
| --- | --- | --- | --- |
| 15b.1 | SNR ≥ 3 per pixel (100 per source) for a z = 1 – 2 galaxy in an integration time ≤ 1/2 hour. Strehl > 0.3 at J band. |  |  |
| 15b.2 | Target sample size of ≥ 200 galaxies, with density on the sky of 10 per square degree. Survey time ~ 3 years. | Overhead less than 10 min between targets. | 10 per square degree implies that you will only be able to observe one target at a time – average of 1 in every ~19x19’ patch. |
| 15b.3 | Observing wavelengths = J, H and K (to 2.4 µm). Emphasis is on shorter wavelengths. Could use z if available. Thermal part of K band less important. |  |  |
| 15b.4 | Spatial resolution better than 50 mas | Need a good model of the PSF or a simultaneous image of a PSF star. Need a figure of merit for goodness of the PSF: how well the model fits the “real” PSF in two dimensions. | Need Nyquist sampling of pixels at each wavelength. |
| 15b.5 | Field of view > 15” diameter for survey. Bigger is better. Some degradation between center and edge of field is tolerable. (Need to quantify.) |  |  |
| 15b.6 | Relative photometry to ≤ 0.1 mag for observations during a single night |  |  |
| 15b.7 | Absolute photometry ≤ 0.3 mag |  |  |
| 15b.8 | Sky coverage at least 50% with enclosed energy radius within 0.1 arc sec at H or K. Sky coverage should be better than with current LGS. |  |  |
| 15b.9 | Should know the relative position of the galaxy to ≤ TBD of spaxel size. (Whatever works for high z galaxy case) |  |  |
| 15b.10 | The following observing preparation tools are required: |  |  |
| 15b.11 | The following data products are required: accurate distortion map (to 1% of the size of the galaxy, or 0.01 arc sec rms) |  |  |

Notes:

May want to repeat this exercise with galaxies lensed by clusters.

Wide field: Typical size of the highly magnified region of a cluster is 1 arc min.  Need low background: lens arcs from z ~ 7 are at most Vega magnitude 23 or 24 in H (brightest arcs).  Typical size small (half light radii 0.1 arc sec).

Closer galaxies with giant arcs: deployable IFU application.  Several arc sec long.  Want field of regard of about an arc min.  Usually 3 to 5 multiple arcs within a square arc min.  (But each might be long, and require more than 1 IFU unit.)  Less than 10 IFU units needed in a square arc min.

Is a requirement needed on achieving a particular astrometric accuracy in a particular time needed for survey-type science (i.e., 1 mas in 10 min).

Aren’t encircled energy requirements needed for the IFUs?

## Other: Backup Science

This will primarily be NGS science that can be done when the lasers cannot be propagated (e.g. due to cirrus), or less-demanding examples of LGS science that can be done when the laser power available is lower than nominal due to hardware problems.  The derived requirements for Backup Science will largely involve science preparation and operations issues.

Table 16.  Other: Backup Science derived requirements

## Roll-up of AO and Instrument Requirements

# Appendix A.  Table of potential infrared tip-tilt stars close to the Galactic Center

Figure A1.  Mosaic of Galactic Center at K band, from J. Lu and A. Ghez.  The FITS file of this observation, which includes the WCS coordinate system, is available for use in correlating the image with the tables that follow.

Following pages: Tables from Blum et al. 1996, ApJ 470, 864