Keck Adaptive Optics Note 732

Keck Next Generation Adaptive Optics:

Data Server Preliminary Design

Version 1.0

Date: 7/31/2026

Authors:

Jimmy Johnson

Contributors:

Kevin Tsubota

W. M. Keck Observatory

California Association for Research in Astronomy

65-1120 Mamalahoa Highway

Kamuela, Hawaii  96743

808-885-7887

Table of Contents

1	Introduction	3

2	Glossary	3

3	Project Description	3

3.1	Clients	5

3.1.1	Viewers	5

3.1.2	NGAO Components/Controllers/Applications (via services)	5

3.1.3	Other	5

3.2	Databases	5

3.2.1	Configuration Database	5

3.2.2	Logging Database	5

3.2.3	Telemetry Database	5

3.2.4	Alarm Database	6

3.2.5	Ad-hoc Archive Database	6

4	Hardware	6

4.1	Optimal Solution	6

4.1.1	Specifications	7

4.2	Traditional solution	8

5	Software	10

5.1	Relational database	10

List of Figures

Figure 1 Data Server Overview	4

Figure 2 Data Server Hardware Architecture	9

Figure 3 Sun Fire X4140	9

Figure 4 SurfRaid Triton 14FA4	10

List of Tables

Table 1 MySql and Postgres comparison	11

# Introduction

The Data Handling System for the AO Controls has been called the “Data Server”. The NGAO Data Server is defined as a computer in the network, with associated software, that is dedicated to the data storage and retrieval needs required by the AO Controls System. It holds the database management system , the databases and is part file server capable of housing non-database files such as FITS. In addition to providing data persistence, retrieval and data management there will be a telemetry data recorder, an ad-hoc data recorder, an alarm recorder, general logger, and configuration.  High speed RTC data will be stored in its own data recording system.

# Glossary

The following terms and acronyms will be used throughout this document:

| Acronym | Definition |
| --- | --- |
| AO | Adaptive Optics |
| ATA | AT (Advance Technology) Attachment |
| ATST | Advance Technology Solar Telescope |
| CPU | Central Processing Unit |
| DRAM | Dynamic Random Access Memory |
| FITS | Flexible Image Transport System |
| GB | Gigabyte (1024^3) |
| HDD | Hard Disk Drive |
| KAON | Keck Adaptive Optics Note |
| MB | Megabyte (1024^2 Bytes) |
| NGAO | Next Generation Adaptive Optics |
| NGWFC | Next Generation Wavefront Controller |
| RAID | Redundant Array of Independent Disks |
| RAM | Random Access Memory |
| RPM | Revolutions Per Minute |
| RTC | Realtime Control |
| SATA | Serial ATA |
| SSD | Solid State Drive |
| TB | Terabyte (1024^4 Bytes) |
| TRS | Telemetry Recording System |
| ZFS | Sun Microsystems’ Z file system |

# Project Description

Figure 1 shows a high level overview of the data server. The data server hardware consists of expandable and reliable (RAID) hardware capable of supporting multiple databases and file formats.

Figure 1 Data Server Overview

The database engine is expected to be either PostgreSQL or MySql. Both are fairly comparable at this stage though some analysis should be done to compare the two in detail. As a frame of reference the telemetry server in NGWFC uses PostgreSQL as does the Advanced Technology Solar Telescope. Regardless of which database engine is used there will be a database interface layer to provide a suitable abstraction to the database clients.

Three distinct sets of clients have been identified:

- Viewers

- NGAO Components/Controllers/Applications (via services)

- Other

Five (possibly six) distinct databases have been identified:

- Configuration

- Logging

- Telemetry (polled RTC data may go here or may go to its own database)

- Alarm

- Ad-hoc archive

## Clients

### Viewers

It is expected that there will be user interfaces (viewers) to allow easy viewing of logged information, archived data, alarm history and possible telemetry. KCSF will provide viewers for logged and archived data as well as alarm history. The need for a standardized telemetry viewer is TBD.

### NGAO Components/Controllers/Applications (via services)

KCSF will provide services to all components, controllers and applications that use to framework. Many of these services interact with the data server. Each component will have the ability to retrieve its configuration from the configuration database. A configuration user interface will provide the ability to configure and re-configure the overall system. The log, archive and telemetry service will allow components to write appropriately formatted data to the corresponding service databases.

### Other

It is TBD as to what these clients will be but it is expected that applications will be developed outside of the framework that want the ability to read and write to one or more databases and to be able to store and retrieve files form the server.

## Databases

### Configuration Database

The configuration database and associated support will be provided by KCSF. The database is quite complex and is covered in detail under KAON xxxxx.

### Logging Database

The log database and associated support will be provided by KCSF. The log database is simple and will have the following schema:

| Field | Type | Description |
| --- | --- | --- |
| Time_Stamp | timestamp | UT Timestamp |
| Mode | VCHAR | Message type: e.g. Warning, Info, Error .... |
| Source | VCHAR | Who generated the message |
| Message | TEXT | The actual message |

| Field | Type | Description |
| --- | --- | --- |
| Time_Stamp | timestamp | UT Timestamp |
| Source | VCHAR | Source of the archived data |
| Name | VCHAR | Attribute/Variable name |
| Value | Text | Archived value |

### Telemetry Database

The telemetry database schema is dynamic in that each telemetry topic will have its own table with column/field information specific to that specific telemetry item.

### Alarm Database

The alarm database and associated support will be provided by KCSF. The alarm history schema is still TBD.

### Ad-hoc Archive Database

The archive database and associated support will be provided by KCSF. The archive database is simple and will have the following schema: TBD

# Hardware

Two hardware solutions are presented. The first is what is considered to be the optimal solution allowing for great performance, easy growth and a wide range of configuration options. The down side is that we are unfamiliar with the product and the potential increased initial cost. Option two is more a tried and tested solution that can do the job but is not quite a flexible or expandable. Regardless of the solution the expectation would be that the hardware will be located in the telescope computer room and will, most likely, be mounted in a standard 19” rack.

## Optimal Solution

The optimal solution is to use Suns (Oracles) Unified Storage Solution which is a family of storage solutions designed for high performance and high availability. The solution uses an optimized storage hierarchy with Hybrid Storage Pools containing DRAM, SSD, and HDD drives. To deliver high performance using cost-effective components, the Sun Storage 7000 series file system, ZFS, seamlessly optimizes access to the different types of media in the Hybrid Storage Pools. ZFS was designed to automatically recognize different I/O patterns and place data in the best storage media for optimal performance.

ZFS transparently executes writes to low-latency SSD media so that writes can be quickly acknowledged, allowing the application to continue processing. ZFS then automatically flushes the data to high-capacity drives as a background task. Another type of SSD media acts as a cache to reduce read latency, and ZFS also transparently manages the process of copying frequently accessed data into this cache to seamlessly satisfy read requests from clients.

The systems can be scaled as needed by adding additional RAM, SSD or hard disks. Scalability is available in multiple dimensions by increasing compute power, storage capacity, or performance independently. As application requirements change, it is possible to

- Increase computational power by adding more CPUs and cache

- Expand total capacity by adding enterprise-class SATA drive expansion units

- Increase performance using additional SSD to buffer storage reads or writes

The system is touted as having very low energy consumption with up to 75 percent less than the cost of competitive solutions. This is due to the reduced power consumption of SSD and HDD disk drives rather than the use of high RPM drives.

All systems come with a management interface that guides installation, configuration and tuning as well as DTrace Analytics software to identify and diagnose and issues.  Real-time analysis and monitoring is supported.

To meet a variety of needs for capacity, price, and performance, the Sun Storage 7000 series family comes in three different configurations plus a cluster configuration that offers maximum availability.  These are known as Sun Storage 7110, 7210, 7310 and 7410.

All non-clustered systems come bundled with the same software including data protocols, replication, compression, and DTrace Analytics software for system troubleshooting and performance optimization. Clustered systems include an additional software module for cluster software features.

### Specifications

| Product | Key Benefits | Availability |
| --- | --- | --- |
| Sun Storage 7110 Unified Storage System Updated! | - Enterprise-class storage features at entry-level prices - Up to 4 TB capacity, one of the most dense in its class - Easy-to-use management interface and DTrace Analytics reduce management costs - Enterprise-class data services with no additional fees | From $10,995 |
| Sun Storage 7210 Unified Storage System | - High-density unified storage solution - 46 TB in energy-efficient 4U, scales up to 142 TB - Optimize performance with DTrace Analytics for increased productivity - Hybrid Storage Pool and Enterprise Flash technology deliver breakthrough performance while reducing power, cooling, and rack space | From $34,995 |
| Sun Storage 7310 Unified Storage System Updated! | - Entry-level cluster option for high availability - Easy-to-use management interface and DTrace Analytics reduce management costs - Hybrid Storage Pool and Enterprise Flash technology deliver breakthrough performance while reducing power, cooling, and rack space - Scale up to 96 TB to grow with your business | From $40,140 |
| Sun Storage 7410 Unified Storage System Updated! | - Highest performance with highly available cluster option - Easy-to-use Dtrace Analytics increases uptime while maximizing performance - Hybrid Storage Pool and Enterprise Flash improve application response times - Scales up to 288 TB (576 TB when 2 TB drive is available) | From $73,165 |

## Traditional solution

A more traditional approach (similar to what is used for NGWFC TRS) is to use a rack-mounted server such as the Sun Fire X4140 and a RAID storage solution such as the SurfRaid Triton 16FA4 RAID controller. They will be located in the telescope computer room and will be mounted in a standard 19” rack.

This proposes two main components, a storage server and a disk array. The storage server can be linked to the rest of the system through Fibre Channel or Gbit Ethernet. The preferred way to connect the disk array to the storage server will be through Fibre Channel, with the Fibre Channel configured as a point-to-point topology.

Figure 2 Data Server Hardware Architecture

TRS uses the discontinued Sun Fire X4100 and SurfRaid Triton 16FA (2Gb Fibre Channel).  The Sun Fire x4140 and SurfRaid Triton 16FA4 are the current equivalents. For reference the performance tests carried out by Microgate achieved a sustained rate of 35.4 MB/sec on the TRS hardware.

Without a more detailed analysis of the AO Controls data handling needs these may not meet the requirements though there are other similar more performant hardware options. For example there are a complete line of servers with AMD in the Sun Fire X4x50 range and Intel in the Sun Fire X4x70 range. Likewise there are alternatives to the Triton offering 8 Gb Fibre Channel and greater storage capacity. If additional processor performance was needed a blade solution such as the Sun Blade X6270 could be used instead of the Sun Fire X4140.

The Sun Fire X4140 supports two 6 core Opteron processors (older model supported quad core) and significantly more RAM and the Triton 16FA4 supports up to 32 TB (older model supported 6 TB).

Pictures of the Sun Fire X4140 and the Triton 16FA4 are shown in Figure 3 and Figure 4 respectively. The maximum power dissipation is 650 watts for the X4140 and 460 watts for the Triton 16FA, for a total of 1100 watts maximum. This power will be dissipated in the telescope computer room, which is air conditioned.

Figure 3 Sun Fire X4140

Figure 4 SurfRaid Triton 14FA4

# Software

## Relational database

The following shows a high level comparison between MySql and Postgres.

| Feature | MySQL 5.x | PostgreSQL |
| --- | --- | --- |
| SubSelects | Yes | Yes |
| Views | Yes | Yes |
| Foreign Key relationships | Yes | Yes |
| Foreign Key constraints | Yes | Yes |
| Triggers | Yes | Yes |
| Indexing on non trivial types | Yes | Yes |
| Sequences | Some1 | Yes |
| Transactions | Yes | Yes |
| Transactional DDL statements | No | Yes |
| OO (Inheritance of tables) | No | Yes |
| Async Notifications | No | Yes |
| Constraints | Yes | Yes |
| INSERT ... SELECT | Yes | Yes |
| Stored Procedures | Yes | Yes |
| Row level locking | Yes2 | Yes |
| Table level locking | Yes | Yes |
| Geospatial Datatypes (ie WKT) | Yes | No3 |
| Error! Hyperlink reference not valid. | No | Yes4 |
| Native replication (M/M, M/S, Ring) | Yes | No5 |

Table 1 MySql and Postgres comparison

In general the performance of the two databases is now comparable. Each has unique areas that they are stronger in. Postgres has better transaction support, MySql has better large text area support.