Keck Adaptive Optics Note 456

Next Generation Adaptive Optics:

System Requirements Document

Version 0.3

February 1, 2007

1	Introduction	1

2	Scope and Applicability	1

3	References	2

3.1	Related Documents	2

3.2	Referenced Drawings	2

4	Revision History	2

5	Background	3

5.1	Purpose	3

5.2	Motivation for the Development of NGAO	3

5.3	Overview	3

6	Overall Requirements	6

6.1	Science Requirements	6

6.1.1	Purpose and Objectives	6

6.1.2	Performance Requirements	6

6.1.3	Science Instrument Requirements	7

6.2	Observatory Overall Requirements	9

6.2.1	Purpose and Objectives	9

6.2.2	Facility Requirements	9

6.2.3	Science Instrument Requirements	10

6.2.4	Operational Requirements	11

7	Optical Requirements	13

7.1	Purpose and Objectives	13

7.2	Performance Requirements	13

7.2.1	Parametric Performance Requirements	15

7.2.2	Operational Performance Requirements	15

7.3	Implementation Requirements	15

7.4	Design Requirements	15

8	Mechanical Requirements	16

8.1	Purpose and Objectives	16

8.2	Performance Requirements	16

8.2.1	Parametric Performance Requirements	16

8.2.2	Operational Performance Requirements	16

8.3	Implementation Requirements	16

9	Electronic/Electrical Requirements	17

9.1	Purpose and Objectives	17

9.2	Performance Requirements	17

9.2.1	Parametric Performance Requirements	17

9.2.1.1	Electrical Power and Thermal Dissipation	17

9.3	Implementation Requirements	17

9.3.1	Feature Implementation Requirements	17

9.4	Design Requirements	17

9.4.1	Technological Design Requirements	17

9.4.2	Integration Related Design Requirements	17

10	Safety Requirements	18

10.1	Purpose and Objectives	18

10.2	Scope	18

10.3	Laser Safety Requirements	18

10.4	Laser Projection Safety Requirements	18

10.4.1	Aircraft Safety	18

10.4.2	Space Command	18

11	Software Requirements	19

11.1	Purpose and Objectives	19

11.2	Scope	19

11.3	Performance Requirements	19

11.3.1	Parametric Performance Requirements	19

11.3.2	Operational Performance Requirements	19

11.3.2.1	Overhead	19

11.3.2.2	Error Recovery	19

11.4	Implementation Requirements	19

11.4.1	Feature Implementation Requirements	19

11.4.1.1	User interfaces	19

11.4.1.2	Data Reduction Pipeline	19

11.4.1.3	Target Computers	20

11.5	Design Requirements	20

11.5.1	Technological Design Requirements	20

11.5.2	Integration Related Design Requirements	20

12	Interface Requirements	21

12.1	Purpose and Objectives	21

12.2	Performance Requirements	21

12.2.1	Parametric Performance Requirements	21

12.2.2	Operational Performance Requirements	21

12.3	Implementation Requirements	21

12.3.1	Optical Requirements	21

12.3.2	Mechanical	21

12.3.3	Electrical	21

12.3.4	Software	22

12.4	Design Requirements	22

12.4.1	Technological Design Requirements	22

12.4.2	Integration Related Design Requirements	22

12.4.2.1	Optical Interface	23

12.4.2.2	Mechanical Interface	23

12.4.2.3	Electrical/Electronic Interface	23

13	Reliability Requirements	24

13.1	Purpose	24

13.2	Scope	24

13.3	Performance	24

14	Spares Requirements	24

15	Service and Maintenance Requirements	24

16	Documentation Requirements	25

16.1	Documentation Package	25

16.2	Drawings	25

16.3	Electrical/Electronic Documentation	25

16.4	Software	25

17	Glossary	26

Error! No table of figures entries found.

Table 1: NGAO Graphical User Interfaces	19

Table 2: Glossary of Terms	26

# Introduction

This document describes the requirements for the Next Generation Adaptive Optics (NGAO) system to be built for the W. M. Keck Observatory (WMKO).

The requirements in this document are at a draft level appropriate for the system design phase.  Further development of the requirements will take place in the next phase of the project (preliminary design).  In particular, parametric performance requirements given at this stage are intended to indicate the scope and format of the requirements, but do not in all cases establish final values for the specified parameters.  In some cases values for these parameters have yet to be established and are given as TBD.

A more generic set of requirements for new WMKO instrumentation is described in the Observatory’s “Instrumentation Baseline Requirements Document.”  These requirements are also applicable to NGAO.  The NGAO System Requirements Document will take precedence over the “Instrumentation Baseline Requirements Document” in the event of a conflict.

It is important to understand that at this stage of development the requirements provide a basis for identifying the parameters that will be part of the system’s specifications, but the values given are subject to change as the development continues.  During the next phases of the project work will be done to refine the requirements for review at the preliminary design review.  The final requirements to be reviewed at the detailed design review will form the basis for the acceptance test criteria for the instrument.

The purpose of this document is to define and communicate the requirements for the NGAO-specific design and implementation in terms of the needed scientific and technical performance.  The document also expresses specific requirements for implementation or design where those requirements are essential to satisfactory integration and interoperation of NGAO with the observatory systems.  The document avoids prescribing specific design or implementation solutions except for solutions that embody the Observatory’s unique knowledge or experience.  The document establishes requirements for the NGAO that will guide the design through the detailed design phase.

# Scope and Applicability

This document establishes requirements for all aspects of NGAO beyond those already specified in the “Instrumentation Baseline Requirements Document”.  This document also establishes requirements for changes to related Keck telescope subsystems and software where required.

This document does not address the requirements for the science instruments that will work with NGAO, although it does cover the NGAO interfaces to these instruments.  Separate system requirements documents will need to be prepared for each of these instruments as part of their design process.

# References

## Related Documents

- Instrumentation Baseline Requirements Document.

- KAON 455.  Science Case Requirements Document.

- KAON 399.  NGAO Proposal Executive Summary.

- KAON 400.  NGAO Proposal.

- KAON 428.  Implications and requirements for Interferometry with NGAO.

- ANSI Z136.1 Safe Use of Lasers Indoors (2000).

- ANSI Z136.6 Safe Use of Lasers Outdoors (2000).

## Referenced Drawings

Table X lists the drawing numbers, revisions and date, source and title for all drawings referenced in this document.

# Revision History

| Version | Date | Author | Reason for revision / remarks |
| --- | --- | --- | --- |
| 0.1 | Jan. 16, 2007 | Wizinowich | Initial version |
| 0.3 | Feb. 1, 2007 | Wizinowich | Multiple edits |

# Background

## Purpose

The purpose of the background section of this document is to provide context and related information for the requirements defined in later sections of this document.

## Motivation for the Development of NGAO

The Keck telescopes are the world’s largest optical and infrared telescopes.  Because of their large apertures the Keck telescopes offer the highest potential sensitivity and angular resolution currently available.  WMKO has already demonstrated scientific leadership in high angular resolution astronomy with the first NGS and LGS AO systems on 8-10-m telescopes.  The importance of achieving the full potential of the Keck telescopes is recognized in the Observatory’s strategic plan which identifies leadership in high angular resolution astronomy as a key long-term goal.

In order to maintain our leadership in this field we must pursue new AO systems and the instrumentation to exploit them.  We have examined, and our continuing to examine, a broad range of key science goals in order to identify the most compelling future science goals of our community and to determine what is needed to realize these goals.  As a result we have identified that NGAO should provide the following suite of capabilities:

- Near diffraction-limited performance at near infrared wavelengths, producing a point spread function with unprecedented precision, stability and contrast;

- Vastly increased sky coverage and multiplexing capability, enabling a much broader range of science programs; and

- AO correction in the red portion of the visible spectrum (0.6-1.0 µm), delivering the highest angular resolution images available for filled aperture telescopes.

NGAO will be a broad and powerful facility with the potential to achieve major advances in astrophysics.  It will provide dramatic gains in solar system and galactic science where AO has already demonstrated a strong scientific impact.  NGAO will allow for extraordinary advances in extragalactic astronomy, far beyond the initial gains being made with the Observatory’s current AO systems.

The NGAO proposal (KAON 400) and NGAO proposal executive summary (KAON 399) provide more background on the motivation for the development of NGAO.  Further scientific motivation is provided in the NGAO science case requirements document (KAON 455).

## Overview

The scientific and technical requirements for NGAO result in the following basic systems:

- AO system.  The AO system will likely consist of an AO enclosure, an opto-mechanical system, and software and electronics for both non real-time and real-time control.

- Laser facility.  The laser facility will likely consist of a laser enclosure, the laser(s), the launch facility including beam transport, safety systems and laser system control electronics and software.

- Science operations facility.  The science operations facility will primarily include the software and computers required to support operation of the AO system and science instruments.  This includes operating the systems for nighttime observing as well as pre- and post-observing activities.

- Science instruments.  The three highest priority instruments are currently a near-IR imager, a visible imager and a deployable near-IR integral field unit (IFU).  Three lower priority instruments have also been identified including a near-IR IFU, a visible IFU and a 3-5 µm imager.  There is also a requirement that the NGAO project be designed so as to allow the continued AO support of the Interferometer as well as a fiber injection module used for the ‘OHANA (Optical Hawaiian Array for Nanoradian Astronomy) project.

These systems will have to interface with the telescope structure.  Figure 1 shows a schematic view of a Keck telescope.  The most likely location for the NGAO system and science instruments is on one of the Nasmyth platforms of the telescope.  Nominally we have chosen the left Nasmyth platform of the Keck II telescope as our starting point.  The most likely location for the projection telescope is behind the f/15 secondary mirror in the top end of the telescope.

# Overall Requirements

## Science Requirements

### Purpose and Objectives

The purpose of the science overall requirements section is to summarize and convey requirements that apply generally to the overall NGAO system and its accessories.  These are based on the Science Case Requirements Document (KAON 455) and, in this initial release, also on the NGAO proposal (KAON 399).

### Performance Requirements

A first cut at the performance requirements, based on the science requirements, is taken directly from the NGAO proposal.  These will be updated as the science case requirements and performance budgets become better defined.

| Budget | Recommended Value | Based on Requirements |
| --- | --- | --- |
| Throughput to Science Instrument (telescope + AO) | >= 70% at 0.6-5.5 um | Science object wavelength |
|  | >= 60% at 5.5-14 um | Magnitude limit |
|  |  | Desired SNR |
|  |  | Typical exposure time |
| Emissivity to Science Instrument (telescope + AO) | <= sky emissivity at K, L & M | Science object wavelength |
|  |  | Magnitude limit |
|  |  | Desired SNR |
|  |  | Typical exposure time |
|  |  | Companion sensitivity |
| Wavefront Error Budget | 140 nm for 1% sky coverage (celestial sphere average, z=30) | Image quality |
|  | 160 nm for 20% sky coverage | Sky coverage |
|  | 190 nm for 80% sky coverage |  |
| Photometric accuracy | 0.01 mag at 0.7-2.5 um for < 5" from H < 16 NGS |  |
|  | 0.02 mag at 0.7-3.5 um for < 10" from H<16 NGS |  |
|  | 0.05 mag at 0.9-2.5 um for < 20" off-axis & 20% sky coverage |  |
|  | 0.1 mag at 0.7-2.5 um for < 20" off-axis & 20% sky coverage |  |
| Astrometric accuracy | 0.1 mas for Galactic Center |  |
|  | < 10 mas for 0.7-3.5 um & 30% sky coverage |  |
|  | < 50 mas for 0.7-3.5 um & 50% sky coverage |  |
| Polarimetry accuracy | < 0.5% |  |
| Companion Sensitivity | > 4 magnitudes at 0.055" at 1-2.5 um for Galactic Center |  |
|  | > 10 mags at 0.5" at 0.7-3.5 um for 30% sky coverage & < 20" object diameter |  |
| Observing Efficiency | <= 20 min overhead per night | Facility-class |
|  | <= 120 sec between end of slew & ready for science exposure |  |
|  | <= 10 sec between start of nod command & ready for science exposure |  |
|  | <= 120 sec to switch between LGS & NGS mode |  |
|  | <= 600 sec to switch between NGS & LGS mode (assuming daytime setup) |  |
|  | <= 120 sec to switch between science instruments (assuming daytime setup) |  |
| Up-time | <= 5% of time lost to problems | Facility-class |

The atmosphere and telescope parameters assumed for achieving these numbers need to be summarized in this KAON or a document that can be referenced.

### Science Instrument Requirements

The NGAO system must be capable of supporting the following science instruments (in rough order of priority), based on the NGAO proposal:

- Visible imager.  Wavelength range = 0.6 to 1.1 µm.  Field of view = 20”x20”.  Image sampling = 6 mas pixels.

- Near-IR imager.  Wavelength range = 1.0 to 2.45 µm.  Field of view = 20”x20”.  Image sampling = 10 mas.

- Deployable near-IR Integral Field Unit (IFU).  Wavelength range = 1.0 to 2.45 µm.  Field of Regard = 1.5’x1.5’ with 3x3.4” fields of view for each IFU.  Image sampling = 100 mas.

- Near-IR IFU.  Wavelength range = 1.0 to 2.45 µm.  Field of view from 2”x1.25” to 16”x5”.  Image sampling = 20 to 100 mas.

- Visible IFU.  Wavelength range = 0.6 to 1.1 µm.  Field of view from 1.2”x1.36” to 12”x6.8”.  Image sampling = 20 to 100 mas.

- 3-5 µm imager.  Wavelength range = 3.0 to 5.3 µm.  Field of view = 25”x25”.  Image sampling = 25 mas.

It is TBD whether the NGAO system will need to support any of the existing science instruments (NIRC2, NIRSPEC or OSIRIS).  The Interferometer and ‘OHANA requirements are discussed in section 6.2.3.

## Observatory Overall Requirements

### Purpose and Objectives

The purpose of the overall requirements section is to convey requirements that apply generally to the overall instrument and its accessories based on the Observatory’s requirements.

Note that the Observatory’s standard requirements for all new instrumentation are summarized in the Instrumentation Baseline Requirements Document.  Additional Observatory requirements specific to NGAO are listed in the following sections.

### Facility Requirements

The following are requirements imposed by the nature of the existing facility.

| # | Observatory Facility Requirements | Discussion |
| --- | --- | --- |
| 1 | The NGAO system & science instruments should be located on the Nasmyth platform of one of the Keck telescopes | The Keck telescope foci and Nasmyth deck storage locations are already heavily utilized. The current AO systems occupy the left Nasmyth platform locations of both telescopes. HIRES occupies the right Nasmyth on Keck I while DEIMOS and NIRSPEC share the right Nasmyth on Keck II. The Cassegrain foci are occupied by LRIS (and MOSFIRE in the future) on Keck I and by ESI on Keck II. The bent Cassegrain ports are believed to have inadequate space and weight capacities. The prime focus could potentially be available but there would be many constraints on an instrument at this location. The most viable option is in the location of an existing AO system. Alternatives would be to decommission HIRES or for the existing AO system and the NGAO system to be able to share the same platform. |
| 2 | The NGAO system should accommodate the entire Keck pupil | The Keck primary has a maximum edge-to-edge diameter of 10.949 m. |
| 3 | If the existing f/15 or f/40 secondary mirrors are used then the NGAO system will be constrained by the resultant f/#, focal plane and pupil location | Both telescopes have f/15 secondary mirrors, as well as chopping secondary units that can accommodate f/25 and f/40 secondary mirrors. The choice of f/15 secondary mirrors for the current AO systems was largely driven by the resultant reduced size of the AO systems and the availability of PCS (Phasing Camera System) via a rotation of the tertiary mirror. The inability of the current systems to chop at the telescope pupil has been a limitation for thermal IR observations. The focal length of the telescope with the f/15 secondary mirror is 150 m. The 10.949 m primary corresponds to an f/13.66 beam with an exit pupil diameter of 1.460 m located 19.948 m in front of the focal plane. |
| 4 | The NGAO facility must not compromise the performance of a non-NGAO instrument when that instrument is being used for science or engineering | This requirement is intended to ensure that the NGAO facility, when not in use, does not introduce vibrations or stray light that might impact the performance of another science instrument or an engineering instrument such as PCS. |
| 5 | The NGAO facility must not compromise the performance of the telescope when the telescope is used for non-NGAO observations. | The NGAO system should not impact the dynamic performance of the telescope through vibrations or different telescope dynamics. If NGAO hardware is to be mounted in the top-end then it must be designed/implemented not to compromise the secondary mirror performance. |
| 6 | The NGAO system should provide compensation for field rotation | The Alt-Az telescope design requires compensation for field rotation in order to keep the science field fixed on the science instrument. |
| 7 | The NGAO system should provide compensation for pupil rotation | The Alt-Az telescope design and the irregular shape of the Keck primary mirror require that NGAO system provide appropriate compensation for pupil rotation. Examples: The existing Keck AO system updates the reconstructor to compensate for pupil orientation. The NIRC2 coronagraph mask rotates to match the rotating pupil. |
| 8 | The NGAO system should provide compensation for LGS projector rotation | The Alt-Az telescope design will cause the laser projector (and the resultant LGS asterism) to rotate with respect to the Nasmyth platform. Compensation will be required to maintain the off-axis LGS on the corresponding wavefront sensor. |

### Science Instrument Requirements

In addition to the science instrument requirements specified in the previous section the NGAO facility must allow the Observatory to continue supporting Interferometer science with the two Keck telescopes.  The requirements for these instruments are developed in KAON 428 and are further specified in section __.

| # | Observatory Instrument Requirements | Discussion |
| --- | --- | --- |
| 1 | The NGAO facility should support the Keck Interferometer (KI) with performance as good or better than provided by the pre-NGAO Keck AO systems | The KI dual star modules (DSM) currently move into both AO enclosures on rails to feed the KI. The requirement to feed the KI requires that collimated light can be fed to the DSM and that the field rotation, pupil rotation, longitudinal dispersion and polarization from the NGAO system and the AO system on the other telescope be identical. See KAON 428 Implications and Requirements for Interferometry with NGAO. |
| 2 | The NGAO system should support the ‘OHANA interferometer with performance as good or better than provided by the pre-NGAO Keck AO systems | Injection modules are currently placed on each AO bench to feed an optical fiber that goes to the KI. In future the output from these fibers will be interfered with those from multiple telescopes. |

### Operational Requirements

The purpose of this section is to document the Observatory’s overall requirements for the support of science operations.

| # | Observatory Operational Requirements | Discussion |
| --- | --- | --- |
| 1 | The NGAO system must be facility-class | Facility-class has many implications on safety, operability, reliability, maintainability, lifetime, documentation, configuration management, etc. |
| 2 | The NGAO system must complete an operations transition review where operational responsibility is transferred from development to operations | This has implications on defining transition requirements and on training. |
| 3 | The NGAO system must be capable of supporting 275 nights/year of science operation |  |
| 4 | The Mauna Kea laser projection requirements must be satisfied | This includes requirements on laser power, wavelength, laser traffic control participation, aircraft safety and space command. |
| 5 | The NGAO system must operate within specifications under the normal summit temperature and humidity conditions |  |
| 6 | The NGAO facility should not require more than two engineering nights per year for system maintenance |  |
| 7 | The NGAO facility should not routinely require more than 30 minutes of daytime telescope restriction on a science night |  |
| 8 | The NGAO facility should be designed to keep the total annual operational personnel and non-personnel costs below the following budgets: - Four FTEs and $400k (FY07 dollars) in non-personnel costs for the first 100 nights/year of science operations - An additional cost of 0.5 FTEs and $50k (FY07 dollars) in non-personnel costs for each additional 50 nights of science operations. |  |

# Optical Requirements

## Purpose and Objectives

The purpose of this section is to describe requirements for the performance, implementation and design of the NGAO optical system.

Preliminary optical parametric performance requirements are grouped into two categories, typical parameters and goal parameters.  Requirements for typical parameters are given where enough is known to establish a range of values for the listed parameters.  Requirements for goal parameters are given where scientific or technical uncertainty prevents a full definition of the achievable range of values for the listed parameters.

During the preliminary and detailed design phases these requirements (which at least in part drive the specifications for the system) will be refined into a set of specifications.  In this process goal parameters will be refined through research and development activity into typical values.  The minimum and maximum values for parameters listed in the requirements for goal parameters indicate the acceptable bounds on worst-case performance.

## Performance Requirements

| # | Optical Performance Requirements | Discussion |
| --- | --- | --- |
| 1 | The telescope plus NGAO throughput to the science instruments shall be ≥ 70% at 0.6-5.5 µm |  |
| 2 | The telescope plus NGAO throughput to the interferometer shall be ≥ 60% at 5.5-14.0 µm |  |
| 3 | The telescope plus NGAO emissivity to the science instrument shall be ≤ the sky emissivity at K and L-bands |  |
| 4 | The rms wavefront error for the combined telescope, NGAO system and visible and NIR imagers shall be TBD for the following TBD conditions |  |
| 5 | The rms wavefront error for the combined telescope, NGAO system and visible and NIR IFUs shall be TBD for the following TBD conditions |  |
| 6 | The rms wavefront error for the combined telescope, NGAO system and deployable NIR IFU shall be TBD for the following TBD conditions |  |
| 7 | The rms wavefront error for the combined telescope, NGAO system and thermal NIR imager shall be TBD for the following TBD conditions |  |
| 8 | The rms wavefront error for the combined telescope and NGAO system delivered to the Interferometer shall be TBD for the following TBD conditions |  |
| 9 | The residual static wavefront errors from the NGAO system shall be ≤ 50 nm rms as delivered to the visible and NIR imagers |  |
| 10 | The NGAO system shall introduce ≤ TBD % polarization in the path to the TBD science instruments |  |
| 11 | The stability of the field produced by the NGAO system shall be TBD as delivered to the TBD science instruments |  |
| 12 | The rms residual tilt from the NGAO system output to the interferometer should be ≤ 0.007” |  |
| 13 | Instability must not decrease the Strehl on the visible and NIR imagers by more than 5% during any 1-hour period or over a 3 ºC temperature change | The image improvement provided by the facility must be stable over periods of hours, under stable atmospheric conditions. The wavefront sensor calibrations must be stable with time, temperature and telescope position. This requirement does not include performance degradations due to changing atmospheric conditions, or different magnitude and location of NGS or LGS |
| 14 | Instability must not decrease the Strehl on the visible and NIR imagers by more than 10% during any 24-hour period or over a 5 ºC temperature change |  |

## Implementation Requirements

| # | Optical Performance Requirements | Discussion |
| --- | --- | --- |
| 1 | The field of view provided to the science instruments must be centered on the telescope elevation axis to within 2” |  |

## Design Requirements

NGAO is required to provide an AO corrected beam to each of the science instruments, including the Interferometer and ‘OHANA fiber injection module.

| # | Optical Performance Requirements | Discussion |
| --- | --- | --- |
| 1 | A wavelength range of 0.6 to 1.0 µm must be provided to the visible science instruments (imager and IFU) |  |
| 2 | A wavelength range of 1.0 to 2.45 µm must be provided to the NIR science instruments (imager, IFU and deployable IFU) |  |
| 3 | A wavelength range of 3.0 to 5.3 µm must be provided to the thermal NIR imager |  |
| 4 | A wavelength range of 1.1 to 14 µm must be provided to the Interferometer |  |
| 5 | A wavelength range of 1.1 to 2.45 µm must be provided to the OHANA injection module |  |
| 6 | A field of view of ≥ 20”x20” must be provided to the science imagers (visible and NIR) |  |
| 7 | A field of view of ≥ 12” diameter must be provided to the visible IFU |  |
| 8 | A field of view of ≥ 16” diameter must be provided to the NIR IFU |  |
| 9 | A field of regard of ≥ 1.5’ diameter must be provided to the NIR deployable IFU |  |
| 10 | A field of view of 25”x25” must be provided to the 3-5 µm imager |  |
| 11 | A field of view of ≥ 1’ diameter must be provided to the Interferometer |  |
| 12 | A field of view of 5” diameter must be provided to the OHANA injection module |  |
| 13 | The focal ratio provided to the science instrument should be TBD |  |
| 14 | The exit pupil location provided to the science instrument should be TBD |  |
| 15 | The NGAO system should be capable of correcting TBD level of non-common path aberration in the science instrument | Image sharpening can be used to correct for aberrations in the science instruments. New science instruments should be designed to have small optical aberrations. The interferometer and possible legacy instruments such as NIRC2 or OSIRIS may be allowed larger aberration budgets. |
| 16 | The peak-to-peak range of tip/tilt correction provided by NGAO shall be ≥ 3” | The existing Keck AO systems have a peak-to-peak range of 1.6” which has proven to be inadequate in windy conditions. |
| 17 | NGAO must be able to support a chopping mode for the interferometer. | The nuller requires small amplitude chopping with the AO loops closed at each end of the chop for alignment purposes. |
| 18 | The interferometer output of NGAO must be polarization matched to the interferometer output of the AO system on the other telescope in order to produce ≤ 3º of differential s-p phase shift | The current KI achieves polarization matching by keeping the number, angle and coatings of all reflections the same in the beam trains from each telescope. The differential s-p phase shift in the current KI is measured at 6º resulting in a loss in V2 of 0.003. |
| 19 | The interferometer output of NGAO must have the same image rotation as the interferometer output of the AO system on the other telescope |  |
| 20 | The interferometer output of NGAO must have the same pupil rotation as the interferometer output of the AO system on the other telescope |  |
| 21 | The interferometer output of NGAO must have the same longitudinal chromatic dispersion as the interferometer output of the AO system on the other telescope | Transmissive optics fabricated from different materials can have different amounts of longitudinal chromatic dispersion resulting in the loss of fringe visibility |
| 22 | The ratio of the Strehls from the interferometer output of NGAO and the interferometer output of the AO system on the other telescope must be ≤ 1.2 and ≥ 0.9. | A Strehl mismatch of 22% or an intensity ratio of 1.22 results in a V2 loss of 0.010. |
| 23 | NGAO must be able to accommodate the accelerometers needed to support the Interferometer | On the current AO bench one accelerometer is placed near the telescope focus and another near the output to the DSM. These are used to measure vibration along the optical path and are used in the fringe tracker control system. The accelerometer acquisition system is housed in an electronics rack in the AO electronics vault. |
| 24 | NGAO or NGAO in combination with a modified DSM must be capable of supporting the laser metrology beams from the interferometer | These metrology beams are a potential source of background light on the wavefront sensors |
| 25 | NGAO must incorporate the required tools and tolerances to support alignment to the interferometer | For example, the current AO bench hosts a corner cube to aid in aligning the interferometer to the optical axis of the AO system and telescope |
| 26 | NGAO or NGAO in combination with a modified DSM must provide a collimated 100 mm diameter beam to the interferometer | In the current AO system a removable (on a translation stage) dichroic beamsplitter, located between the deformable mirror and second off-axis parabola, folds the collimated beam to the DSM |

# Mechanical Requirements

## Purpose and Objectives

The purpose of this section is to describe requirements for the performance, implementation and design of the NGAO mechanical systems.

The mechanical requirements address issues of design, reliability and maintainability.  Based on experience with previous instruments the observatory is sensitive to certain aspects of performance, implementation and design that have proven to be important factors in the up time of its instruments.  The mechanical requirements section has as a main objective the description of the expected performance, features and configuration of the instrument’s mechanical systems.  A secondary objective is to identify specific areas where experience indicates particular attention is required with respect to performance, implementation or design.

## Performance Requirements

### Parametric Performance Requirements

### Operational Performance Requirements

## Implementation Requirements

| Budget | Recommended Value | Based on Requirements |
| --- | --- | --- |
| Weight | <= 10,000 kg for AO system on telescope |  |
|  | <= 150 kg for hardware in the top-end module |  |
|  | <= 10,000 kg for laser system on telescope |  |

# Electronic/Electrical Requirements

## Purpose and Objectives

The purpose of this section is to describe requirements for the performance, implementation and design of the NGAO electronic and electrical systems.

The electronic/electrical requirements address issues of safety, design, reliability and maintainability.  Based on experience with previous instruments the observatory is sensitive to certain aspects of performance, implementation and design that have proven to be important factors in the up time of its instruments.  The electronic/electrical requirements section has as a main objective the description of specific requirements for implementation and design.

A key consideration is the safety of personnel and equipment, and proper electrical design and implementation practices in compliance with recognized standards are an essential aspect of electrical safety.  A second consideration is the electromagnetic compatibility of the instrument with the observatory systems, and specific implementation and design requirements are given to aid in achieving the required electromagnetic emissions and susceptibility performance.

## Performance Requirements

### Parametric Performance Requirements

#### Electrical Power and Thermal Dissipation

The preliminary electrical power and thermal dissipation requirements for NGAO are given in the following table:

| Budget | Recommended Value | Based on Requirements |
| --- | --- | --- |
| Electrical Power | <= 10 kW for AO system |  |
|  | <= 20 kW for laser system |  |
| Thermal Dissipation | <= 100W from AO system |  |
|  | <= 100W from laser system |  |
|  | <= 50W at top-end |  |

## Implementation Requirements

### Feature Implementation Requirements

## Design Requirements

### Technological Design Requirements

### Integration Related Design Requirements

# Safety Requirements

## Purpose and Objectives

Safety is the paramount concern for all activities at the observatory.  The purpose of this section is to provide requirements related to specific safety concerns during the operation and handling of NGAO.

## Scope

The general safety requirements that are applicable to NGAO are already contained in the Instrumentation Baseline Requirements Document.  This section also covers the additional laser safety and laser projection safety requirements.

Adequate earthquake restraints are also required for all systems.

## Laser Safety Requirements

A safety system will be implemented to ensure the safe use of the laser.  This safety system will include both engineering and administrative/procedural controls to assure safe operations.  The system will apply ANSI Z136.1 and Z136.6 standards for safe use of laser for indoor and outdoor.

The NGAO system will conform to OSHA and local codes in addition to codes specified for each subsystem.

## Laser Projection Safety Requirements

This section covers the additional requirements on safety for projecting the laser beams outside the dome.

### Aircraft Safety

An aircraft safety system compliant with FAA requirements must be implemented and approved by the FAA.

### Space Command

A system must be implemented to facilitate effective communication with U.S. Space Command of projection dates and targets, and to ensure that no projection occurs on a target without Space Command approval.  This requirement may be deleted if it is determined that Space Command approval is no longer required.

# Software Requirements

## Purpose and Objectives

The software requirements section describes requirements for performance, implementation and design.  Based on experience with previous instruments the observatory is sensitive to certain aspects of performance, implementation and design that have proven to be important factors in the up time of its instruments.  The software requirements section has as a main objective ensuring compatibility of the NGAO software with existing observatory software systems.  A secondary objective is guiding the selection of software architecture and implementation decisions towards those that fit within the software skill sets at the observatory in order to maximize the ability of the observatory to support and maintain the NGAO software.

## Scope

Unless otherwise indicated all of the requirements of this section apply to all software components of NGAO.

## Performance Requirements

## Implementation Requirements

## Design Requirements

The interfaces between the current

| # | Software Requirements | Discussion |
| --- | --- | --- |
| 1 | The NGAO system must be able to support the external interfaces supported by the existing AO systems | The external keywords and EPICS channels used by the existing AO systems are documented in KAON 315, Summary of External Interfaces in the Current WFC and Implications for the NGWFC Design” |
| 2 | The NGAO system must be able to accept tip/tilt offloads from the science instrument |  |
| 3 | The NGAO system must be able to acquire and configure the AO system based on the star magnitude and spectral type provided in the target list |  |
| 4 | NGAO must be able to support a chopping mode for the Nuller mode of the interferometer |  |

# Interface Requirements

## Purpose and Objectives

This section is reserved for interface requirements that are not addressed by other portions of the document.

## Performance Requirements

### Parametric Performance Requirements

None.

### Operational Performance Requirements

## Implementation Requirements

### Optical Requirements

### Mechanical

| # | Interface Requirements | Discussion |
| --- | --- | --- |
| 1 | An agreed upon kinematic interface between the NGAO opto-mechanical systems and the telescope structure must be provided. | Kinematic interfaces are proposed so that thermal changes do not distort optical benches. |
|  | NGAO must be compatible with the interferometer dual star module (DSM) or replicate its functionality | The current Keck AO systems interface to the interferometer via an opto-mechanical system known as the DSM. |
| 2 | An agreed upon mechanical interface between the NGAO electronics and the telescope must be provided |  |
| 3 | An agreed upon mechanical interface between any NGAO enclosures and the telescope must be provided. |  |
| 4 | An agreed upon mechanical interface between any NGAO glycol cooled systems and the telescope instrument and/or facility glycol systems must be provided |  |
| 5 | An agreed upon mechanical interface between any NGAO CCR-cooled systems must be provided |  |

### Electrical

| # | Interface Requirements | Discussion |
| --- | --- | --- |
| 1 | An agreed upon electrical interface for power between NGAO systems and the Observatory/telescope must be provided | The Observatory is responsible for providing power to the various NGAO system locations. |
| 2 | An agreed upon interface for communication between NGAO systems and the Observatory/telescope must be provided | The Observatory is responsible for the implementation of all cables between NGAO system locations. The Observatory must in particular approve all cabling required to go in the elevation or azimuth wraps. |

### Software

| # | Interface Requirements | Discussion |
| --- | --- | --- |
| 1 | An interface should be provided to offload tip/tilt errors to telescope pointing through the telescope drive and control system (DCS) |  |
| 2 | An interface should be provided to offload focus errors to the secondary mirror piston through DCS |  |
| 3 | An interface should be provided to offload coma errors to the secondary mirror tilt through DCS |  |
| 4 | An interface should be provided to offload segment stacking errors to the active control system (ACS) |  |
| 5 | An interface should be provided to write all appropriate NGAO keywords to the science instrument FITS header |  |
| 6 | An interface should be provided to allow NGAO to read/write NGAO science instrument keywords |  |

## Design Requirements

### Technological Design Requirements

None.

### Integration Related Design Requirements

| # | Implementation Requirement | Discussion |
| --- | --- | --- |
| 1 | The time between decommissioning an AO capability on the telescope where the NGAO system is to be installed and making NGAO available for limited shared-risk science must not be longer than 6 months | Down time Impact on Interferometer and AO science at the Observatory. |
| 2 | The telescope downtime required to implement NGAO must not be longer than 5 days |  |

#### Optical Interface

#### Mechanical Interface

#### Electrical/Electronic Interface

# Reliability Requirements

## Purpose

A process should take place to confirm that the NGAO system will provide a high level of reliability for a 10 year lifetime.

## Scope

Unless otherwise indicated all of the requirements of this section apply to all components of NGAO.

## Performance

System downtime should be minimized by a combination of component reliability, ease of repair, maintenance and appropriate sparing.

| # | Reliability Performance Requirements | Discussion |
| --- | --- | --- |
|  | ≤ 5% of observing time lost to problems | This includes any loss to an exposure in progress and the time to start the next exposure after recovering from a fault |
|  | The median time between faults during observing time should be ≥ 3 hrs | Frequent short duration faults are not acceptable since they have a high impact on science productivity |

# Spares Requirements

TBD

# Service and Maintenance Requirements

# Documentation Requirements

The documentation requirements are defined in the baseline requirements document.

## Documentation Package

## Drawings

## Electrical/Electronic Documentation

## Software

# Glossary

Table 1 defines the acronyms and specialized terms used in this document.

Table 1: Glossary of Terms

| Term | Definition |
| --- | --- |
| ACS | Active Control System |
| ANSI | American National Standards Institute |
| AO | Adaptive Optics |
| DCS | Drive and Control System |
| FAA | Federal Aviation Administration |
| FOV | Field Of View |
| FWHM | Full Width at Half Maximum. |
| IFU | Integral Field Unit |
| KAON | Keck Adaptive Optics Note |
| KI | Keck Interferometer |
| LGS | Laser Guide Star |
| MTBF | Mean Time Between Failures |
| NGAO | Next Generation Adaptive Optics |
| NGS | Natural Guide Star |
| NIR | Near InfraRed |
| ‘OHANA | Optical Hawaiian Array for Nanoradian Astronomy |
| OSHA | Occupational Safety and Health Administration |
| SSC | Science Steering Committee |
| TBC | To Be Completed |
| TBD | To Be Determined |
| WMKO | W. M. Keck Observatory |