Making Centroid offsets from a DM map

04/28/2007

This procedure is to make a centroid offset map from a DM map. This procedure should be used among other things to convert the DM maps generated from phase retrieval to centroid offsets.

- Load the desired DM map to use

  - If needed copy the map (here called XXX) to /proj/aocp/tables/dm_calib_pos/XXX

  - Load the map: “load wfp=ho, dm_calib_pos=aocp:/tables/dm_calib_pos/XXX)

- load co_zero:  “load wfp=ho,cent_offsets=aocp:/default/cent_offsets/co_zero”

- use pixel_gains all: “load wfp=ho, pixel_gains=aocp:/default/pixel_gains/pixel_gains_all”

- white_z should be at the position used to create the DM map, nominally 25749 (“move white_z 25749”)

- acquire white light as normal on the WFS (Take a sky, register DM)

- close TT only

- Create centroid offset file

Here we want to average 10 seconds of centroid data to create the centroid file.

  - Change directories to where centroids are stored “cd /proj/aocp/tables/cent_offsets”

  - Start IDL and type “ao_db_find,start='start',stop='start+10 ',/ho_avg_centroids,/nobin”, where the star t time is something like 1175755371. You can get this from the log file or print_conf

  - copy the create filed to  the desired name:  “cp ho_avg_centriods co_ XXX”

  - Create the centroid offset file divided by 2 and divided by 4 as normal. Or use the procedure below. In IDL

    - Data = ao_read_co(“co_XXX)

    - ao_write_co, data/2, “co_XXX_2”

    - ao_write_co, data/4, “co_XXX_4”

- Check that things worked as expected

  - load above centroids offsets (load wfp=ho,cent_offsets=aocp:/tables/cent_offsets/co_XXX”

  - Close the DM loop

  - Check the image on PHARO