Palomar Adaptive Optics Test Plan

| Title | Switching Reflecting Spot |
| --- | --- |
| Date |  |
| Lead | J. Roberts |
| Time requested | 1h |
| Required conditions | Optics down, in lab or on the telescope (daytime) |

Purpose Shift the dichroic spot between the all dichroic and the NGS reflecting spot

Required equipment Lamp and extension cord Gloves

Test procedure – LGS to NGS - Register and lock on white light - Move acq_z to 15800 (focused on dichroic beamsplitter) - Illuminate the beamsplitter with a lamp - Turn up the white light power to 100 (white_light on, pwr=100) - Shift the spot location by noting the current location of the rotary mount and rotating 10.6mm from there (1 full turn = 1mm) - Visually check the location on the video monitor - It helps to set the video camera to integrate (30s) - Adjust the spot as necessary - Y centration can be adjusted using the rotary mount; X centration can be adjusted using the linear stage directly under the dichroic mount - Remove lamp and turn the white light down to 50. - Register and lock on the white light - Turn up the white light power to 100 (white_light on, pwr=100) - Visually check again if the spot appears centered - If an adjustment is necessary, repeat steps 7-9 - With all the loops open, move ttm_a in .25” steps until the white light spot shows up on the edge of the reflecting spot in the acquisition camera. Note the number of steps. - Close the TT loop to recenter, then open it. - Move ttm_a in -.25” steps until the white light shows up on the opposite edge of the reflecting spot in the acquistion camera. Note the number of steps - If the number of steps on each side differ by more than 2, repeat steps 7-9 and retest with the TTM - Check ttm_b in the same way - After the final adjustment, check both ttm_a and ttm_b - Note the final difference in steps in X and Y.

Test procedure – NGS to LGS - Register and lock on white light - Move acq_z to 15800 (focused on dichroic beamsplitter) - Illuminate the beamsplitter with a lamp - Turn up the white light power to 100 (white_light on, pwr=100) - Shift the spot location by noting the current location of the rotary mount and rotating 10.6mm from there (1 full turn = 1mm) - Visually check the location on the video monitor - Adjust the spot as necessary - Y centration can be adjusted using the rotary mount; X centration can be adjusted using the linear stage directly under the dichroic mount - Using acqview, grab and save a frame with the dichroic illuminated. - From this frame, note the center pixel of the dichroic spot (use fitsread and atv in idl) - Note the pixel locations of the edges of the spot and use these to find the center - Remove lamp and turn the white light down to 50. - Register and lock on the white light - Turn the white light back up and using acqview, grab and save a frame of the spot - From this frame, note the center pixel of the spot (use fitsread and atv in idl) - Note the pixel locations of the edges of the spot and use these to find the center - If the centers differ by more than 3 pixels, repeat steps 7-13 - Note the final center locations

Number of 0.25” steps

| ttm_a + |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ttm_a - |  |  |  |  |  |  |
| ttm_b + |  |  |  |  |  |  |
| ttm_b - |  |  |  |  |  |  |

Spot Pixel Coordinates

|  | Top | Bottom | Left | Right | Center |
| --- | --- | --- | --- | --- | --- |
| Dichroic 1 |  |  |  |  |  |
| WL 1 |  |  |  |  |  |
| Dichroic 2 |  |  |  |  |  |
| WL 2 |  |  |  |  |  |
| Dichroic 3 |  |  |  |  |  |
| WL 3 |  |  |  |  |  |
| Dichroic 4 |  |  |  |  |  |
| WL 4 |  |  |  |  |  |