A curvature-based laser guide star 
adaptive optics system for Gemini-South

Mark Chuna, Celine d'Orgevillea, Brent Ellerbroeka, J.E. Gravesb, Malcolm Northcottb, Francois Rigauta

aGemini Observatory, 950 N. A'Ohoku Place, Hilo, HI 96720

bInstitute for Astronomy, 2680 Woodlawn Drive, Honolulu, HI 96822

The Gemini Observatory and University of Hawaii are planning to install an 85-element curvature adaptive optics sytem with a laser guide star system on its Cerro Pachon telescope in 2001.  This paper discusses the motivation, issues on implementing a laser guide star with a curvature-based system, the implementation of a laser guide star based on a commercially available 2W ring-dye laser, and the expected performance of the system.  Detailed simulations show very promising results for system performance down to natural guide star magnitudes of 19-20th magnitude.  The performance cross-over point between NGS and LGS is between 13-16th magnitude depending on the performance parameter of interest (e.g. Strehl, energy through a slit, etc.)

- curvature sensing, laser guide star

- INTRODUCTION

Observations with current adaptive optics systems are limited by the requirement of a bright near-by reference source.  For astronomical adaptive optics systems (AOS) performance typically degrades for sources fainter than V~14.  This limit depends strongly on the type of AOS as well as the intrinsic seeing at the site.  However, an example of a system with one of the faintest limiting magnitudes is the University of Hawaii's Hokupaa on the Gemini-North telescope on Mauna Kea in Hawaii.  While this system currently has only 36 correcting elements, it is at a good site and the wavefront sensor is a curvature sensor based on zero-read noise avalanche photo-diodes.  Partial performance gains are seen with this system even at magnitudes as faint as R~17.  However even this system is limited in sky coverage.

To address the issue of sky coverage several groups have begun implementing artifical guide stars using a tuned laser to excite a resonance line of sodium atoms in the mesosphere1,2,3.  All orders spatially higher than tip and tilt are corrected on using the wavefront measured from this artifical source.  A natural guide star is required for measuring the tip and tilt but since the entire telescope aperture can be used for this, the limiting magnitude of the required natural guide star is fainter.  In addition, while the atmospheric phase variance is largest in the tip and tilt modes, these modes affect the image quality in a simple way.  Namely the corrected image is smeared by a two-dimensional guassian.  More importantly, for spectroscopic applications, the slit throughput degrades slowly for increasing tip/tilt variance for slits larger than the diffraction limit.

In practice the implementation of a sodium laser guide star has been difficult1,2.  Implementing the LGS with a curvature-based system brings two key advantages to ease the process.  First, the curvature-based adaptive optics system has already proven itself to be robust and reliable.  In particular, the system is less sensitive to optical misalignment than a Shack-Hartmann-based system.  Second, because the wavefront sensor is based on zero read-noise APDs, the LGS brightness requirement is greatly relaxed compared to CCD-based AOS.  This fact lowers the required photon return rate to below a critical threshold to allow the use of commercial, off-the-shelf laser components running at the manufacturer's nominal power levels.

In this paper we discuss the issues in implementing a LGS with a curvature-based system, describe the modifications to the natural guide star AOS, discuss the components of the LGS, and finally present the expected performance of the system within the context of the focal-plane instrumentation and science observations.

- MOTIVATION

We expect that the gains from adding a LGS to a curvature-based AOS will be substantial.  Expected gains over the NGS mode are (1) high-order corrections are independent of the NGS magnitude, (2) sensing of the higher-order aberrations on-axis negates higher-order angular anisoplanatism errors, and (3) an optimized sensor can be used to sense the tip/tilt errors from the NGS.  These advantages should lead to an increase in the sky coverage of the adaptive optics system.  The use of a LGS to sense the wavefront aberration modes higher than tip and tilt fixes the level of wavefront correction for these modes.  Namely, the photon noise and bandwidth errors in the wavefront correction are small if the LGS is sufficiently bright.  In addition, because the LGS is placed on- or very nearly on-axis with the science object, the angular anisoplanatism error for the higher-order terms is negligible.  There is a trade in performance between using an AOS with a NGS off-axis and using an AOS with a LGS 'on-axis' since the LGS wavefront includes errors due to focal-anisoplanatism.  From the results of the Cerro Pachon site survey4, the mean isoplanatic angle in the visible is 2.1 arcseconds while the mean scale length for focal-anisoplanatism (d0) is 3.8 meters.  Scaling both of these quantities into the H-band (1.65 microns) give characteristic errors in which the angular anisoplanatism error is generally larger than the focal-anisoplanatism error

(0)5/3 >(D/d0)5/3

This is an over-simplification of the actual performance trades since the isoplanatic error is over estimated by (0)5/3 and a signficant portion of this error is due to the tilt anisoplanatism which the LGS system still incurs.  However, it illustrates the point that there is an advantage to sensing the higher-order aberations on-axis with the laser guide star to using a natural guide star off-axis.

By decoupling the higher-order modes from the tip and tilt, we can now optimize the NGS sensor for the required lowest order modes.  Consider a curvature sensor sensing all wavefront modes.  For a given photon flux and seeing condition, the optical gain (extrafocal distance in the measurement) must be balanced with the order of the correction.  For sensing higher order modes, larger extra-focal distances are needed.  Increasing the extra-focal distance however decreases the sensitivity of the sensor since the measured signal decreases with increasing optical gain (ℓ) for a given wavefront aberration5.

I1−I2I1+I2=λf(f−ℓ)2πℓ∂ϕ∂nfrℓδc−∇2ϕfrℓ

This means that for the tip & tilt modes the optimal extra-focal distance is small.  As pointed out by Rousset (1994), in this limit the curvature sensor becomes a quad-cell that uses the entire aperture for sensing wavefront tip/tilt errors.  In the case of an LGS AOS, the NGS is used only for tip and tilt however it may be convienent to sense focus as well due to the varying distance to the sodium layer so we plan to implement either a quad-cell or a low-order curvature sensor.

Finally we note that while the atmospheric tip and tilt phase variances are the largest, the effect of residual tip/tilt errors is simply image smear.  In the focal-plane of a slit or integral-field spectrograph, if the higher-order aberrations are well corrected (i.e. there is a diffraction-limited core in the short-exposure PSF), then the degradation in slit throughput is small.

- AO AND LGS CONFIGURATION

The details of the NGS 85-element adaptive optics system can be found in Graves et al.6.  The basic modification to the AOS to accomodate a laser guide is the addition of a steerable NGS acquisition mirror, a NGS tip/tilt quad-cell sensor, and a stage to accomodate the variable range to the LGS.

The overall CP LGS system consists of five major subsystems: Laser System, Beam Transfer Optics, Laser Launch Telescope, LGS Control System, and Safety Systems.  The CP LGS system will be implemented and integrated on the telescope in late 2000/1.  The LGS system will be based around a 2W continuous-wave single-mode laser.  The power requirement was derived from the performance analysis of and detailed calculations of the interaction of photons with the sodium atoms in the mesosphere by Telle et al.7.  The requirement accounts for the beam transfer efficiency along the upward and downward paths, as well as the detailed physics of the interaction at the sodium layer of a single-mode CW laser8.  Under the assumption of a low sodium column density and an optical efficiency in the beam propagation from the laser to the sodium layer of 40%, the 2W requirement translates into a LGS magnitude of about V~11.0 at the ground.

A number of commercial lasers can produce the specified 2 Watts at good beam quality at the sodium D2 line.  A ring dye laser, similar to the lasers used by the Max Planck Institute für Astronomie and the University of Arizona have reached power levels as high as 4 Watts when pumped by a 25 W Argon-ion laser.  In our case, the reduced power requirements allow a simplification to be made with regard to the pump laser.  Our pump laser will be a continuous-wave 10 W doubled Nd:YVO4 laser @ 532 nm.  There are two possible suppliers for this diode-pumped solid-state laser: the Verdi by Coherent and the Millennia X by Spectra Physics.  Both companies have demonstrated 2.5 W of single frequency operation out of a solid-state pumped dye laser emitting at 589 nm and will warranty our 2 W output power requirement over 5000 hours of operation (about 500 nights of LGS operation).  We will need to make some modifications to the laser system to turn it into an efficient, fully automated source.  These include a sodium cell to lock the laser frequency on the peak of the sodium D2 line, modifying the laser frequency control loop electronics, and replacing the dye nozzle and the dye pump with high pressure systems.

The dye laser cannot be located on the telescope itself because the use of an open-air dye jet requires mounting the laser horizontally and monomode lasers are very sensitive to vibrations, temperature variations and dust.  Instead, the laser will be mounted in a stable environment, possibly in a room under the cable-wrap in the telescope pier.  It will be mounted on an isolated table, in a thermally controlled enclosure.  We will also use filtered air above the laser optics in order to prevent dust deposit.

The beam transfer optics brings the laser output beam to the launch telescope on the top-end of the Gemini telescope.  The positioning of the LGS on the sky is controlled by this subsystem.  The system includes active control of the beam direction and supply diagnostics on the beam pointing (far- and near-field), an indication of the beam quality, and the beam power.  The nominal design consists of a train of about 15 high quality, high reflectivity dielectric mirrors optimized for 589 nm, a relay telescope to control the beam size on the laser launch telescope, and a fast steering mirror to stabilize the LGS as seen by the WFS. At these power levels scattering from aerosols or smaller particles does not increase the background in the WFS detector.  However, in anticipation of future higher-power LGS projects at Gemini-South, the beam path may be enclosed along the telescope structure.

On the top-end, behind the secondary, we will place a beam-expanding telescope.  Two main configurations are available for designing the laser launch telescope (LLT): it could be off-axis attached to the side of the main telescope, or on-axis mounted behind the secondary mirror.  Gemini has chosen the on-axis configuration.  While this requires a more careful implementation, it minimizes the laser power and facilitates the LGS wavefront sensing.  Our current design is equivalent to a Keplerian beam expander with a deployable primary mirror to ensure that the infrared background of the Gemini telescope is not increased when the LGS is not in use.  To simplify the integration of the system, the LLT is envisioned as a stand-alone instrument mounted in its own structure with specific interfaces to the secondary frame.  This allows the unit to be tested and aligned before mounting on the telescope.

- EXPECTED PERFORMANCE

To determine the level of performance with such a system, Monte-Carlo simulations were run using a package developed by of F.Rigaut.  This package includes

- multiple layered phase screen turbulence,

- temporal evolution of the turbulence assuming "frozen" turbulence,

- full simulation of the image formation in the curvature sensor including focal-anisoplanatism,

- deformable mirror influence functions, and

- laser projection including propagation through the atmosphere and spot elongation.

Simulations were made for both the LGS and NGS configuration.  In each case (NGS and LGS), we first optimized the extra-focal distance.  For the LGS case this was optimized for a 12th magnitude LGS.  The difference between the above LGS magnitude of 11.0 and 12.0 takes into account the expected transmission loses from the telescope to the wavefront sensor detectors.  In the case of the LGS AOS we next optimized the higher-order control servo.  Simulation runs on the NGS magnitude (tip/tilt only for the LGS case) were then made.  The parameters used in the simulations are summarized in the table below.

Table 1  Summary of simulation parameters

| TELESCOPE | diameter | 7.9 meters |  |
| --- | --- | --- | --- |
|  | central obscuration | 0.1295 |  |
| ATMOSPHERE | D/r0 (@ LGS) | 43.8 |  |
|  | r0 (0.5 ) | 16.6 cm |  |
|  | 0 (0.5 ) | 2.6 arcsec |  |
|  | CN2 distribution |  |  |
|  | altitude (km) | Vwind(m/s) | Relative CN2 Strength |
|  | 2.7 | 6.7 | 0.64 |
|  | 4.5 | 8.3 | 0.08 |
|  | 6.0 | 13.4 | 0.119 |
|  | 8.5 | 25.6 | 0.035 |
|  | 10.1 | 33.9 | 0.025 |
|  | 15.75 | 22.2 | 0.08 |
|  | 18.5 | 8.9 | 0.015 |
| SENSOR | WFS | 589 nm |  |
|  | extra-focal distance | 0.7 meters |  |
|  | field stop | 4.0 arcseconds |  |
| LASER GUIDE STAR | sodium layer altitude | 95 km |  |
|  | sodium layer thickness | 8 km |  |
|  | launch telesope location | behind telescope secondary |  |
|  | beam waist diameter | 0.3 m |  |
|  | launch telescope diameter | 0.45 m |  |
|  | LGS magnitude | 12.0 (corresponding to 11.0 at entrance of telescope) |  |
|  | off-axis angle | 0.0 (LGS-mode) 0.0 (NGS-mode) |  |
|  | sky magnitude | 20.7 mag / arcsec2 |  |
| TIP-TILT GUIDE STAR | magnitude | 13 – 21 |  |
|  | sky magnitude | 19.2 magnitude / arcsec2 |  |
|  | pixel size | 0.5 arcsec/APD |  |
|  | field of view | 2x2 APDs |  |
|  | TT | 700 nm |  |
|  | offaxis angle | 30.0 arcseconds |  |

The H-band (1.65 micron) performance of the AOS in NGS and LGS mode is given in Figure 1 below.  .  Note that these performance estimates are for the adaptive optics system only so telescope and instrument errors must still be considered.  Also note that while tip/tilt angular anisoplanatism is included in the LGS calculation (gs=30''), no angular anisoplanatism was put in the NGS calculations.  The anticipated cross-over in performance between the NGS and LGS modes is evident and occurs around R~14.5 in the system StrehlThe limiting magnitude in the LGS case is set by the photon counts above the sky background required for tip and tilt correction.  Figure 1 also shows the expected slit throughput for slit widths of 1.5x and 2.5x the diffraction-limited width for a point source.  Throughput greater than 30% is maintained to limiting magnitudes of ~20.  At this point, the sky background becomes the limiting noise source and the slit throughput drops steeply.  Although not shown, simulations at other wavelengths were also run.  Good performance is given for wavelengths greater than 1 micron.  For example, the LGS AOS system at a NGS (tip/tilt only) magnitude of R~17 gives a Strehl at 1 microns of only 5% but a FWHM of about two times the diffraction limit (~55mas).

Figure 1  Estimated system performance (AOS only)

Increasing the limiting magnitude increases the number of targets that can be observe by roughly 20% for each magnitude past R~15.  Taking the star counts of Bahcall and Soniera9 we find that at the R<=18th magnitude there are roughly 3000(galactic latitude=30 degrees) and 1000 (galactic pole) stars per square degree.  For a NGS patrol field radius of 30 arcseconds this translates into a sky coverage of between 20-50% depending on the galactic latitude.  Importantly, as illustrated in Figure 2, slit throughput above 30% can be obtained for most of the sky.

Table 2.  Sky Coverage past for fainter limiting magnitudes

| NGS magnitude | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| galactic latitude = 30 | 12% | 23 | 35 | 48 | 73 | 76 |
| galactic latitude = 90 | 4% | 8 | 12 | 20 | 28 | 35 |

Figure 2.  Sky coverage as a function of slit throughput

At these limiting magnitudes, we can begin to consider other astrophysical objects as viable tracking sources.  For example, galactic cores or bright star formation regions are often compact and brighter than R~19-20.  As an example we note that the distribution of quasar brightnesses peaks around v~18.  From this target set, systematic studies of the fields around quasars can be made out to redshifts of z~4.

- Conclusions

A curvature-based adaptive optics system outfitted with a laser guide star on the Gemini-South telescope is expected to give good performance in image resolution and slit throughput for a significant fraction of the sky.  The LGS system extends the image correction capabilities to limiting NGS magnitudes of R~19.  At this limit more than one third of the sky is observable.  The use of avalanche photo diodes allows the use of a lower-cost commercial laser to pump the mesospheric sodium layer.  We expect that from a 2-Watt laser we will have sufficient photon return to use the system in all conditions.

- References

- S. Olivier et al. 1999, "Improved performance of the aser guide star adaptive optics system at Lick Observatory", Proc. SPIE Vol 3762 p. 2-7.

- S. Hippler et al. 1998, "ALFA: the MPIA/MPE adaptive optics with a laser project", Proc. SPIE Vol 3352, 44-55.

- E. Kibblewhite and F Shi, "Design and field-tests of an 8-W sum-frequency laser for adaptive optics", Proc SPIE Vol 3353, 300-309.,1998

- R. Avila et al , "Turbulence and Wind Profiling with Generalized SCIDAR at Cerro Pachon", these proceedings, 2000

- G. Rousset, G., in Adaptive Optics for Astronomy Ed. D.M. Alloin and J.-M. Mariotti., 1994

- J.E. Graves et al., these proceedings, 2000.

- John M. Telle, Peter W. Milonni, and Paul D. Hillman, "Comparison of pump-laser characteristics for producing a mesospheric sodium guidestar for adaptive optical systems on large-aperture telescopes", SPIE Proceedings vol. 3264, 1998

- C. d'Orgeville et al., "Gemini Mauna Kea Laser Guide Star system", SPIE Proceedings vol. 3762, 1999.

- Bahcall and Soneira 1984, ApJS, 55, 67.