Palomar Adaptive Optics Test Plan

| Title | BTO afternoon tests |
| --- | --- |
| Date | 6/12/06 |
| Lead | A. Bouchez |
| Time requested | 4:00pm to 6:30pm |
| Required conditions | N/A |

Purpose Afternoon test and optimization of BTO performance with the 589nm laser, and demonstration of off-zenith laser projection.

Previous analysis See http://www.oir.caltech.edu/twiki_oir/bin/view.cgi/Palomar/PalmLGS/BtO

Test procedure - Test safety system. - Arm safety system with zone 8 jumpered (no dome interlock). - Unlock and open Coude port. HP and AB to prime focus with filters. - Test LLT focus mechanism. - Project the 660nm laser. Adjust M4 and Q3 b-s to put light on Q3. Verify that signal levels are acceptable. - Demo low- and high-speed servo loop on Q3. - Set Q3 safety system threshold to just above Q3 660nm signal level. - Reinsert Coude port. - Project 589nm laser in Coude lab. Adjust beam through Coude BTO optics if necessary. - Close laser shutter - Open Coude port. Perform final safety checks. - Project 589nm laser in dome. - Close servo loops on all quad cells. - Verify signal levels on quad cells. If necessary, modify filters in front of Q1 (currently RG610) and Q3 (currently RG630), or ½-wave plate zeropoint. - Optimize and test Q3 safety system threshold level. - Set new threshold if needed (10% above 660nm level) - Shutter laser - Offset M2y by 200 counts. - Open shutter - verify that Q3 safety interlock triggers. - Test servo loops on 589nm beam, and tune gains to optimize performance. - Measure BTO transmission to LLT. - Shutter laser, prepare to move telescope. - Demonstrate off-zenith laser projection at [0h, 20dec], [+1h, 20dec], [+2h, 0dec] - Slew to Dec location on meridian. - Close servo loop on 660nm laser. - Slew to final position. - Open 589nm laser shutter. - Demonstate Q3 interlock operation as above.

Results and conclusions