Palomar Adaptive Optics Test Plan

| Title | LGS Acquisition and characterization |
| --- | --- |
| Date | 7/12/06 |
| Lead | M. Troy |
| Time requested | 1 hr |
| Required conditions | - |

Purpose Acquire LGS, focus LLT, determine LGS magnitude and spot size,

Test procedure - Setup - Telescope at zenith, at best NGS focus. - Acquisition camera filter installed. - Acq focused at 90 km altitude (NGS focus-2450) - NGS 14500 - LGS 11950 - LLT focused on at 90 km altitude (NGS focus - 300). - NGS 11600 - LGS 11300 - laser focus set to 4000 - Acquisition camera setup: - AO> video cam=acq, int_time=5000000, interval=1 - In an appropriate experiment directory: IDL> ao_plot_vid_image - Laser propagating. - Acquire LGS on Acq - Steer LGS onto Acq using following commands: - offset llt_a +1.0 moves LGS 6 pixels down. - offset llt_b +1.0 moves LGS 4.6 pixels right. - Focus Acq, LLT, and HOWFS on Na layer - Rough focus, Look at spot in acq move llt_focus in steps of 50, acq_z in steps of 300 - Detune laser, take and save a 5s background image. - Focus Acq - AO> video cam=acq, int_time=5000000, interval=1 - IDL> ao_focus_loop, current-600, 300, 5, ‘acq’, ‘save_name’, ‘sky_name’, time=6.0 - IDL> ao_read_focus, data,file=’save_name’ - AO> move acq_z best_focus - Focus LLT - IDL> ao_focus_loop, current-100, 50, 5, ‘llt’, ‘save_name’, ‘sky_name’, time=6.0 - IDL> ao_read_focus, data,file=’save_name’ - CSH> bto_control “move llt_focus XXX” - Repeat steps 3.2 and 3.3 again if necessary - Tune center wavelength - This depends…. - Check position of laser_focus - This depends - Take photometry - Record and save a final set of detuned and tuned 5s Acq images for measuring photometry and spot size. - Take NGS photometric - Go to photometric standard NGS, Landolt ~mV=9.5 - Move acq_z to NGS focus - Move star to approximate position of LGS - Take two images, with a ~10” dither between them - Set HOWFS z stage - start lgsfoc in IDL, set Height to 90km, send HWFS stage by clicking 1-shot, WFS stage should move to 80,840um - Move LGS onto HOWFS. See step 2. - Close UTT loop.

Results and conclusions