Palomar Adaptive Optics Test Plan

| Title | LGS Acquisition and characterization |
| --- | --- |
| Date | 7/12/06 |
| Lead | A. Bouchez, M. Troy |
| Time requested | 1 hr |
| Required conditions | Clear enough to project laser. |

Purpose Acquire LGS, focus LLT, determine LGS magnitude and spot size,

Test procedure - Setup - Telescope at zenith, at best NGS focus. - Acquisition camera filter installed. - Move LLT mirror to a its default position (determined during LLT alignment) - Load best available flatmap. - Focus acqisition camera to 90 km altitude: move acq_z 11950 (for NGS focus = 14500) - Focus LLT to 90 km altitude: bto_control "move llt_focus 11300" (for NGS focus = 11600) - Check that laser focus is set to optimal position (4000 on 7/12/06): bto_control "move laser_focus 4000" - Setup acquisition camera: - Start up IDL program acqview - Set integration time to 2s. - In the correct experiment directory: IDL> ao_plot_vid_image - Perform final safety checks and fire laser. - Acquire LGS - If LGS is not in Acq FOV, use ellipticity of dichroic spots (apex points to LGS) and Raleigh gradient (brighter towards LGS) to steer it in: offset llt_a +X = down; offset llt_b +X = right. - Move LGS onto HOWFS by clicking on "center LGS". - Move LGS to clear region of Acq field: offset llt_b + 30 - Focus on Na layer - Roughly focus laser spot: bto_control "move llt_focus ..." in steps of 50 move acq_z ... in steps of 300 - Detune laser, save a 5s background image. - Focus Acq IDL> ao_focus_loop, current-600, 300, 5, ‘acq’, ‘save_name’, ‘sky_name’, time=6.0 IDL> ao_read_focus, data,file=’save_name’ AO> move acq_z best_focus - Focus LLT IDL> ao_focus_loop, current-100, 50, 5, ‘llt’, ‘save_name’, ‘sky_name’, time=6.0 IDL> ao_read_focus, data,file=’save_name’ AO> bto_control “move llt_focus XXX” - Repeat steps 3.2 and 3.3 again if necessary - Tune center wavelength - Save a 5s background image. - Scan over wavelength, recording peak counts. - Adjust to peak return. - Record photometry - Record and save a final set of detuned and tuned 5s Acq images for measuring photometry and spot size. - Image photometric calibrator - Go to photometric standard NGS, Landolt ~mV=11.0 - Move acq_z to NGS focus - Move star to approximate position of LGS - Take two images, with a ~10” dither between them

Results and conclusions