Palomar Adaptive Optics Test Plan

| Title | LGS and NGS acquisition and lock |
| --- | --- |
| Date | 9/4/06 |
| Lead | M. Troy |
| Time requested | - |
| Required conditions | - |

Purpose Acquire NGS and LGS, Lock all loops and be ready to perform science at this end of this procedure

Test procedure - Setup - This procedure assumes the LGS acquisition and characterization procedure has been executed - Load co_zero centroid offsets for HOWFS - Focus acquisition camera to NGS, ‘move acq_z 14500’ - Focus LLT to 90 km altitude,’ bto_control "move llt_focus 11300’ - Check that laser focus is set to optimal position (4000 on 7/12/06), ‘bto_control "move laser_focus 4000’ - Set chopper delay and gate on generator using PC - delay value = 32000 - Gate value = 3998 - Move Telescope to NGS (follow BTO slew procedure) - Lock AO on white light and register DM pupil - Move telescope to put NGS on HOWFS - Lock on NGS star (if a NGS lock has not been done in last ~1hr and star is bright enough) - Lock TT and DM as normal and offload focus to telescope - Make new telescope flat map and load - Pre-pare for LOWFS lock - Using TAO select LGS mode (Under the Observation menu) - Focus acquisition camera to LGS, ‘move acq_z 11900’ - Propagate laser - Center Laser on reflecting spot, using acqview (center LGS button) - Set HOWFS z stage - start lgsfoc in IDL, set Height to 90km, send HWFS stage by clicking 1-shot, WFS stage should move to 80,840um - Lock LOWFS - Focus acquisition camera to NGS, ‘move acq_z 14500’ - Start up LOWFS plots, ‘ao_plot_lo_gui’ in IDL - Move LOWFS to star using acqview (send LOWFS button) - Set LOWFS camera rate, ‘wfs_cam id=lo,rate=XXX’ - Take sky - Move telescope 60” E - From TAO menu take LOWFS sky CCC - Move telescope back - Startup pixel display, ‘ao_display_latest_lo_pixel’ in IDL - Lock, TTM using button on TAO - Check lock performance in plots - Lock HOWFS on LGS - Start up LGS IDL plots (ao_plot_gui_logs_hwfp) - Start Chopper - Using Terminal window on PC - Set to external reference - Set to run - Set WFS camera to 100Hz - Take sky - offset llt_a 20 arcseconds - From TAO menu take HOWFS sky - offset llt_a -20 arcseconds - set log interval to avoid late messages, ‘log wfp=ho, interval=10’ - Lock LLT - Turn on camera to check flux, ‘wfs_cam on id=ho’ - Type “llt on” - Check performance in GUIs - Lock DM - Click on DM button on TAO - Start focus offloading from LOWFS - Need to write this - Tweak Performance - Register DM? - Chopper versus none - subap min flux - Centroid offsets - Servo loop gains

Results and conclusions