Palomar Adaptive Optics Test Plan

| Title | LOWFS Perfomance and Optimization |
| --- | --- |
| Date | 10/11/06 |
| Lead | J. Roberts |
| Time requested | 3 hours |
| Required conditions | Nighttime, NGS |

Purpose To look at the closed loop bandwidth of the LOWFS at various frame rates To optimize the servo gains for the LOWFS

Previous analysis Wavefrot Error Budget LOWFS Calculated optimal frame rates for various magnitudes Magnitude; Frame Rate; Flux; Predicted Bandwidth. 8th; 793; 11328; 31. 10th; 419; 3397; 17. 12th; 211; 1069; 8.4. 14th; 96; 372; 3.8. 16th; 38; 150; 1.5. 18th; 13; 69; 0.5. * saturated!

Test procedure - Depending on the sky brightness, acquire either an 18th mag star or a 10th mag star in the LOWFS (first center star on dichroic spot). - Set log wfp=lo, data=all - Set the frame rate to the first listed in Table 1 for this magnitude star. Check that the flux is approximately what is expected. If not, adjust the frame rate. - Steer off and take a LOWFS sky from TAO. Take a PHARO sky as well. - Close TT loop - Set the log interval as given in Table 2. - Turn on the LOWFS and take data for approximately 30sec. Take a PHARO image. - Set the TT servo gain to the first value in Table 3 for this star. - Lock the LOWFS on the star and take 30sec of data and take a PHARO image. - Continue to change the gains. Take 30sec of data and a PHARO image for each. - Open the loop and repeat steps 3-9 for the rest of the frame rates (take PHARO skys as needed) - Repeat steps 1-10 on each of the following magnitudes: 14th, 12th, 16th, (10th or 18th) Table 1 Magnitude; Rate 1; Flux 1; Rate 2; Flux 2; Rate 3; Flux 3; Rate 4; Flux 4. 10th; 200; 7123; 400; 3561; 500; 2849; 700; 2035. 12th; 100; 2258; 200; 1129; 300; 753; 400; 564. 14th; 50; 715; 100; 358; 150; 239; 200; 179. 16th; 30; 189; 50; 113; 100; 56; 150; 38. 18th; 30; 30. Table 2 Frame Rate; 30-400; 500-800; 900-1000; 1250-1500. Log Interval; 1; 2; 3; 4. Table 3 Frame Rate; Gain 1; Gain 2; Gain 3; Gain 4. 30-100; 0.2; 0.4; 0.8; 1.2. 150-300; 0.2; 0.6; 1; 1.4. 400; 0.2; 0.6; 1; 1.2. 500; 0.15; 0.3; 0.6; 0.9. 700; 0.1; 0.2; 0.4; 0.8.

Results and conclusions