Palomar Adaptive Optics Test Plan

| Title | Laser, BTO, and safety system checkout |
| --- | --- |
| Date | 12/07/06 |
| Lead | A. Bouchez |
| Time requested | 1 hr |
| Required conditions | Dome closed |

Purpose - Align 589nm laser to BTO bench in Coude lab. - Verify that 589nm laser is aligned to BTO in dome. - Measure transmission of BTO, optimize laser focus (first night of run only). - Test BTO servo loop and Q3 safety interlock

Test procedure Personnel required: Data room coordinator (lead), laser engineer, BTO operator, 2x dome personnel (first night only) Setup: - 660nm laser must previously have been aligned to BTO, and through LLT (procedure XXX, performed on first afternoon of run). - Test BTO alignment by restoring zenith defaults, inserting 660nm laser, closing BTO loops on the laser, then opening servo loops. - LLT primary mirror cover removed and LLT ready for high power laser (visually check diagnostics bench, cables). - Zone 8 of safety system jumpered out for in-dome propagation. - Laser at low power (~50 mw) In Coude lab: - Install Coude block. - Partially close both polar axis alignment irises. - Cautiously open shutter. - Adjust final laser mirror to center beam on M1a. - Adjust M1a to center beam on second iris. - Adjust final laser mirror and M1a to center beam on both irises. - Insert 660nm fold mirror and verify that alignment to irises appears identical. Repeat previous steps if necessary. - Close shutter. In dome: (steps followed first night only are in red) - Remove coude block. - At low laser power with personnel in dome, open shutter and verify that BTO alignment looks good (check beam pattern on ceiling.) - Shutter beam, increase to high power. - Measure laser power at table output with thermal sensor. - Pass power meter to personnel in dome. - Open the shutter, close BTO loops on Q3. Verify loop stability. Save BTO DEFAULT and zenith positions. - Shutter beam, send personnel to prime focus with power meter. - Open shutter, close BTO servo loop - Measure laser power at entrance to prime focus. - Measure laser power on LLT optical axis (before secondary). - Visually estimate spot size on LLT primary. Adjust laser focus stage to minimize spot on primary. - Shutter, return personnel to control room. - Measure laser power again. - Enable Q3 safety system. - Test-fire laser at zenith in dome. - Test-fire laser with block or power meter in beam to demonstrate Q3 interlock functionality.

Results and conclusions