Palomar Adaptive Optics Test Plan

| Title | Internal Focusing for LGS operations |
| --- | --- |
| Date | 12/1/2006 |
| Lead | M. Troy |
| Time requested | 2 hr |
| Required conditions | - |

Purpose We outline here the procedure to co-focus the LOWFS, HOWFS and PHARO for night time operation. All work is done with the white light.

Test procedure - Setup - Reflecting spot should be in Laser mode? - LOWFS - load zero offsets - Set rate to 500Hz - HOWFS - load zero offsets - Set rate to 200 Hz - Set log interval to 20 - Turn WFS on - Acquire white light in HOWFS with power at 40 - Make sure LOWFS is off - Take of load a white light flat map - co-focus LOWFS and HOWFS to white light - Lock in dual mode (see procedure XXX) - zero LOWFS focus using wfs_z - start, lgsfoc, set the LOWFS zero point to zero, set gain to 0.25 - Turn lgsfoc “on” - Note: insure HOWFS residuals show no focus, may need to decrease DM proportional servo loop term. - note the old and new wfs_z position - turn off lgsfoc - re-focus white light to zero focus on DM (optional) - close HOWFS loops on white light - Setup TAO to offload focus to telescope, but prompt user (1mm of secondary motion = 22.9 mm of white_z (or wfs_ motion) - Use the TAO prompts to move the ‘white_z’ motor to zero the focus on the DM. - Note old and new white_z position - Make new white light flat map - Make DM flat map (ao_make_flat_map,’flat_map_white_light_<date>’ - Open TTM and DM loops - Re-focus acquisition camera - move LOWFS out of the way - Set the video camera to run at 1Hz, saving 1 frame a second - Check to make sure the peak counts are less then ~100. If necessary decrease the white light power - Shut off white light and take and save a background using ao_plot_vid_image in IDL - Turn the white light back on and focus using - ao_focus_loop, ‘current acq_z position’ -600, 300, 5, ‘acq ,’acq_wl_focus’,’sky name from step 4.3’,’time_to_avg=2 - ao_read_focus,data,file=’acq_wl_focus_focus.dat’ - move acq_z to optimal position and note new position - Save new AO config file - PHARO alignment - re-focus PHARO - lock HOWFS on zero offsets - focus PHARO to maximize Strehl/minimize FWHM. - re-tune centroid offsets in HOWFS

Results and conclusions 9/7/2006 wfs_z: starting position:10659 um, ending position: 7150 um white_z: starting position: 27500 um, ending position: 30917 um saved as 060908a did not re-focus acq camera, procedure above was not written did re-focus and tune up PHARO 10/11/2006 wfs_z: starting position:10650 um, ending position: 7260 um white_z: starting position: 27500 um, ending position: 29792 um acq_z: starting position: 14600 um, ending position: 14740 um did re-focus and tune up PHARO