Palomar Adaptive Optics Test Plan

| Title | LGS Acquisition and characterization |
| --- | --- |
| Date | 12/06/06, V4.2 |
| Lead | A. Bouchez,M. Troy |
| Time requested | 1 hr |
| Required conditions | Clear. |

Purpose Acquire LGS, focus LLT, determine LGS magnitude and spot size,

Test procedure - Setup - Insure laser operator has started execution of procedure for preparation of laser propagation - Configure AO to look at sky - Configure telescope for sky (open mirror cover, etc) – zenith only - Telescope at zenith, at best NGS focus. - Acquisition camera V filter not installed. - Move LLT mirror to its default position (determined during LLT alignment) [-11,53] - Load best available flatmap - Check white light position on HOWFS, - Move SSMs to center if necessary, then move back to sky - Acq focused at 90 km altitude (NGS focus-2450) move acq_z 12350 (for NGS use 14600) - LLT focused to 90 km altitude (NGS focus - 300). bto move llt_focus 11730 - check/set laser focus to correct value bto move laser_focus 9000 - Setup acquisition camera: - Start up IDL program acqview - Set integration time to 2s - In an appropriate experiment directory: IDL> ao_plot_vid_image - Perform final safety checks and propagate laser - Acquire LGS on Acq - If LGS is not in the Acq. FOV, use ellipticity of dichroic spots (apex points to LGS) and Raleigh gradient (brighter towards LGS) to steer it in: offset llt_a +X=up; offset llt_b +X=left - Calibrate UTT mirror throw (one-time calibration) - Center LGS in acquisition camera field. - Record image with ACQVIEW and note time tag in log in Results section. - Record images dithering laser in a cross pattern (see log). - Manually center laser behind reflective spot. - Focus on Na layer - Move LGS to clear region of ACQ. field: offset llt_b +30 - Rough focus laser spot: bto offset llt_focus …” in steps of 100 move acq_z … in steps of 300 - Block laser, take and save a 2s background image. - Focus Acq - ao_focus_loop, current-600, 300, 5, ‘acq’, ‘save_name’, ‘sky_name’, time=6.0 - ao_read_focus, data,file=’save_name’ - move acq_z best_focus - Focus LLT - ao_focus_loop, current-100, 50, 5, ‘llt’, ‘save_name’, ‘sky_name’, time=6.0 - ao_read_focus, data,file=’save_name’ - bto move llt_focus XXX - Repeat steps 4.3 and 4.4 again if necessary - Determine laser focus - Rough focus laser spot, looking at image in acq. camera bto_control “offset laser_focus …” in steps of 2000 - ao_focus_loop, 0, 2000, 7, ‘laser_focus’,’save_name,’sky_name’,time=6.0 - ao_read_focus,data,file=’save_name’,/total_flux - bto move laser_focus XXX - Take photometry - Install V filter in Acq. camera - Record and save a final set of detuned and tuned 2s Acq images for measuring photometry and spot size. offset llt_b 10 between images. - Image photometric calibrator - Go to photometric standard NGS, Landolt ~mV=11.0 - Move acq_z to NGS focus (move acq_z 14600) - Move star to approximate position of LGS - Take two images, with a ~10” dither between them; record star name and image time tags. - Remove V filter from Acq. camera

Results and conclusions UTT mirror calibration (section 4) Mirror commands; llt_a position; llt_b position; Image time tag; x centroid (pix); y centroid (pix). centered; -19; 19.9; 1165393512; 468.3; 267.3. offset llt_a 20; 0.9; 19.9; 1165393596; 455.2; 380.8. offset llt_a -40; 0.9; 59.9; 1165393656; 271.7; 357.3. offset llt_a 20; -19; 59.9; 1165393694; 285.7; 238.1. Optimal focus settings (section 5) UT Date; Dec 6 2006; Dec 7 2006. LLT focus (µm); 11730; 11730. Acq_z (µm); 12350; 12246. LGS FWHM (pix); 11.5 (!); 13.5. Laser Focus (µm); 7000; 9000. Photometric calibration (section 7) UT Date; Dec 6 2006; Dec 7 2006. Int. time (secs); 5. Laser Acq. filenames; laser5. laser6. Star ID; Landolt 98-185; Landolt 95_149. Image time tags; landolt98-185_1; 95_149_1. landolt98-185_2; 95_149_2. Star FWHM (pix); 10; 16.5.

Notes:

Dec 6, 2006: Fainter star of the pair (mV=10.54) – 2s exposures

Dec 7, 2006: At end of test laser spot was ~20 pixels (versus 13.5 at the start) so it is clear the seeing degraded during the test.