Palomar Adaptive Optics Test Plan

| Title | Rayleigh Scatter in LOWFS |
| --- | --- |
| Date | 9/7/06 |
| Lead | J. Roberts |
| Time requested | 1 hour |
| Required conditions | Nighttime; LGS |

Purpose To determine the effect of Rayleigh scatter in the LOWFS without a filter and using the dichroic as a filter

Previous analysis None

Test procedure - Propagate laser - Acquire laser in HOWFS - Send the LOWFS to a location away from the dichroic spot - With the laser propagating, take a LOWFS background and save it to a data directory - Shutter the laser and repeat the background - Send the LOWFS to a location behind the reflecting spot - Repeat steps 4-5 - Move the laser off the dichroic spot by 20” (offset llt_b 20) - Repeat steps 4-5

Results and conclusions