Palomar Adaptive Optics Test Plan

| Title | LGS and NGS acquisition and lock |
| --- | --- |
| Date | 12/1/06 |
| Lead | M. Troy |
| Time requested | - |
| Required conditions | - |

Purpose Acquire NGS and LGS, Lock all loops and be ready to perform science at this end of this procedure

Test procedure - Setup - This procedure assumes the LGS acquisition and characterization procedure has been executed - Load co_zero centroid offsets for HOWFS - Set servo loop gains - llt_integral_gain 1.6 - ttm_integral_gain 0.15 - dm_integral_gain 0.15 - dm_porportional_gain 0.02 - Focus acquisition camera to NGS, ‘move acq_z 14740’ - Set chopper delay and gate on generator using PC - delay value = 58400 - Gate value = 3998 - Move Telescope to a bright star near the TT NGS (follow BTO slew procedure) - Move telescope to put NGS on HOWFS - Lock on NGS star - Register DM - Lock TT and DM as normal and offload focus to telescope - Make new telescope flat map and load - Pre-pare for LOWFS lock - Using TAO select LGS mode (Under the Observation menu) - Focus acquisition camera to LGS, ‘move acq_z 12300’ - Propagate laser - Center Laser on reflecting spot, using acqview (center LGS button) - Set HOWFS z stage - start lgsfoc in IDL, set Height to 90km, send HWFS stage by clicking 1-shot, WFS stage should move to 80,840um - Lock LOWFS - Focus acquisition camera to NGS, ‘move acq_z 14740’ - Start up LOWFS plots, ‘ao_plot_lo_gui’ in IDL - Move LOWFS to star using acqview (send LOWFS button) - Set LOWFS camera rate, ‘wfs_cam id=lo,rate=XXX’,gain=0 - Take sky - Move telescope 60” E - From TAO menu take LOWFS sky CCC - Move telescope back - re-start LOWFS, wfs_cam on, id=lo - Log all data, log wfp=lo, data=all - Startup pixel display, ‘ao_display_latest_lo_pixel’ in IDL - Zero LOWFS centroids, using acqview zero_lo_cent button - Lock, TTM using button on TAO - Check lock performance in plots - Lock HOWFS on LGS - Start up LGS IDL plots (ao_plot_gui_lgs_hwfp) - Start Chopper - Using Terminal window on PC - Set to external reference - Set to run - Set WFS camera to 100Hz - Take sky - offset llt_a 20 arcseconds - From TAO menu take HOWFS sky - offset llt_a -20 arcseconds - turn howfs on, wfs_cam on, id=ho - set log interval to avoid late messages, ‘log wfp=ho, interval=10’ - Lock LLT - Type “llt on” - Check performance in GUIs - Lock DM - Click on DM button on TAO - Start focus offloading from LOWFS - Need to write this - Tweak Performance - HOWFS rate - Register DM? - Chopper versus none - subap min flux - Centroid offsets - Servo loop gains - reconstructors

Results and conclusions