Palomar Adaptive Optics Test Plan

| Title | LGS and NGS acquisition and lock |
| --- | --- |
| Date | 1/1/2007 V3.1 |
| Lead | M. Troy |
| Time requested | - |
| Required conditions | - |

Purpose Acquire NGS and LGS, Lock all loops and be ready to perform science at this end of this procedure

Test procedure - Setup - This procedure assumes the LGS acquisition and characterization procedure has been executed - Load co_zero centroid offsets for HOWFS - Set servo loop gains - llt_integral_gain 1.6 - dm_proportional_gain 0.02 - Focus acquisition camera to NGS, ‘move acq_z 14600’ - Set chopper delay and gate on generator using PC - delay value = 58400 - Gate value = 3998 - Stop Chopper - Move Telescope to a bright star near the TT NGS (follow BTO slew procedure) - Move telescope to put NGS on HOWFS, insure WFS_Z is at NGS position - Lock on NGS star - Register DM - Lock TT and DM as normal and offload focus to telescope - Make new telescope flat map and load - Take 30 second PHARO exposure with TT closed to get seeing, note image number and FWHM - Move telescope back to TT NGS - ttm_integral_gain 0.15 - dm_integral_gain 0.15 - Pre-pare for LOWFS lock - Using TAO select LGS mode (Under the Observation menu) - Set HOWFS camera frame rate for LGS - Focus acquisition camera to LGS, ‘move acq_z 12350’ - Propagate laser - Verify BTO HS lock on Q3 - Center Laser on reflecting spot, using acqview (center LGS button) Offset llt_a +x moves LGS up, llt_b +x moves LGS left - Set HOWFS z stage - start lgsfoc in IDL, set Height to 90km, send HWFS stage by clicking 1-shot, WFS stage should move to 80,840um - Start chopper running - Find laser in HOWFS to verify location and approximate frame rate - Lock LOWFS - Focus acquisition camera to NGS, ‘move acq_z 14600’ - Start up LOWFS plots, ‘ao_plot_lo_gui’ in IDL - Move LOWFS to star using acqview (send LOWFS button) - Set LOWFS camera rate, ‘wfs_cam id=lo,rate=XXX’,gain=0 - Take sky - Move telescope 60” E - From TAO menu take LOWFS sky CCC - Move telescope back - re-start LOWFS, wfs_cam on, id=lo - Log all data, log wfp=lo, data=all - Startup pixel display, ‘ao_display_latest_lo_pixel’ in IDL - Zero LOWFS centroids, using acqview zero_lo_cent button - Lock, TTM using button on TAO - Check lock performance in plots - Lock HOWFS on LGS - Start Chopper - Using terminal window on PC - Set to external reference - Set to run - Check chopper settings - Startup aomonitor in idl to check average counts - Startup connection to delay generator (see setup procedure) - Detune laser - Adjust delay on chopper (channel 3) to see no Rayleigh. Find the limit where Rayleigh is just visible. - Retune laser - Adjust delay to see maximum signal. Find the limit where signal starts to drop off. - Select a delay time between the two limits - Set WFS camera to best estimate for framerate (100Hz) - Take sky - offset llt_a 20 arcseconds - From TAO menu take HOWFS sky - offset llt_a -20 arcseconds - turn howfs on, wfs_cam on, id=ho - set log interval to avoid late messages, ‘log wfp=ho, interval=10’ - Start up LGS IDL plots (ao_plot_gui_lgs_hwfp) - Lock LLT - Type “llt on” - Check performance in GUIs - Lock DM - Set TAO to prompt to offload focus - Click on DM button on TAO - Start focus offloading from LOWFS - In /data/scratch start idl and run “lgsfoc” - Verify LOWFS focus offset is 0 - With LOWFS locked on TT and the DM loop locked on the laser, set the loop to On.

Results and conclusions 12/8/06 UT 3:39 start procedure Take flatmap_sky_061207 Pharo image #14, seeing ~1.5 at V Getting ~120 counts in WFS at 100Hz, LLT locked 4:06 try and lock DM with gain of 0.15, unstable Dm gain 0.05 200Hz counts at 100Hz DM gain of 0.15 getting 0.5” FWHM at K 4:20 procedure ended