Palomar Adaptive Optics Test Plan

| Title | Laser, BTO, and safety system checkout |
| --- | --- |
| Version | 2.0 |
| Date released | 4/5/07 |
| Lead | A. Bouchez |
| Time requested | 30 min. |
| Required conditions | Dome closed |

Purpose - Align 589nm laser to BTO bench in Coude lab. - Verify that 589nm laser is aligned to BTO in dome. - Measure transmission of BTO, optimize laser focus (first night of run only). - Test BTO servo loop and Q3 safety interlock

Test procedure Personnel required: Data room coordinator (lead), laser engineer, BTO operator, 2x dome personnel (first night only) Setup: - LLT primary mirror cover removed and LLT ready for high power laser (visually check diagnostics bench, cables). - Coude block installed. - Zone 8 of safety system jumpered out for in-dome propagation. - BTO motors in default zenith position for 660nm stimulus laser. - Switch BTO to 589nm laser position. In Coude lab: - Adjust laser for Very low power (~50 mw) - Partially close both polar axis alignment irises. - Cautiously open shutter. - Adjust final laser mirror to center beam on first iris. - Adjust M1a to center beam on second iris. - Insert 660nm fold mirror and verify that alignment to irises appears identical. Repeat previous steps if necessary. - Close shutter. - Adjust laser for Moderate power (~500 mw) In dome: - Remove coude block. - At low laser power with personnel in dome, open shutter and verify that laser is coming through LLT (check pattern on the ceiling). - If not acceptable, attempt to close BTO loops on Q1, then Q1+Q3. - If still not aligned, restart procedure at step 6. - Shutter beam. - Adjust laser to High power (>5W). Record laser power below. - Open the shutter. If laser does not fall on Q3, block beam immediately! - Set threshold values to allow Q3 servo lock. (set good_flux = 0.5) - Close servo on all quad cells. - Reset threshold values (set good_flux = 0.5) - Set Zenith and save BTO DEFAULT positions. - Shutter beam and bring personnel back into data room. - Enable Q3 safety system. - Test-fire laser at zenith in dome. - Install Coude block. - Test-fire laser to demonstrate Q3 interlock functionality.

Results and conclusions