Palomar Adaptive Optics Test Plan

| Title | Laser optimization at zenith |
| --- | --- |
| Date | 4/2/2007 V1 |
| Lead | j. roberts |
| Time requested | - |
| Required conditions | - |

Purpose Perform any optimizations on the laser that can be done at zenith

Test procedure - Go to LGS mode - Move lgs_x to lgs position (92000) - Lock HOWFS on LGS - Acquire laser on HOWFS - Start Chopper - Using terminal window on PC - Set to external reference - Set to run - Take a background - Lock UTT and DM - UTT gains - With loops locked, note UTT residuals at UTT gains of 0.2, 0.4, 0.8, 1.6, 3.2 - Set gain to “optimal” gain - Chopper - Check chopper settings - Startup aomonitor in idl to check average counts - Startup connection to delay generator (see setup procedure) - Detune laser - Increase delay on chopper (channel 3) to see no Rayleigh. Decrease the delay on the chopper to find the limit where Rayleigh is just visible. - Retune laser - Increase the delay to see maximum signal. Decrease the delay to find the limit where signal starts to drop off. - Select a delay time between the two limits

Results and conclusions