Palomar Adaptive Optics Test Plan

| Title | Laser, BTO, and safety system checkout |
| --- | --- |
| Version | 2.4 |
| Date released | 9/18/07 |
| Lead | A. Bouchez |
| Time requested | 30 min. |
| Required conditions | Dome closed |

Purpose - Align 589nm laser to BTO bench in Coude lab. - Verify that 589nm laser is aligned to BTO in dome. - Measure transmission of BTO, optimize laser focus (first night of run only). - Test BTO servo loop and Q3 safety interlock

Test procedure Personnel required: Data room coordinator (lead), laser engineer, BTO operator, 2x dome personnel (first night only) Setup: - Dome closed. - LLT primary mirror cover removed and LLT ready for high power laser (visually check diagnostics bench, cables). - Remove Coude block. - BTO interlock jumpered out for in-dome propagation. - Set BTO trigger to “Laser” in BTO GUI. - Lock BTO servo on the stimulus laser: - Set Laser to “Stimulus” in BTO GUI. - Set all quad cells enabled and High-Speed to Q3 in BTO GUI. - “Track On” in BTO GUI. - Wait for “Track: completed.” message in BTO GUI message window. - “Servo On” in BTO GUI. - Verify BTO servo is locked with high-speed on Q3. - “Set Zenith and Save DEFAULT” in BTO GUI. - Servo Off in BTO GUI. - Set laser to 589nm in BTO GUI. - On BTO command line: "set laser_pulse_freq = <freq>" - Close Coude block. In Coude lab: - Adjust laser for Very low power (~50 mw) - Frequency lock off. - Phase lock off. - Dephase IR lasers using “phase control” knobs. - Partially close both polar axis alignment irises. - Remove power meter from beam. - Cautiously open shutter. - Check collimation of beam (visually or with shear plate). - Adjust final laser mirror to center beam on first iris. - Adjust M1a to center beam on second iris. - Set laser to stimulus in BTO GUI. - Verify that alignment to irises appears identical. Repeat previous steps if necessary. - Close shutter. - Open both irises. - Put in power meter. - Adjust laser for Low power (~500 mw). - Remove power meter. In dome: - Remove Coude block. - If it's been >20 min. since BTO was locked on stimulus, repeat steps 6-9. - Prepare BTO for in-dome propagation at low power: - Set 589 power to low in BTO GUI. - Set High-Speed to Q1 in BTO GUI. - Disable Q2 and Q3 in BTO GUI. - At low laser power with personnel in dome, open shutter and verify that laser is coming through LLT (check pattern on the ceiling). - Lock BTO on low-power 589nm laser: - Record Flux in BTO GUI. - Servo On in BTO GUI. - Verify BTO servo locked on Q1. - Record Flux in BTO GUI. - Enable Q3 in BTO GUI. - Verify BTO servo locked on Q3. - If still not aligned, restart procedure at step 11. - Set Zenith in BTO GUI - Turn servo off in BTO GUI. - Shutter beam. - Adjust laser to High power (>5W). Record laser power below. - Open the shutter. If laser does not fall on Q3, block beam immediately! - Lock BTO servo on high-power 589nm laser: - Set 589 Power to High in BTO GUI. - Enable Q2 in BTO GUI. - Adjust laser_pol to verify that light on Q2 is not a ghost (light should go up dramatically with +3 degree adjustment). Return to original laser_pol position. - Record flux in BTO GUI. - Servo On in BTO GUI. - Verify BTO Servo locked. If counts decrease on Q3, block beam immediately! - Record flux in BTO GUI. - Set High-Speed to Q3 in BTO GUI. - Set Zenith and save DEFAULT in BTO GUI. - Shutter beam and bring personnel back into data room. - Enable BTO Interlock. - Connect TAO to BTO. - Set HA/Dec to TAO in BTO GUI. - Set "Track ON" in BTO GUI. - Reset alarm in BTO GUI. - On TAO, verify that BTO reports "Ready". - Test-fire laser at zenith in dome. - Verify that TAO shows shutter open. - Shutter laser. - Disarm and install Coude block. - Test-fire laser to demonstrate BTO interlock functionality. - Reset alarm on BTO.