Palomar Adaptive Optics Test Plan

| Title | BTO shutdown |
| --- | --- |
| Version | 1.2 |
| Date released | 8/25/2007 |
| Lead | Laser Operator |
| Time requested | 30 min. at end of last night of an observing run. |
| Required conditions | N/A |

Purpose Shut down and protect optics and electronics of the BTO.

Test procedure Prime Focus Requires: Kimwipes, tape, compressed air, 0.05" allen key, radio. - Dust off LLT primary and install primary mirror cover. - Turn off LLT FSM controller (on/off switch on front of controller). - Dust off LLT FSM and install blue metal cover (stored in box in PF elevator, tiny screws require 0.05" allen key). - Wrap up Q3 beamsplitter with kimwipe and tape. - Wrap up LLT diagnostics beamplitter wedge with kimwipe and tape. Trolley Requires: Kimwipes, tape, compressed air. - Sent trolley to bottom of track (“Trolley down” button or move trolley 5100000). - Dust and cover trolley mirror with kimwipe and tape. - Dust and cover Q1 beamsplitter with kimwipe and tape. - Send trolley to storage position (“Trolley stow” button or move trolley 2650000). - Turn off BTO computer - In a new terminal window, ssh root@bto.palomar.caltech.edu (password is on AO system) - shutdown –h now - logout - Wait for computer to shutdown - Exit GUI - File > quit - Turn off all BTO electronics on network power switch: - telnet viswa_power - /off 1 - /off 2 - /off 3 - /off 4 - /off 7 - /x - Check that the status is off for everything Coude Lab Requires: compressed air, 0.05" allen key. - Turn off 660nm laser power supply (on/off button on front). - Turn off Newport FSM controller, ESP300 controller, and delay generator. - Dust off Coude FSM and Install blue metal cover (stored on top edge of bench). - Cover BTO optical bench. - Turn off tiny switches on backs of both microphones, and turn off microphone power supply. - Turn off video camera power supply (can be done by reaching between old optics above and to the right of the Coude bench).

Results and conclusions