Palomar Adaptive Optics Test Plan

| Title | Laser shut-down |
| --- | --- |
| Version | 1.0 |
| Date released | 8/12/2007 |
| Lead | V. Velur |
| Time requested | 10 minutes |
| Required conditions | N/A |

Purpose Safely shut down laser and ancillary equipment, both on a daily basis and at the end of an observing run.

Test procedure Daily shut-down procedure - Turn both mode locker “Power Adjust” knobs to 0. - Block both IR beams. - Adjust laser function generator to 10 Hz pulse rate (=1 on knob). - Press “STOP” on both Analogue Modules power supplies. - Turn off key switch on both power supplies. - Press blue button on laser controller (“Bypass Flow”). Verify that pressure and flow go to near zero. - Turn Na cell temperature controller off (hit “enable” to turn off). - Turn both chillers off (same button as turn-on). Additional long-term shut-down procedure - Turn off power meter. - Turn off all three photodiodes (1.06, 1.32, 1.06 diode). - Turn off oscilloscope. - Close laptop, turn off monitors. - Return second oscilloscope to E-lab. - Replace plexiglass lids on laser. Tape side should be on top. - Empty trash - Turn out lights.

Results and conclusions