Palomar Adaptive Optics Test Plan

| Title | LGS-AO Science Night Checklist |
| --- | --- |
| Version | 1.1 |
| Date released | 12/11/2008 |
| Lead | A. Bouchez |
| Time requested | N/A |
| Required conditions | N/A |

Purpose - Provide a schedule and checklist of science night tasks.

Procedure First night tasks indicated in blue. Before 4:00pm - AO bench LGS setup and checkout. - Cofocus AO wavefront sensors and science instrument. - Verify LOWFS-Acquisition camera mapping. Recalibrate if necessary. - Start up laser. Bring to full power without optimization to allow temperature to stabilize. - Perform first half of BTO setup procedure (no stimulus laser) Subsequent nights, start up BTO as follows: - ssh -Y aousr@198.202.125.152 - bto_ctrl - Uncover LLT primary mirror cover. After 4:00pm - Remove Coude block. - Test laser interlock system. - Perform second half of BTO setup procedure (using stimulus laser). - Check laser power. Optimize power and wavelength tuning. - Perform BTO checkout procedure. (Abbreviated checkout procedure on subsequent nights) Sunset - Perform NGS checkout. - Perform LGS checkout. - Optimize laser as necessary throughout night. Science Observations - Follow LGS Target Acquisition procedure. End-of-night - Close and lock Coude block. - Power down laser. - Install LLT primary mirror cover (Cover all BTO optics on final night)

Notes