Palomar Adaptive Optics Test Plan

| Title | Acquisition Camera Calibration |
| --- | --- |
| Version | 1.0 |
| Date released | 4/2/2007 |
| Lead | A. Bouchez |
| Time requested | 20 min. NGS, 20 min. LGS |
| Required conditions | Partly cloudy OK. |

Purpose Calibrate the Acquisition camera NGS and LGS plate scale and orientation.

Test procedure NGS: - Acquire a binary star with separation 10"-30", magnitudes <10 from the WDS Calibration Candidate catalogue: http://ad.usno.navy.mil/wds/orb6/orb6c.html. - Install HOWFS reflective spot, Acq fold mirror. - Center binary in Acq field of view. - Record 2 unsaturated images with the acquisition camera. - Acquire a second binary star and repeat steps 2-4. LGS: - Install HOWFS reflective spot, Acq fold mirror. - Center bright (V=10) NGS on PHARO. - Project laser. - Center laser in Acq (not on reflective spot). - Record Acq image of LGS and a PHARO image of the NGS (integrate long enough to get ~10k peak counts) - Offset telescope N, S, E, W in a cross pattern with a throw of 7.5". At each position, record an Acq image and a PHARO image.

Results and conclusions