Palomar Adaptive Optics Test Plan

| Title | LGS Target Acquisition |
| --- | --- |
| Version | 3.3 |
| Date released | 4/3/2007 |
| Lead | A. Bouchez |
| Time requested | 30 min. |
| Required conditions | N/A |

Purpose Acquire science target, lock all loops and be ready to perform science at this end of this procedure.

Test procedure - Tune up AO system on a nearby bright NGS. - Slew telescope to a V=7 star near the LGS target. - Setup AO system for NGS. - Set TAO mode to NGS. - Load co_zero centroid offsets. - Move LGS_Z to NGS position. - Move ACQ_Z to NGS position. - Move WFS_Z to NGS position. - Set Acq to video (installs fold mirror). - Stop chopper if running. - Acquire star on the HOWFS. - Close TT loop. - Register DM (coarse and fine). - Close DM and check that focus offload is allowed. - Make a new telescope flat map and load. - Optional: Record a 30 second PHARO exposure with only TT loop closed to measure seeing. - Slew telescope to coordinates of LGS target and center target on PHARO field. - Center target on HOWFS sweet spot. - Acquire the LGS. - Setup AO system for LGS. - Set TAO mode to LGS. - Set HOWFS frame rate to 150 Hz (or previous optimized value). - Set llt_integral gain 1.6 (or previous optimized value). - Set dm_proportional_gain 0.02 - Set dm_integral_gain 0.15 - Leave LGS_X in NGS position. - Move WFS_Z to LGS position (90km or best estimate). - Move ACQ_Z to LGS position (best estimate from focus runs). - Set Acq to integrate 2s. - Start chopper. - Project laser. - Center laser on HOWFS reflective spot. (Offset llt_a +x moves LGS up, llt_b +x moves LGS left) - Close the DM loop on the laser. - Move LGS_X to LGS position. - Take a HOWFS background (offset llt_a 20") - Close UTT loop. - Verify HOWFS framerate is correct. If necessary: - Open UTT loop. - Change HOWFS framerate. - Take a new HOWFS background (offset llt_a 20"). - Return UTT mirror to center and close UTT loop. - Close DM loop. - Acquire the NGS and close TT loop. - Setup LOWFS - Set ttm_integral_gain 0.15 - Set LOWFS framerate to expected value for NGS. - Move ACQ_Z to NGS position. - Set Acq to integration time appropriate for NGS. - Identify NGS in Acq field and send LOWFS using ACQVIEW button. - Take a LOWFS background (offset telescope 60"). - If LOWFS residuals are >0.2, zero LOWFS centroids. - Close TTM loop. - Verify that LOWFS framerate is correct. If necessary: - Open TTM loop. - Change LOWFS framerate. - Take a new LOWFS background (offset telescope 60"). - Return telescope to center and close TTM loop. - Close focus loop (pulldown on LGSFOC). - Close focus loop on LGSFOC tool. - Wait for focus to converge to <0.5 mm. - Acquisition is complete!

Results and conclusions