Palomar Adaptive Optics Test Plan

| Title | Field-dependent LOWFS focus |
| --- | --- |
| Version | 1.0 |
| Date released | 4/3/2007 |
| Lead | A. Bouchez |
| Time requested | 30 minutes |
| Required conditions | Any |

Purpose Measure the focus as a function of field angle (aka. field curvature) at the LOWFS focus.

Previous analysis Lab tests performed on 3/26/07 using the stimulus gave conflicting results. LOWFS focus offset from the HOWFS "sweet spot" to pixel (490,345) was +1.50 mm when using the white_x motor to offset the fiber, and -0.60 mm when using SSMs to offset.

Test procedure - Setup - Center SSMs. - Move LGS_X to LGS position. - Set Acq to Video. - Set LOWFS framerate to 500 Hz. (or optimum) - Set ttm_integral_gain to something reasonable. - Acquire a bright (V~10.0) star near zenith. Center on HOWFS sweet spot: (282,262) - Acquire star with LOWFS. - Send LOWFS to star position. - Offset to sky and take a LOWFS background. - Return to star and zero LOWFS centroids. - Close TTM loop. - Record average LOWFS focus. - Offset telescope and reacquire star - Open TTM loop. - Set Acq to Video. - Offset the telescope 20" West. - Repeat steps 3.1-3.5 - Repeat step 4 at 20" intervals over entire field. - Return to center and take a final focus measurement.

Results and conclusions X offset ("); Y offset ("); X (pix); Y (pix); LOWFS focus. 0.0; 0.0.