Palomar Adaptive Optics Test Plan

| Title | LOWFS Transfer Curves |
| --- | --- |
| Date | 4/2/2007 V1 |
| Lead | Jenny |
| Time requested | - |
| Required conditions | - |

Purpose Measure TT transfer curves for the LOWFS on the sky

- Acquire ~10th mag star in the HOWFS - Go to NGS mode - Move lgs_x to lgs position (92000) - Set HOWFS rate to 200Hz - Take HOWFS background - Lock TT loop - Acquire star in LOWFS - Start aomonitor to watch residuals - Set LOWFS rate to 1000Hz - Acquire the star in the LOWFS using acqview. - Open the TT loop and go to DNGS mode - Take a LO sky - Go back to NGS mode and relock TT on the HO - Verify that lwfp_recon_4subap is loaded and that pixel, lo_residual and centroid data are being recorded - Zero LO centroids - Move lowfs_x in steps of 50um up to a maximum of ± 350um or to where the residuals become too noisy - ± 350um should be equivalent to ± .9” which is the maximum field of the quad cell - Return lowfs_x to center and repeat the moves for lowfs_y

Results and conclusions Start 1175585588 Stop 1175586020 Start at 0, +lowfsx, 7 steps, -13 steps Start 1175586119 Stop 1175586560 Start at 0, +lowfs_y, 7 steps, -14 steps