Palomar Adaptive Optics Test Plan

| Title | PalmAO / Pharo Throughput and Zero Point Analysis |
| --- | --- |
| Version | 1.0 |
| Date released | 04/07/2008 |
| Lead | R. Burruss |
| Time requested | 30 minutes |
| Required conditions | Clear skies |

Purpose Determine the Zero Point of the PalAO/Pharo instrument and compare it to values obtained before the latest PalAO realignment and recoating of optics. Previous Zero Point values for Pharo are 24.7 in Ksh and 24.8 in J-band. (Zero Point = magnitude that produces 1 e-/sec)

Test procedure - Slew to an appropriate standard star. Suitable Near-IR standards for Pharo are listed in Persson et al, AJ 116:2475-2488, 1998 and are cataloged in the Palomar TCS as STD 9101 – STD 9188. - At minimum, 3 standard stars are needed. One for airmass near 1.0, one for airmass at 1.5, and one for airmass at 2.0. - Acquire the star with PalAO as normal, lock the TTM loop, but DO NOT lock the DM loop - Set Pharo to 25 mas mode and StdCross. Do not use an ND filter. - Take 3 images in each broadband filter (J, H, and Ksh) with exposure times that produce ~ 20,000 to 30,000 DN peak counts - Unlock the TTM loop, move off the star to take 3 background images with each filter - Keep a log that lists the image file name, peak counts, filter, Pharo settings, and airmass

Results and conclusions