PALM-3000 
Message Types Document

(Commands, Automations, Telemetry, and Status)

Version 0.4

Document Owner:  Rick Burruss

Contributions by:

J. Angione, A. Bouchez, S. Guiwits, D. Hale, J. Roberts, M. Troy, T. Truong

Revision Sheet

| Revision No. | Date | Revision Description |
| --- | --- | --- |
| 0.0 | 03/10/09 | Initial Document (RB) |
| 0.1 | 03/27/09 | First set of revisions (RB) |
| 0.4 | 05/18/09 | Forth set of revisions (RB) |

Contents

1	General information	5

1.1	Purpose	5

1.2	Acronyms and Abbreviations	5

1.3	Related Products	5

2	Statement of Need	6

2.1	Introduction	6

2.2	Message Types	6

2.3	Error Codes	7

2.4	Automations	7

3	Message Types Breakdown	7

3.1	Message Type : Commands	7

3.1.1	HWFP_cmd	7

3.1.2	HWFS_cmd	8

3.1.3	ACAM_cmd	8

3.1.4	HODM_cmd	8

3.1.5	MOTOR_cmd	9

3.1.6	WLIGHT_cmd	9

3.1.7	TCS_cmd	9

3.1.8	PHARO_cmd	9

3.1.9	INST_cmd	10

3.1.10	Automation Commands	10

3.1.11	GET_cmd	10

3.1.12	SHUTDOWN_cmd	10

3.1.13	REFRESH_cmd	10

3.2	Message Type : Status	11

3.2.1	HWFP_stat	11

3.2.2	HWFS_stat	11

3.2.3	ACAM_stat	12

3.2.4	HODM_stat	12

3.2.5	MOTOR_stat	13

3.2.6	WLIGHT_stat	13

3.2.7	TCS_stat	13

3.2.8	Afprogress_stat	14

3.3	Message Type : External Status (not published)	15

3.3.1	PHARO_stat	15

3.3.2	INST_stat	15

3.4	Message Type : Info Status	16

3.4.1	AODD_info	16

3.4.2	AOCA_info	17

3.4.3	GUI_info	17

3.5	Message Type : Telemetry	17

3.5.1	HWFP_data	17

3.5.2	ACAM_data	18

4	ERROR CODES	18

5	AUTOMATIONS	19

5.1.1	AF7 (TTM_OFFLOAD)	19

5.1.2	AF8 (FOC_OFFLOAD)	20

5.1.3	AF9 (STIM)	21

5.1.4	AF10 (BACKGROUND)	21

5.1.5	AF11 (LDM_REG)	23

5.1.6	AF12 (HDM_REG)	24

5.1.7	AF13 (LDM_FLAT)	25

5.1.8	AF14 (HDM_FLAT)	26

5.1.9	AF15 (HWFS_LENS)	27

5.1.10	AF16 (NGS_ACQ)	28

5.1.11	AF17 (NGS_SEEING)	29

5.1.12	AF18 (SAVE)	29

5.1.13	AF19 (RESTORE)	30

5.1.14	AF20 (RESET)	30

5.1.15	AF21 (STAR_SETUP)	31

5.1.16	AF22 (LOG_RATE)	32

5.1.17	AF23 (SHUT_ALL)	32

5.1.18	AF24 (FLEX)	32

5.1.19	AF25 (APRC)	33

6	FULL P3K COMMAND LIST	34

7	P3K Messaging TYPES TABLE	37

# General information

## Purpose

This document presents the Message Types for the PALM-3000 adaptive optics system.  It contains all of the message types to be used by the P3K external devices and subsystems.  The message types are separated into commands, status structures, and telemetry structures.  This document also details all of the automations and the automation algorithms used by the P3K instrument.

## Acronyms and Abbreviations

ACAM			Acquisition Camera component

AO			Adaptive Optics

AOCA			Adaptive Optics Command Dispatch and Automations component

AODB			Adaptive Optics Database components

AODP			Adaptive Optics Database Publisher

AODS			Adaptive Optics Database Subscriber

AODD			Adaptive Optics Device Driver component

DBFIND		Database Find

GUI			Graphical User Interface

HODM			High Order Deformable Mirror

HWFP			High Order Wave Front Processor component

IDL			Interactive Data Language scripting language

LODM			Low Order Deformable Mirror

P3K			PALM-3000

PALAO		The original NGS AO system at Palomar commissioned in December 1999

PALM-3000		The visible light AO upgrade to PALMAO

PALMAO		Upgrades to PALAO, particularly after the April 2003 upgrade

TTM	Tip/tilt Mirror

## Related Products

- P3K Software Architecture Document; T. Truong

- P3K TWiki Pages at: http://www.oir.caltech.edu/twiki_oir/bin/view.cgi/Palomar/Palm3000/WebHome

# Statement of Need

## Introduction

The P3K control software messaging types, error codes, and automations need to be clearly defined in order to present the messaging design from a high level perspective to the development team.  A clear messaging design will help the P3K development meet the software requirements, preserve the software architecture, and clarify communication between components.

## Message Types

The primary function of defining message types is to distinguish and enumerate system level messaging for the P3K instrument.  P3K message types are divided into three main classes:

- Commands

  - Commands will be issued from external devices (GUI, Pharo, Science Instruments, IDL) over socket connections to the P3K Command Dispatcher (AOCA).  If the command exists in the automations command list, then the automation is started.  Otherwise the command is published for the appropriately subscribed sub-system components.  Components that receive their keyword command without parameters will publish their current status.

- Status Structures

  - Each of the “low level” components will publish a structure of status fields appropriate to its state.  The HWFP structure will also include all of the requested telemetry data.  The AODP component **may** extract the HWFP status information into a unique status structure, and then publish both the status and telemetry structures separately (see HWFP_stat).  If these structures are separated, then the status structure will only be published when a field changes.

- External Structures

  - These status structures will be maintained by AOCA and delivered to external components over a standard Tcp/IP socket connection

- Info Structures

  - Each of the “low level” components will publish a small status structure containing a short list of non-changing values unique to the component.  These information structures will only be published ONCE at system startup, and will contain useful information like firmware version, linux kernel version, and software version numbers.

- Telemetry Structures

  - The HWFP and ACAM components will publish telemetry data.  The HWFP structure will also include its status fields appropriate to its state. The AODP component **may** extract the HWFP telemetry data into a unique telemetry structure(s), and then publish both the status and telemetry structures separately (see HWFP_data).

## Error Codes

Error codes will be defined globally for errors common to the entire P3K system.  Each low level component (MOTOR, ACAM, etc) will also include error status and strings unique to that component in the interest of detail.  External devices such as the GUI may display some or all of these fields as appropriate, whereas all of the information will be published.  System health may be gleaned from error codes, and the system may respond to such cases, although this is not yet in any AOCA or other high level design.

## Automations

Automations are defined as a set of two or more primitive P3K commands.  An automation may also subscribe to and evaluate telemetry data, and execute various algorithms.  An automation may publish a status structure appropriate to its state.  Some automations require that the P3K system be blocked while running.  The AOCA component will block all external devices from connecting to it when an automation requires blocking.  Blocked automations may be canceled while they are running.  AOCA will monitor the state of all the automations (idle, running, or finished), and publish a status structure with this information.

The full list of automations and their parameters are listed in section 5 below.

# Message Types Breakdown

## Message Type : Commands

This is the list of the primitive commands that external devices such as the GUI and the Science Cameras, as well as external scripts such as IDL, will pass to the AOCA component over a socket connection.  The AOCA component will publish these primitive commands.

Command Syntax:

COMMAND <Required Parameter> {Optional Parameter}

### HWFP_cmd

These commands are published by AOCA and subscribed to by the High Order Wavefront Processor (HWFP) component.  They are intended to command the real time processes for P3K, such as tip / tilt, low and high order deformable mirror control, table loading, system telemetry logging, and real time parameter settings.

Complete HWFP Syntax:

HWFP TTM {ON|OFF} {MODE = 1 : i}*

HWFP LODM {ON|OFF} {MODE = 1 : i}*

HWFP HODM {ON|OFF} {MODE = 1 : i}*

HWFP LOAD <TABLE = filename>

<TABLE> = <CENT_OFFSETS | PIX_OFFETS | HO_DM_MAP | LO_DM_MAP | PIX_GAINS | RECONST>

HWFP SET <PARAM = float>

<PARAM> = <HODM_PRO_GAIN | HODM_INT_GAIN | LODM_PRO_GAIN | LODM_INT_GAIN | TTM_PRO_GAIN | TTM_INT_GAIN | SUBAP_MIN_FLUX | HODM_MODE | LODM_MODE>

HWFP LOG <DATA = telem> {INTERVAL = 1 : 500}

<telem> = <(PIXELS, CENTROIDS, SUBAP_FLUX, HODM_POS, LODM_POS, HODM_RES, LODM_RES, TTM_POS, TTM_RES, WFP_STAT) | NONE | ALL>

* MODE is not yet defined

### HWFS_cmd

These commands are published by AOCA and subscribed to by the Device Driver component (AODD) and the HWFP component (to know when HWFS in ON or OFF).  AODD then passes the command to the High Order Wave Front Sensor (HWFS) component, which controls a SciMeasure camera similar to the PalAO WFS camera.

Complete HWFS Syntax:

HWFS {ON|OFF} {RATE = 1 : ?} {BIAS = ?} {GAIN = ? : ?} {DELAY = ?}

{CLAMP = ?} {FUNCTION = 1:16} {OFFSET = ?} {INFO}

### ACAM_cmd

These commands are published by AOCA and subscribed to by the Device Driver component (AODD), which then passes the commands to the Acquisition Camera (ACAM) component, which controls an Imperx CCD Camera over a ??? interface.

Complete ACAM Syntax:

ACAM {ON|OFF} {INT_TIME = 0.0 : ?} {RATE = 1 : ?} {WINDOW = 1 (full 1x1) | 2 (full 2x2) | 3 (full 4x4) | 4 (acq 4x4) | 5 (pupil 2x2} {TEST = 0 | 1} {BACK_SUB = 0 | 1} {BACK_FILE = file_path} {INFO}

### HODM_cmd

These commands are published by AOCA and subscribed to by the Device Driver component (AODD), which then passes the commands to the High Order Deformable Mirror (3000 actuator DM) over the HODM “slow speed” serial interface.

Complete HODM Syntax:

HODM {ENABLE | DISABLE | RESET}

### MOTOR_cmd

These commands are published by AOCA and subscribed to by the Device Driver component (AODD), which then passes the commands on to the MOTOR component.

Complete MOTOR Syntax:

MOTOR   (request for MOTOR to publish its complete status)

MOVE <MOTOR> <POSITION> {VELOCITY = 0.0 : ?}

OFFSET <MOTOR | VIRTUAL_MOTOR> <OFFSET> {VELOCITY = 0.0 : ?}

HOME <MOTOR>

STOP <MOTOR>

<MOTOR> = <ACQ | FQ_RELAY | FQ_STIM | STIM | SSM1_A | SSM1_B | SSM2_A | SSM2_B | WFS_Z | WFS_LENS | WHITE_X | WHITE_Y | WHITE_Z | LASER_Z | TTM_A | TTM_B>

<VIRTUAL_MOTOR> = <IMAGE_X | IMAGE_Y | PUPIL_X | PUPIL_Y>

### WLIGHT_cmd

These commands are published by AOCA and subscribed to by the Device Driver component (AODD), which then passes the commands on to the WLIGHT component.

Complete WLIGHT Syntax:

WLIGHT {ON|OFF} {PWR = 0 : 100}

### TCS_cmd

These commands are passed from AOCA directly to the TCS through a socket connection opened by the AOCA “TCS CMD_task”.

Complete TCS Syntax:

TCS <TCS COMMAND>

### PHARO_cmd

These commands are passed from AOCA directly to PHARO through a NetServices socket connection wrapper, and opened by the AOCA “PHARO CMD_task”.  The NetServices wrapper will also parse incoming commands from PHARO into primitive AOCA commands.

Complete PHARO Syntax:

PHARO <PHARO COMMAND>

### INST_cmd

These commands are passed from AOCA directly to a science instrument through the AOCA socket connection.  This implementation will be delayed until a need is required and defined.

Complete INST Syntax:

INST <INST COMMAND>

### Automation Commands

These commands are passed from AOCA directly to the called Automation “AFn start_task”.  See section 5 for a complete list of these commands.

### GET_cmd

These commands may be issued from any non-GUI external component (ie: IDL).  AOCA will receive the command over a “commands” port, and the DBFIND component will send the requested data back through a “data” port.

Complete GET Syntax:

GET <COMPONENT> {START} {STOP}

### SHUTDOWN_cmd

This command is intended to gracefully shutdown the P3K software or one of the sub system processes.  SHUTDOWN with no arguments will close all processes.  (should we include a RESTART command to restart individual or collections of processes?  This also clarifies that low level components should run as unique processes)

Complete SHUTDOWN Syntax:

SHUTDOWN {PROCESS}

{PROCESS} = {ALL | AOCA,AODD,AODP,AODS}

### REFRESH_cmd

This command will force all system components to publish all states.

Complete REFRESH Syntax:

REFRESH

## Message Type : Status

### HWFP_stat

The HWFP component will publish a structure containing ALL of the HWFP data at the frame rate specified by the HWFS frame rate.  The HWFP LOG_INTERVAL parameter may be set to > 1 to slow the data output down, but the output rate will generally be high.  If deemed necessary by the RTC engineer (ie: GUI display requirements), the AODP component **may** split the HWFP data into two or more structures. This status structure is an EXAMPLE of one such split option (see 3.3.1 HWFP_data below).  If these structures are separated, then the status structure will only be published when a field changes.  Following is a list of necessary status variables that are typically either single values or unchanged over long periods of time.

HWFP Status Structure:

typedef struct {

float ttm_int_gain			/* TTM integral gain */

float ttm_pro_gain			/* TTM proportional gain */

float hodm_int_gain			/* HODM integral gain */

float hodm_pro_gain			/* HODM proportional gain */

float lodm_int_gain			/* LODM integral gain */

float lodm_pro_gain			/* LODM proportional gain */

float subap_min_flux 			/* subaperture minimum flux */

int ho_mode				/* HODM reconstruct mode TBD */

float ho_rss_res				/* HODM root sum square for line plots */

int lo_mode				/* LODM reconstruct mode TBD */

float lo_rss_res				/* LODM root sum square for line plots */

int log_types				/* telemetry types to log */

int log_rate				/* data logging rate (necessary ?) */

int ttm_rate				/* TTM rate */

int ttm_on				/* ON | OFF */

int hodm_on				/* ON | OFF */

int lodm_on				/* ON | OFF */

char  pix_gains_file[SIZE]		/* gain table from HWFS camera */

char  pix_offst_file[SIZE]		/* offset table from HWFS camera */

char  cent_offst_file[SIZE]		/* centroid offset file */

char  reconst_file[SIZE]			/* reconstructor */

char  ho_flatmap[SIZE]			/* HODM flatmap */

char lo_flatmap[SIZE]			/* LODM flatmap */

} HWFP_stat;

### HWFS_stat

This data structure contains the status of the HWFS component.

HWFS Status Structure

typedef struct {

int rate				/* camera rate (Hz) */

int bias				/* camera bias */

float gain			/* camera gain */

float delay			/* camera delay(?) */

int clamp			/* camera clamp */

int function			/* camera function, replaces program or mode (1:16)

int state				/* disconnected | ON | error */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} HWFS_stat

### ACAM_stat

This data structure contains the status of the ACAM component.

ACAM Status Structure:

typedef struct {

int int_time			/* integration time (milliseconds) */

int rate				/* camera rate (Hz) */

float gain			/* camera gain */

int window			/* full | pupil | acq */

int height			/* ACAM image height */

int width			/* ACAM image width */

int start_row			/* ACAM row start */

int start_col			/* ACAM column start */

int back_sub			/* ON | OFF */

char back_file{SIZE}		/* path to acam background file */

float temp			/* ACAM temperature */

int state				/* disconnected | ON | OFF | error */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} ACAM_stat

### HODM_stat

This data structure contains the status of the HODM component.

HODM Status Structure:

typedef struct {

int state				/* standby | normal | test */

struct temp			/* array of all temperatures */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} HODM_stat

### MOTOR_stat

This data structure contains the status of the MOTOR component.  There will be as many motor structures as there are motors, each to be published separately as a different data item.

MOTOR Status Structure:

typedef struct {

char name			/* motor name */

float position			/* motor position (microns or degrees) */

float velocity			/* requested velocity */

bool home			/* at home? */

int state 			/* disconnected | moving | stopped | error */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} MOTOR_stat

### WLIGHT_stat

This data structure contains the status of the WLIGHT component.

WLIGHT Status Structure:

typedef struct {

int state 			/* ON | OFF */

int pwr 				/* 0 : 100 */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} WLIGHT_stat

### TCS_stat

This data structure is pulled from the TCS by AOCA via the TCS Status_task every 1 second, and then published.  This structure must also be pushed by AOCA to the Pharo Netservices wrapper every one second.

TCS Status Structure:

typedef struct {

char  utc[16];			/* time tag; "ddd hh:mm:ss.s" */

char  ra[12];                   		/* hh:mm:ss.ss */

char  dec[12];                		/* [+/-]dd:mm:ss.s */

char  ha[12];                  		/* [E/W]hh:mm:ss.s */

char  raw_dec[12];        		/* [+/-]dd:mm:ss.s */

char  raw_ha[12];          		/* [E/W]hh:mm:ss.s */

int   telescope_id;          		/* 200 */

int   conn_status;           		/* AO-TCS connection status */

int   tel_stable;               		/* 0 = unstable; 1 = stable */

float airmass;			/* telescope airmass */

float cass_ring_angle;		/* telescope cassegrain ring angle (degrees) */

float focus;                    		/* telescope focus (mm) */

float tubelength;            		/* telescope tube length (mm) */

float ra_offset;               		/* in arcsec */

float dec_offset;            		/* in arcsec */

float ra_track_rate;        		/* in arcsec/hr */

float dec_track_rate;     		/* in arcsec/hr */

} TCS_stat

### Afprogress_stat

This data structure is a list of states for each of the available automations.  An automation state is either “idle”, “finished”, or “running”.  The state parameter is a float from 0 to 1, such that if it is “running”, the percentage to “finished” is known.  This structure is maintained by AOCA and published every one second (?).

Auto Progress Status Structure:

typedef struct {

float af1			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af2			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af3			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af4			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af5			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af6			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af7			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af8			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af9			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af10			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af11			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af12			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af13			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af14			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af15			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af16			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af17			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af18			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af19			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af20			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af21			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af22			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af23			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af24			/* 0.0 : 1.0 (idle | running (%) | finished) */

float af25			/* 0.0 : 1.0 (idle | running (%) | finished) */

} Afprogress_stat

## Message Type : External Status (not published)

### PHARO_stat

This data structure must be pushed to the Pharo Netservices wrapper every one second, to be used in the Pharo FITS header.  The TCS_stat structure (above) is also pushed to Pharo every one second.  This structure is not published by AOCA, it will be sent across the Tcp/IP socket connection.  Since the Pharo control software will not be modified, the structure and variable names will not change.  AO_SCP stands for AO status, configuration, and performance.

PHARO Status Structure:

typedef struct {

float    wfs_delta_x;		/* offset from WFS center to PHARO center (asec) */

float   wfs_delta_y;

float   fsm_int_gain;           		/* FSM servo gains */

float   fsm_prop_gain;

float   dm_int_gain;            		/* DM servo gains */

float   dm_prop_gain;

boolean fsm_on;                 		/* low order on/off */

boolean dm_on;                  		/* high order on/off */

int     conn_status;            		/* AO controller connection status */

int     fsm_rate;               		/* FSM update rate */

int     wfs_cam_rate;           		/* WFS camera readout rate */

int     log_data;               			/* data types currently being logged */

char    reconst_name[NAME_LEN]; 	/* reconstructor name */

float   ao_quality;             		/* AO quality measure (TBD) */

} AO_SCP;

### INST_stat

A generic instrument status structure for instruments that connect to AOCA outside of the Netservices protocol.  A longer list of motors may be required (ie: SWIFT)?

INST Status Structure:

typedef struct {

float ttm_int_gain			/* TTM integral gain */

float ttm_pro_gain			/* TTM proportional gain */

float hodm_int_gain			/* HODM integral gain */

float hodm_pro_gain			/* HODM proportional gain */

float lodm_int_gain			/* LODM integral gain */

float lodm_pro_gain			/* LODM proportional gain */

float subap_min_flux 			/* subaperture minimum flux */

int ho_mode				/* HODM reconstruct mode TBD */

float ho_rss_res				/* HODM root sum square for line plots */

int lo_mode				/* LODM reconstruct mode TBD */

float lo_rss_res				/* LODM root sum square for line plots */

int ttm_rate				/* TTM rate */

int ttm_on				/* ON | OFF */

int hodm_on				/* ON | OFF */

int lodm_on				/* ON | OFF */

char  pix_gains_file[SIZE]		/* gain table from HWFS camera */

char  pix_offst_file[SIZE]		/* offset table from HWFS camera */

char  cent_offst_file[SIZE]		/* centroid offset file */

char  reconst_file[SIZE]			/* reconstructor */

char  ho_flatmap[SIZE]			/* HODM flatmap */

char lo_flatmap[SIZE]			/* LODM flatmap */

int hwfp_int_time			/* integration time (milliseconds) */

int hwfp_rate				/* camera rate (Hz) */

float ssm_1a_pos			/* SSM_1A position */

float ssm_2a_pos			/* SSM_2A position */

float ssm_1b_pos			/* SSM_1B position */

float ssm_2b_pos			/* SSM_2B position */

} INST_stat;

## Message Type : Info Status

### AODD_info

This status structure will be published once at system startup, and will contain hardware info, version numbers, and other system relevant information that we would like to keep track of but only need published once.  The linux version information comes from the Cass cage AODD computer.

AODD Info Structure:

typedef struct {

char version{SIZE}		/* p3k software version number */

char version_date{SIZE}	/* p3k revision date */

char linux_version{SIZE}	/* cass cage linux version */

char hwfs_info{SIZE}		/* HWFS firmware info */

char acam_info{SIZE}		/* ACAM firmware info */

struct {				/* firmware info struct for all motors & controllers */

char aerotech{SIZE}

char newport{SIZE}

} motor_info

} AODD_info

### AOCA_info

This status structure will be published once at system startup, and will contain version numbers and other system relevant information that we would like to keep track of but only need published once.  The linux version information comes from the Data room telemetry AOCA computer.

AOCA Info Structure:

typedef struct {

char version{SIZE}		/* p3k software version number */

char version_date{SIZE}	/* p3k revision date */

char linux_version{SIZE}	/* telemetry linux version */

char aoca_info{SIZE}		/* aoca version number */

struct {				/* list of all the current automations */

char af1{SIZE}

} auto

} AOCA_info

### GUI_info

This status structure will be published once at system startup, and will contain version numbers and other system relevant information that we would like to keep track of but only need published once.  The linux version information comes from the Data room telemetry computer.

AOCA Info Structure:

typedef struct {

char version{SIZE}		/* p3k software version number */

char version_date{SIZE}	/* p3k revision date */

char linux_version{SIZE}	/* telemetry linux version */

char gui{SIZE}			/* gui version number */

} AOCA_info

## Message Type : Telemetry

### HWFP_data

The HWFP component will publish a structure containing ALL of the HWFP data at the frame rate specified by the HWFS frame rate.  The HWFP LOG_INTERVAL parameter may be set to > 1 to slow the data output down, but the output rate will generally be high.  If deemed necessary by the RTC engineer (ie: GUI display requirements), the AODP component **may** split the HWFP data into two or more structures. This data structure is an EXAMPLE of one such split option (see 3.2.1 HWFP_stat above).  The following telemetry data is required for system operation, but will likely be arranged differently, and/or may be split further into smaller data structures.

HWFP Data Structure:

typedef struct {

unsigned short pixels			/* HWFP pixel data */

float centroids				/* HWFP centroids */

float ttm_a_res             			/* TTM residuals */

float ttm_b_res

struct {                        			/* TTM positions */

float ttm_a

float ttm_b

} pos

float hodm_residuals			/* HODM residuals */

float hodm_positions			/* HODM positions */

float lodm_residuals			/* LODM residuals */

float lodm_positions			/* LODM positions */

float subap_flux				/* HWFP subaperture flux */

} HWFP_data

### ACAM_data

The Acquisition Camera (ACAM) will publish the CCD image data at the rate specified by the integration time, which can be as fast as 10 Hz.  This data will be published in a structure separate from the ACAM status structure.

ACAM Data Structure:

typedef struct {

float pixels[height][width]	/* ACAM pixel data */

} ACAM_data

# ERROR CODES

Following is a list of error codes to be used globally by the P3K system (first pass is cut&paste from ao3 with only a few modifications.  This list needs to be better defined).  A more detailed list of states and errors will be handled by the individual lower level components.

(* Should we monitor system health with a process?)

/* non critical errors */

#define SYNTAX_ERROR		200

#define INVALID_TABLE_ID        	201     /* Table id is unknown by system */

#define INVALID_CMD_ID          	202     /* command id is unknown by system */

#define INCORRECT_FILE_FORMAT	204     /* Error detected in file format */

#define FILE_NOT_FOUND          	205     /* Could not find file */

#define NOT_IMPLEMENTED         	206     /* funtion is not implemented yet */

#define INVALID_COMMAND         	208

#define OUT_OF_RANGE            	218     /* Parameter out of range. */

#define WFP_LOAD_FAILED         	219     /* WFP failed to acknowledge table load */

/* critical errors */

#define OUT_OF_MEMORY           	250     /* Attempt to allocate memory failed */

#define INVALID_POINTER         	251

#define OPEN_PIPE_FAILED        	254     /* */

#define INITIALIZATION_ERROR    259     /* controller is not initialized before being used *

# AUTOMATIONS

### AF7 (TTM_OFFLOAD)

Description

This automation is run whenever an external device requests to offload TTM residual error to the telescope.  This automation may be set to ON by default during system startup.

Command Syntax

TTM_OFFLOAD ON|OFF PROMPT {GAIN = 0.0 : 1.0} {TIME_AVG = ? : ?}

{THRESH = ? : ?}

(ao3 time average duration is 3 seconds)

(ao3 threshold is 0.15”)

Procedure

- subscribe to HFWP to get TTM state, and when TTM is ON (check stimulus state?)

  - determine the average TTM position (or residual error) over TIME_AVG

  - convert to ra and dec arcseconds

  - if abs(ra) || abs(dec) > 2.5, error   (ao3 threshold)

  - if abs(ra) || abs(dec) >= THRESH

    - if PROMPT, prompt the user if they want to offload

      - if YES, offload = offload * GAIN

      - send af7_cmd to the telescope

    - else

      - offload = offload * GAIN

      - send af7_cmd to the telescope

  - loop back to a.

- watch for TTM OFF

Block?		NO

Status?		YES

typedef struct {

float gain			/* ttm_offload gain */

int time_avg			/* duration to gather ttm positions (sec */

float threshold			/* lower threshold for avg ttm pos offloads (arcsec) */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF7_stat

### AF8 (FOC_OFFLOAD)

Description

This automation is run whenever an external device requests to offload residual focus error to the telescope.  This automation may be set to ON by default during system startup.

Command Syntax

FOC_OFFLOAD ON|OFF PROMPT {MODE = 1 : i} {GAIN = 0.0 : 1.0} {TIME_AVG = ? : ?} {THRESH = ?:?}

(ao3 time average duration is 7 seconds)

(ao3 threshold is 0.03 mm)

Procedure

- subscribe to HFWP to get LODM and HODM state, and depending on MODE, when LODM or HODM are ON (check stimulus state?)

  - depending on MODE determine the average LODM and/or HODM positions (or residual error) over TIME_AVG

  - extract focus and convert to millimeters of telescope focus

  - if focus > 0.8, error   (ao3 threshold)

  - if focus >= THRESH

    - if PROMPT, prompt the user if they want to offload

      - if YES, offload = offload * GAIN

      - send af8_cmd to the telescope

    - else

      - offload = offload * GAIN

      - send af8_cmd to the telescope

  - loop back to a.

- watch for LODM and/or HODM OFF

Block?		NO

Status?		YES

typedef struct {

int mode			/* RTC mode */

float gain			/* ttm_offload gain */

int time_avg			/* duration to gather ttm positions (sec) */

float threshold			/* lower threshold for avg ttm pos offloads (arcsec) */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF8_stat

### AF9 (STIM)

Description

Select a stimulus source

Command Syntax

STIM SKY | WLIGHT | LASER

CANCEL STIM

Procedure

- if SKY

  - move wlight fold mirror out

  - turn wlight off

- if WLIGHT

  - move wlight fold mirror in

  - turn wlight on

- if LASER

  - move wlight fold mirror in

  - turn wlight off

Block?		NO

Status?		YES

typedef struct {

int stim_pos			/* SKY | WLIGHT | LASER | UNKNOWN */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF9_stat

### AF10 (BACKGROUND)

Description

Take HWFS background data (do we want ability to see this image from GUI?)

Command Syntax

BACKGROUND {RA = 0.0 : 1000.} {DEC = 0.0 : 1000.} {CAM = BOTH | HWFS | ACAM} {HWFS_TIME_AVG = 1 : 60} {ACAM_TIME_AVG = 1 : 60}

CANCEL BACKGROUND

Procedure

- subscribe to AF9_stat to get stimulus position

- subscribe to HFWS to get frame rate

- subscribe to HWFP_stat to get TTM, LODM, and HODM state

- if wlight is on and loops are off

  - turn white light off

  - determine frames necessary to reach TIME_AVG duration (for both)

  - subscribe to HWFP_data (and/or ACAM_data) to get subap_flux

  - average the frames, write this data to disk, and publish

  - turn wlight on and signal completion

- if stim = sky

  - parse ra and dec values

  - send the PT command to the TCS (wait for move DONE)

  - determine frames necessary to reach TIME_AVG duration (for both)

  - subscribe to HWFP_data (and/or ACAM_data) to get subap_flux

  - average the frames, write this data to disk, and publish

  - send the –PT command to the TCS (wait for move DONE)

  - signal completion

Block?		YES

Status?		YES

typedef struct {

float ra				/* telescope right ascension (arcseconds) */

float dec			/* telescope declination (arcseconds) */

float hwfs_time_avg		/* duration to gather background HWFS data (sec) */

float acam_time_avg		/* duration to gather background ACAM data (sec) */

int camera			/* BOTH | HWFS | ACAM */

char hwfs_bg_file		/* path to HWFS_background file */

float hwfs_background[data]	/* HWFS_background pixel data */

char acam_bg_file		/* path to ACAM_background file */

float acam_background[data]	/* ACAM_background pixel data */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF10_stat

### AF11 (LDM_REG)

Description

Register the LODM actuators to the HWFS subapertures

Command Syntax

LDM_REG COARSE | FINE | BOTH {THRESHOLD = ? : ?} {COUNTER = 1 : 15}

CANCEL LDM_REG

Procedure

- subscribe to AF9 to get stimulus position

- turn TTM ON (AF1)

- if wlight is on

  - if COARSE || BOTH

    - publish HWFP LOAD HODM_MAP = last wlight pokemap

    - run the coarse registration algorithm until threshold or counter is met (return error)

  - if FINE || BOTH

    - publish HWFP LOAD HODM_MAP = last wlight finemap

    - run the fine registration algorithm until threshold or counter is met (return error)

  - signal completion

- if stim = sky

  - if COARSE || BOTH

    - publish HWFP LOAD HODM_MAP = last telescope pokemap

    - run the coarse registration algorithm until threshold or counter is met (return error)

  - if FINE || BOTH

    - publish HWFP LOAD HODM_MAP = last telescope finemap

    - run the fine registration algorithm until threshold or counter is met (return error)

  - signal completion

Block?		YES

Status?		YES

typedef struct {

int type				/* 1 = coarse | 2 = fine | 3 = both */

float threshold			/* registration completion threshold */

int counter			/* number of registration iterations to run */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF11_stat

### AF12 (HDM_REG)

Description

Register the HODM actuators to the HWFS subapertures (does this depend on HWFS_LENS being used?)  (will there be various modes to worry about?)

Command Syntax

HDM_REG COARSE | FINE | BOTH {THRESHOLD = ? : ?} {COUNTER = 1 : 15}

CANCEL HDM_REG

Procedure

- subscribe to AF9 to get stimulus position

- turn TTM ON (AF1)

- if wlight is on

  - if COARSE || BOTH

    - publish HWFP LOAD HODM_MAP = last wlight pokemap

    - run the coarse registration algorithm until threshold or counter is met (return error)

  - if FINE || BOTH

    - publish HWFP LOAD HODM_MAP = last wlight finemap

    - run the fine registration algorithm until threshold or counter is met (return error)

  - signal completion

- if stim = sky

  - if COARSE || BOTH

    - publish HWFP LOAD HODM_MAP = last telescope pokemap

    - run the coarse registration algorithm until threshold or counter is met (return error)

  - if FINE || BOTH

    - publish HWFP LOAD HODM_MAP = last telescope finemap

    - run the fine registration algorithm until threshold or counter is met (return error)

  - signal completion

Block?		YES

Status?		YES

typedef struct {

int type				/* 1 = coarse | 2 = fine | 3 = both */

float threshold			/* registration completion threshold */

int counter			/* number of registration iterations to run */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF12_stat

### AF13 (LDM_FLAT)

Description

Generate a LODM flatmap

Command Syntax

LDM_FLAT {CLEAR = 0 : 1} {ITERATE = 1 : 10} {TIME_AVG = 1 : 60}

CANCEL LDM_FLAT

Procedure

- subscribe to AF9 to get stimulus position

- turn TTM ON (AF1) and LODM ON (AF3)

- if wlight is on

  - if CLEAR = 1 load the midrange lodm_flatmap(?), else load current wlight lodm_flatmap

  - for iter = 1 : ITERATE

    - average dm_positions over TIME_AVG period

    - save the average to disk

    - open the LODM loop (primitive?)

    - coadd the average positions to the starting flatmap

    - close the LODM loop

  - write the final result to the current wlight flatmap

  - publish (load) this flatmap

  - use this flatmap to generate wlight pokemap and finemap

  - signal completion

- if stim = sky

  - if CLEAR = 1 load the midrange lodm_flatmap(?), else load current telescope  lodm_flatmap

  - for iter = 1 : ITERATE

    - average dm_positions over TIME_AVG period

    - save the average to disk

    - open the LODM loop (primitive?)

    - coadd the average positions to the starting flatmap

    - close the LODM loop

  - write the final result to the current telescope flatmap

  - publish (load) this flatmap

  - use this flatmap to generate wlight pokemap and finemap

  - signal completion

Block?		YES

Status?		YES

typedef struct {

int clear				/* 0 | 1 */

int iterate			/* number of flatmap iterations */

float time_avg			/* duration to average over dm positions (sec) */

char flat_file			/* path to final flatmap file */

float flatmap[data]		/* flatmap */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF13_stat

### AF14 (HDM_FLAT)

Description

Generate a HODM flatmap

Command Syntax

HDM_FLAT {CLEAR = 0 : 1} {ITERATE = 1 : 10} {TIME_AVG = 1 : 60}

CANCEL HDM_FLAT

Procedure

- subscribe to AF9 to get stimulus position

- turn TTM ON (AF1) and HODM ON (AF3) (LODM?)

- if wlight is on

  - if CLEAR = 1 load the midrange hodm_flatmap(?), else load current wlight hodm_flatmap

  - for iter = 1 : ITERATE

    - average dm_positions over TIME_AVG period

    - save the average to disk

    - open the HODM loop (primitive?)

    - coadd the average positions to the starting flatmap

    - close the HODM loop

  - write the final result to the current wlight flatmap

  - publish (load) this flatmap

  - use this flatmap to generate wlight pokemap and finemap

  - signal completion

- if stim = sky

  - if CLEAR = 1 load the midrange hodm_flatmap(?), else load current telescope  hodm_flatmap

  - for iter = 1 : ITERATE

    - average dm_positions over TIME_AVG period

    - save the average to disk

    - open the HODM loop (primitive?)

    - coadd the average positions to the starting flatmap

    - close the HODM loop

  - write the final result to the current telescope flatmap

  - publish (load) this flatmap

  - use this flatmap to generate wlight pokemap and finemap

  - signal completion

Block?		YES

Status?		YES

typedef struct {

int clear				/* 0 | 1 */

int iterate			/* number of flatmap iterations */

float time_avg			/* duration to average over dm positions (sec) */

char flat_file			/* path to final flatmap file */

float flatmap[data]		/* flatmap */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF14_stat

### AF15 (HWFS_LENS)

Description

Set the system to use one of four HWFS lenslet arrays

Command Syntax

HWFS_LENS 8 | 16 | 32 | 64

CANCEL HWFS_LENS

Procedure

- move the HWFS_LENS motor to the appropriate position

- set the necessary HWFP parameters

- <placeholder>

Block?		YES

Status?		YES

typedef struct {

int lens				/* 8 | 16 | 32 | 64 */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF15_stat

### AF16 (NGS_ACQ)

Description

Center a target on the HWFS camera

Command Syntax

NGS_ACQ

CANCEL NGS_ACQ

Procedure

- GUI should prompt the user to click on the target in the ACAM display

- get the X,Y coordinates from the ACAM click

- subscribe to AF9 to get stimulus position

- if wlight

  - publish image_x and image_y to move SSMs to move wlight to the HWFS cam

- if sky

  - get cass ring from TCS_stat

  - calculate ra and dec, publisha TCS PT command to center the target on the HWFS cam

- TTM ON (?)

Block?		YES

Status?		YES

typedef struct {

int clickX			/* ACAM X click position */

int clickY			/* ACAM Y click position */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF16_stat

### AF17 (NGS_SEEING)

Description

Calculate the seeing of a target in the ACAM

Command Syntax

NGS_SEEING {INT_TIME}

CANCEL NGS_SEEING

Procedure

- GUI should prompt the user to click on the target in the ACAM display

- read the ACAM int_time

- set the ACAM INT_TIME

- get the ACAM data

- centroid the target at the click position and measure the fwhm (publish)

- set the ACAM int_time back

- this automation needs more work

Block?		YES

Status?		YES

typedef struct {

int clickX			/* ACAM X click position */

int clickY			/* ACAM Y click position */

int int_time			/* ACAM integration time used for seeing (ms) */

float seeing			/* measured seeing value (arcseconds) */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF17_stat

### AF18 (SAVE)

Description

Save system settings to a file

Command Syntax

SAVE <FILENAME>

Procedure

- Subscribe to “everything” needed for a system restore (does this need to be running at system startup?)

- write this out to a file on disk

Block?		NO

Status?		YES

typedef struct {

int filename			/* path to save filename */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF18_stat

### AF19 (RESTORE)

Description

Restore system settings from a file (default system file will be DEFAULT)

Command Syntax

RESTORE <FILENAME>

CANCEL RESTORE

Procedure

- read FILENAME

- publish everything

Block?		YES

Status?		YES

typedef struct {

int filename			/* path to restore filename */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF19_stat

### AF20 (RESET)

Description

Reset a system component or process

Command Syntax

RESET <AOCA,AODD,AODP,AODS,ACAM,MOTOR,HWFS,WLIGHT>

RESET <MOTOR>

<MOTOR> = <ACQ | FQ_RELAY | FQ_STIM | STIM | SSM1_A | SSM1_B | SSM2_A | SSM2_B | WFS_Z | WFS_LENS | WHITE_X | WHITE_Y | WHITE_Z | LASER_Z | TTM_A | TTM_B>

CANCEL RESET

Procedure

- a MOTOR reset may be a combo of HOME and reset to DEFAULT

- this needs more thought

- <placeholder>

Block?		YES

Status?		YES

typedef struct {

int item				/* list of reset targets */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF20_stat

### AF21 (STAR_SETUP)

Description

Set system parameters based on star magnitude and seeing

Command Syntax

STAR_SETUP <MAG = -1.0 : 15> <SEEING = 0.5 : 2.0>

Procedure

- publish HWFP params and tables based on magnitude and seeing lookup tables

- <placeholder>

- <placeholder>

Block?		NO

Status?		NO

### AF22 (LOG_RATE)

Description

Convert a given log rate into the log interval required by the HWFP LOG_INTERVAL command parameter.

Command Syntax

LOG_RATE <RATE = 1 : 2000>

Procedure

- subscribe to HWFS_stat to get camera rate

- convert the rate into integer interval between frames

- publish HWFP LOG_INTERVAL = interval

Block?		NO

Status?		NO

### AF23 (SHUT_ALL)

Description

Shutdown everything

Command Syntax

SHUT_ALL

Procedure

- move hardware to safe settings

- when hardware is done, issue a SHUTDOWN ALL

- <placeholder>

Block?		YES

Status?		NO

### AF24 (FLEX)

Description

Activate non common path flexure compensation using small SSM moves from a lookup table

Command Syntax

FLEX ON | OFF {RATE 1.0 : 60.0} {GAIN 0.0 : 1.0}

CANCEL FLEX

Procedure

- from a lookup table apply SSM corrections at RATE x GAIN

- <placeholder>

- <placeholder>

Block?		NO

Status?		YES

typedef struct {

float rate			/* Rate at which to apply flexure corrections (sec) */

float gain			/* Gain applied to the correction */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF24_stat

### AF25 (APRC)

Description

Run the APRC scripts to generate the best science flatmaps and centroid offsets

Command Syntax

APRC <ITER = 1 : 20>

CANCEL APRC

Procedure

- run the APRC scripts to reduce the non common path error to a minimum, generating new flatmaps and centroid offsets

- <placeholder>

- <placeholder>

Block?		YES

Status?		YES

typedef struct {

int iter				/* iteration number */

float wfs_error			/* hwfs error estimate after aprc is complete (nm) */

int pharo_focus			/* focus offset for Pharo (um) */

int error_status			/* global enumerated value */

char status_string{SIZE}	/* detailed freeform comment */

} AF25_stat

# FULL P3K COMMAND LIST

COMMAND <Required Parameter> {Optional Parameter}

HWFP TTM {ON|OFF} {MODE = 1 : i}

HWFP LODM {ON|OFF }{MODE = 1 : i}

HWFP HODM {ON|OFF} {MODE = 1 : i}

HWFP LOAD <TABLE = filename>

<TABLE> = <CENT_OFFSETS | PIX_OFFETS | HO_DM_MAP | LO_DM_MAP | PIX_GAINS | RECONST>

HWFP SET <PARAM = float>

<PARAM> = <HODM_PRO_GAIN | HODM_INT_GAIN | LODM_PRO_GAIN | LODM_INT_GAIN | TTM_PRO_GAIN | TTM_INT_GAIN | SUBAP_MIN_FLUX | HODM_MODE | LODM_MODE>

HWFP LOG <DATA = telem> {INTERVAL = 1 : ?}

<telem> = <(PIXELS, CENTROIDS, SUBAP_FLUX, HODM_POS, LODM_POS, HODM_RES, LODM_RES, TTM_POS, TTM_RES, WFP_STAT) | NONE | ALL>

HWFS {ON|OFF} {RATE = 1 : ?} {BIAS = ?} {GAIN = ? : ?} {DELAY = ?} {CLAMP = ?} {FUNCTION = 1:16} {OFFSET = ?} {INFO}

ACAM {ON|OFF} {INT_TIME = 0.0 : ?} {RATE = 1 : ?} {WINDOW = 1 (full 1x1) | 2 (full 2x2) | 3 (full 4x4) | 4 (acq 4x4) | 5 (pupil 2x2} {TEST = 0 | 1} {BACK_SUB = 0 | 1} {BACK_FILE = file_path} {INFO}

HODM {ENABLE | DISABLE | RESET}

MOTOR

MOVE <MOTOR> <POSITION> {VELOCITY = 0.0 : ?}

OFFSET <MOTOR | VIRTUAL_MOTOR> <OFFSET> {VELOCITY}

HOME <MOTOR>

STOP <MOTOR>

<MOTOR> = <ACQ | FQ_RELAY | FQ_STIM | STIM | SSM1_A | SSM1_B | SSM2_A | SSM2_B | WFS_Z | WFS_LENS | WHITE_X | WHITE_Y | WHITE_Z | LASER_Z | TTM_A | TTM_B>

<VIRTUAL_MOTOR> = <IMAGE_X | IMAGE_Y | PUPIL_X | PUPIL_Y>

WLIGHT {ON|OFF} {PWR = 0 : 100}

TCS <TCS COMMAND>

PHARO <PHARO COMMAND>

INST <INST COMMAND>

GET <COMPONENT> {START} {STOP}

SHUTDOWN {PROCESS}

{PROCESS} = {ALL | AOCA,AODD,AODP,AODS}

REFRESH

TTM_OFFLOAD ON|OFF PROMPT {GAIN = 0.0 : 1.0} {TIME_AVG = ? : ?}

{THRESH = ? : ?}

FOC_OFFLOAD ON|OFF PROMPT {MODE = 1 : i} {GAIN = 0.0 : 1.0} {TIME_AVG = ? : ?} {THRESH = ?:?}

STIM SKY | WLIGHT | LASER

BACKGROUND {RA = 0.0 : 1000.} {DEC = 0.0 : 1000.} {TIME_AVG = 1 : 60}

LDM_REG COARSE | FINE | BOTH {THRESHOLD = ? : ?} {COUNTER = 1 : 15}

HDM_REG COARSE | FINE | BOTH {THRESHOLD = ? : ?} {COUNTER = 1 : 15}

LDM_FLAT {CLEAR = 0 : 1} {ITERATE = 1 : 10} {TIME_AVG = 1 : 60}

HDM_FLAT {CLEAR = 0 : 1} {ITERATE = 1 : 10} {TIME_AVG = 1 : 60}

HWFS_LENS 8 | 16 | 32 | 64

NGS_ACQ

NGS_SEEING {INT_TIME}

SAVE <FILENAME>

RESTORE <FILENAME>

RESET <AOCA,AODD,AODP,AODS,ACAM,MOTOR,HWFS,WLIGHT>

RESET <MOTOR>

<MOTOR> = <ACQ | FQ_RELAY | FQ_STIM | STIM | SSM1_A | SSM1_B | SSM2_A | SSM2_B | WFS_Z | WFS_LENS | WHITE_X | WHITE_Y | WHITE_Z | LASER_Z | TTM_A | TTM_B>

STAR_SETUP <MAG = -1.0 : 15> <SEEING = 0.5 : 2.0>

LOG_RATE <RATE>

SHUT_ALL

FLEX ON | OFF {RATE 1.0 : 60.0} {GAIN 0.0 : 1.0}

APRC <ITER = 1 : 20>

# P3K Messaging TYPES TABLE

| ID | Message Type | Parameters | Publishers | Subscribers |
| --- | --- | --- | --- | --- |
| 1 | HWFP cmd | TTM, LODM, HODM, tables, servo, RTC params | AOCA | HWFP, GUI |
| 2 | HWFP status | RTC params | HWFP | GUI, AOCA |
| 3 | HWFP telem | residuals, centroids, pixels, actuators,… | HWFP | GUI, AOCA |
| 4 | HWFS cmd | framerate, bias, gain, | AOCA | AODD, GUI, HWFP |
| 5 | HWFS status | Framerate bias, gain | AODD | GUI, AOCA |
| 6 | ACAM cmd | on/off, integ, frame rate, gain | AOCA | AODD, GUI |
| 7 | ACAM status | Integ, framerate, gain | AODD | GUI, AOCA |
| 8 | ACAM telem | Acquisition image data | AODD | GUI, AOCA |
| 9 | HODM cmd | startup sequence, driver modes, load maps | AOCA | AODD, GUI |
| 10 | HODM status | State, errors | AODD | GUI, AOCA |
| 11 | MOTOR cmd | home, move, offset… | AOCA | AODD, GUI |
| 12 | MOTOR status | moving, position, errors… | AODD | GUI, AOCA |
| 13 | Wlight cmd | on/off, pwr | AOCA | AODD, GUI |
| 14 | Wlight status | State, errors | AODD | GUI, AOCA |
| 15 | TCS cmd | PT, foc_offset | AOCA (socket) | TCS (socket) |
| 16 | TCS status | Telescope params | TCS (socket), AOCA | AOCA (socket) |
| 17 | Auto status | Idle, running (%), finished | AOCA | GUI |
| 18 | PHARO cmd | Pharo cmds in and out | Pharo (Netsvc), AOCA (Netsvc) | AOCA (Netsvc), Pharo (NetSvc) |
| 19 | PHARO status | AO_SCP data | AOCA (NetSvc) | Pharo (NetSvc) |
| 20 | INST cmd | Generic science inst cmds | AOCA | Inst |
| 21 | INST status | Generic AO data | AOCA | Inst |
| 22 | GET cmd | Get data for non-GUI external | AOCA | DBFIND |
| 23 | REFRESH | All components publish status | AOCA | HWFP, AODD |
| 22 | SHUTDOWN cmd | Shutdown system or components | AOCA | HWFP, AODD, GUI |
| 23 | AFn cmd | automation start, parameters, … | AOCA (tasks) | AOCA |
| 24 | AFn status | Status, error codes | AFn | GUI |
| 25 | AF10 background | HWFS background sky data | AF10 (AOCA) | HWFP |
| 26 | AF13 ldm flatmap | LODM flatmap | AF13 (AOCA) | HWFP |
| 27 | AF14 hdm flatmap | HODM flatmap | AF14 (AOCA) | HWFP |
| 28 | AODD info | Firmware, version numbers | AODD | none |
| 29 | AOCA info | version numbers, automation list | AOCA | none |
| 30 | GUI info | Version numbers | GUI | none |