Design Note:  Mechanical limits of 48"dome drive/bogie system

J. Zolkower

July 29, 2015------1st draft

From dwg 101165 & 101166

Calculate Preload Spring rate:

Karl D reports that installed spring compressed length is 11.5".  This is longer than shown in assembly dwg.  (9.375")

Spring coil and wire diameter measure, # of coils counted.  Confirmed spring is according to drawing.

Determine spring rate

Using calculator and assuming closed ground end………10 active coils.

http://www.acxesspring.com/spring-calculator.html

Currently installed preload on tire

Based on measured compressed length of 11.5" (from Karl D), and spring rate from above calculator:  k= spring rate = 500 lbs/in

dL= compression from free length = 13.25 - 11.5=1.75"

Fs= Spring force= dL * k = 1.75" * 500lbs/in = 875 lbs

Ft = Load on tire from Spring force (current installed)

Fs * 38"  =  Ft * 28"

Ft = 875 *38/28= 1187.5 lbs

Nominal design preload on tire:

dL= compression from free length = 13.25 - 9.375 = 3.875 (nominal spring comp. from dwg)

Fs= Spring force= 3.875" * 500lbs/in = 1937.5 lbs

Ft = Load on tire from Spring force = 1937.5 *38/28= 2629.5 lbs  (>50% higher force on tire than currently installed condition)

Dome drive calculations

Friction drive wheel reference

http://www.schwingmetall.com/download/catalog/WT5556_Rotafrix_Catalog_en.pdf

Typical coefficient of friction for elastomeric/steel Friction Drive:  0.7

Friction load:

We measured 320 lb pull to move the dome (breakaway from static was the same, there is no indication of higher static friction).  Assuming 10° pull misalignment in Y and Z,  gives 310 lb tangential pull.  With a 21 ft radius, dome rolling friction ≈ 6510 lb-ft.

Inertial Load:

From calculations using dome and motor properties:  ~7900 lb-ft torque

(see 48in Dome SpecSheet_V1)

This is worst case value based on theoretical motor output.  A more realistic and lower value could be derived from actual dome acceleration measurements.

Total torque:  Friction torque + Inertial torque:  6510 lb-ft + 7900 lb-ft = 14,410 lb-ft

Tangential start-up force on dome drive band:  14,410 lb-ft / 21ft = 686.2 lbs

Theoretical traction drive force:

Current Installed condition:

Drive wheel pre-load: 1187.5 lbs

With CoF:  1187.5 * 0.7 = 831.25 lbs

Nominal design spring preload condition:

Drive wheel pre-load: 2629.5 lbs

With CoF:  2629.5 * 0.7 = 1840.65 lbs

120% higher drive force capable than installed current settings;

Net radial force at drive wheel shaft:

Max Tire tractive force at contact = τ/r = 10100 lb-in / 8.125" = 1243 lb

(10100 in-lb is gearbox max output)

Nominal tire contact force = = 2630 lb   (Ft from above)

Tractive force of tire generates a reaction force at shaft normal to contact force

Net radial shaft force = √ (x²+y²) = √ (2630² + 1243²) = 2909 lb

Dome truck radial bearing capacity

Dome truck radial bearing:  MRC 5208

Double row angular contact bearing

Assume worst case that 2 radial bearings are loaded:

Net radial shaft force:   2909 lbs

Radial load per bearing:  ~ 2909/2=   1454.5 lbs

Per SKF Bearing literature:

Bearing 5208C    80mm OD

Dynamic Rating:  11,100 lbs

Static Rating:  7640 lbs

Assume no thrust load.

Equivalent radial load= P=  1454.5 lbs (from above)

C = Dynamic load rating = 11,100 lbs

L10 life = (C/P)^3 = (11,100 / 1454.5)^3 = 444 million revs

L10 life is 90% reliability of a single bearing life under defined loading.

Bearing dia"  80mm =>  9.9" circumference

Rail circumference:   2*262*PI=1,646.19"

Bearing rev/dome rev =  1646/9.9=   166.2626 bearing rev/dome rev

444E6/166 = 3.6205E6  ≈ 2.6 million dome revs for L10 bearing life

- Assuming 20 year life on bearings, L10 bearing life => 350 dome revs/day.

From drawing:  102160